3GPP TSG-WG2 Meeting #167	S2-2502385

# Athens, Greece, February 17 - 21, 2025 									 (revision of S2-2500441)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0165 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Timestamping UAV's altitude information reports |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3, TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This paper is the revision of S2-2500441 that has been postponed in SA2#166AHE. If not accepted, it could hinder the progress due to missing functionality in RAN. As per Rel-18 functionality, UAV UE’s altitude information is reported by the UAV UE inside “Measurement Report”, and there is no time stamp in this report, see clause 16.18.1 and 16.18.3 of TS 38.300 for details. Moreover, RAN WG2 (who could potentially take this task and consider adding a time stamping of the UAV provided measurement reports) has not considered that in their work plan. Furthermore, in SA2 Reply LS on “UAV regulations” (S2-2413011, https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_166_Orlando_2024-11/Docs/S2-2413011.zip ) SA2 replied that: - “SA2 is specifying a new feature “Pre-flight Planning and In-flight Monitoring for UAVs” in clause 5.12, and USS changeover in clause 5.13 in TS 23.256. In case of in-flight monitoring for UAV, USS/UTM, may request for flight assistance information and that network notifies the USS if UAV deviates from the defined flight path including the UAV altitude and velocity. - SA2 agrees that the UAV altitude and velocity can be received from NG-RAN, assuming to reuse the existing feature like eventH1/H2 reporting together with additional trigger from the core network as specified in Rel-19.” In this regard, two things needs to be emphasized: 1) time information may be required to detect whether a UAV UE deviates from the assigned flight path as a time schedule can be provided for the “Assigned Trajectory” events as specified in clause 5.2.2.3.1 of TS 23.502; 2) NG-RAN provides information by reusing the existing features. #---SA2#166AHE--- The current description of the procedure implies that a UE may add a time stamp when providing a report with the altitude information to the NG-RAN. However, the currently specified feature of aerial UE altitude reporting (see clause 16.18.3 of 3GPP TS 38.300) does not include timestamping of measurement reports sent by aerial UEs. Doing so now would impede Rel-19 work to be done by RAN WGs, which is not the intention of the solution. Therefore, in order to maintain the time information about UE’s changing the altitude, it is proposed to change/clarify that the NG-RAN node, if configured, may timestamp UE’s altitude report indicating when the report was actually received. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying in Step 2b that timestamping is done by the NG-RAN node (not provided by the UE) when it receives the UE’s altitude information report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete functionality due to missing functionality in RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

### 5.16.3	Reporting UE's altitude information

A procedure for reporting aerial UE's altitude information to the core network is shown in Figure 5.16.3-1. The reporting can be done by the aerial UE based on the received altitude reporting thresholds and indication about reporting periodicity as specified in clause 5.16.2.

Figure 5.16.3-1: Procedure for reporting of UE's altitude information

0.	An aerial UE receives request to perform its altitude information reporting as specified in clause 5.16.2. The request may include altitude reporting thresholds and, optionally, indication about reporting periodicity.

1.	UE's flight altitude meets the previously provided altitude reporting conditions (i.e. altitude thresholds and, optionally, reporting periodicity).

Option A: Application layer.

2a.	The UE may report the altitude information over the application layer to the serving USS/UTM in case the USS/UTM has provided altitude reporting threshold and, optionally, indication about reporting periodicity in step 4a in clause 5.16.2.

NOTE 1:	Application layer reporting itself is outside of 3GPP scope.

3a.	The USS may invoke the Nnef_UAVFlightAssistance_Update to inform the UAS NF/NEF about UE's altitude.

Option B: Node-level signalling.

1b.	The UE provides a report with the altitude information to the NG-RAN.

NOTE 2:	Aspects related to a UE providing altitude information to an NG-RAN node are specified by RAN in TS 38.331 [23].

2b.	 After receiving a report with the altitude information from the UE, the NG-RAN node should, for instance, add a time stamp indicating when the altitude information has been received from the UE; the NG-RAN sends the UE's altitude information report in an N2 message to the AMF.

3b.	The AMF provides the UAS NF/NEF the altitude information report inside a Namf_EventExposure_Notify request.

4.	The UAS NF/NEF together with the USS determines the impact on UAV's flight path based on the received altitude information report; this aspect is outside the scope of the 3GPP.

>>>> End of Changes <<<<