3GPP SA WG2 Meeting #167	S2-2502451

# Athens, Greece, 17 – 21 February 2025	(was S2-2502189,S2-2502167)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3881 | rev | 5 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transition between S&F Mode and Normal Mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The exception sheet in SP-241918 for 5GSAT_Ph3-ARC includes for S&F operation the following: - (2b) – Functional specification of expected UE and network behaviour when transitioning between operation in S&F mode and operation in normal mode Changes are thus needed to define the transitioning in both a UE and a satellite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of UE and satellite requirements for transition between S&F mode and normal mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE or satellite may not be able to successfully or correctly transition from normal mode to S&F mode or the reverse which could prevent network access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.13.9.x	Transition between S&F mode and non S&F mode

If a UE is attached to a PLMN using satellite operating in S&F mode, the UE follows the existing AS and NAS procedures to access a satellite that is not operating in S&F mode (either TN or NTN eNodeB) and vice versa.

In order to ensure smooth idle mode transition from S&F mode to non S&F mode, the old (S&F capable) MME in inter MME mobility shall guarantee that any MM context provided to the new MME is up to date.

If a satellite has no feeder link connectivity, it can only operate in S&F Mode. When a satellite supporting S&F mode obtains a feeder link, then, the satellite may cease to operate in S&F Mode depending on operator policy and regulatory requirement.

# * * * * End of changes * * * *