3GPP TSG-SA WG2 Meeting #167	S2-2502458

# Athens, Greece, 17 – 21 February 2025					(revision of S2-2502236,1810,0719)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1562 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean-up of functionality at call setup for Support of UE-Satellite-UE communication in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Nokia, China Telecom, vivo, CSCN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the approved 5GSAT_Ph3_ARCH work item and the conclusion of FS_5GSAT_Ph3_ARCH captured in clause 8.3 of TR 23.700-29, description has been added for functionality at call setup for Support of UE-Satellite-UE communication in IMS. This CR cleans up ENs and references. Functionality update is needed to include call setup with early media |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clean-up of functionality at call setup for Support of UE-Satellite-UE communication in IMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.3.2, AE.4.2, AE.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In rev 1, functionality update is added to include call setup with early media In rev 2, - merging changes from S2-2501666 and S2-2502140 - alignment of the terminology ‘satellite identifier’ |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

## AE.3.2	At call setup

The P-CSCF receives from the PCF the satellite identifier of the satellite serving the UE and receives from the P-CSCF serving the remote UE the satellite identifier of the satellite serving the remote UE as per clause AE.5.1 and clause AE.5.x. Subject to operator policy, the P-CSCF uses this information to determine whether or not activate optimized media routing. With the lack of the satellite identifier of the satellite either serving the local UE or the remote UE, the P-CSCF determines not to activate optimized media routing.

NOTE:	In this Release of the specification, how the P-CSCF uses the satellite identifier to derive whether optimized media routing is possible is not specified.

If P-CSCF determines the activation of optimized media routing, the P-CSCF instructs the PCF to authorize the necessary resources according to clause AE.5.1 and clause AE.5.x, so that the PCF proceeds to establish a 5GS user plane path for optimized media routing.

## ---Start of the 2nd Change---

## AE.4.2	At call setup

The P-CSCF serving the MO UE selects an IMS AGW on ground at call setup according to TS 23.334 [74], clause AE.5.1 and clause AE.5.x. The P-CSCFs serving the MO UE and MT UE proceed with the determination of whether UE-Satellite-UE communication is possible as described in clause AE.3.2. If the P-CSCF determines the activation of optimized media routing is possible, after deciding to use the optimized media routing (e.g. with consideration of whether early media is in progress or not), the P-CSCF releases the IMS AGW on ground if allocated and selects instead an IMS AGW on satellite according to TS 23.334 [74], clause AE.5.1 and clause AE.5.x. If early media is in progress during call setup, the P-CSCF serving the MO UE does not release the IMS-AGW on ground until the IMS session is answered, then selects the IMS-AGW on satellite according to clause AE.5.x for using the optimized media routing.

---Start of the 3rd Change---

# AE.1	General

This annex describes IMS architecture enhancements to support UE-Satellite-UE communication in IMS, i.e., optimized media routing via IMS user plane on-board satellite.

In this Release of the specification, this feature is supported only for IMS voice/video service and for UEs belonging to the same PLMN and in the non-roaming scenario.

The term and the 5GS architecture for UE-Satellite-UE communication is defined in TS 23.501 [93]. The optimized media routing described in this annex refers to a routing of media between UEs under the coverage of the same or of different serving satellites, using the 5GS network as the IP-CAN and with the media not transiting through any network elements on the ground. The ground fallback routing refers to the case with the media transiting through the ground segment.

The P-CSCFs serving the MO UE and the MT UE negotiate with each other to determine whether or not to activate UE-satellite-UE communication, that is, whether it is possible to apply media routing that only relies on IMS user plane on-board satellite.

## ---End of the Changes---