SA WG2 Meeting #167	S2-2502561

17 - 21 February, 2025, Athens, Greece	(revision of S2-2502344)

Source:	ZTE, vivo

Title:	pCR to TS 23.xyz: NEF Services

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: The paper adds NEF service definition.

1. Discussion

The paper defines the NEF services which include Inventory, Command (Read, Write, Disable), and Notify service operations; these services are consumed by AF.

2. Proposal

It is proposed to capture the following changes into TS 23.xyz.

* * * * Start of Changes * * * *

7.4	NEF services

7.4.1	General

In addition to those defined in TS 23.501 [3] clause 7.2.8 and TS 23.502 [4] clause 5.2.6, the table 7.4.1-1 illustrates additional NEF services to support Ambient IoT.

Table 7.4.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

Editor’s note:  It is FFS whether to use separate service operation to respectively support read, write or permanent disable command procedure.

7.4.2	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_ AIoT_Inventory

Description: The consumer requests to perform an inventory operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the inventory operation.

Input, Optional: Approximate number of AIoT Devices.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_ AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the command operation, Command type (Read, Write, or Permanent Disable).

Editor’s note:  The AIoT Data specific for the Read/Write/Permanent Disable command is FFS.

Input, Optional: Approximate number of AIoT Devices, Approximate message size from the AIoT Device for Read command type,

Editor’s note:  It is FFS whether the Approximate message size from the AIoT Device can also apply for the other command types.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.4	Nnef_AIoT_Notify service operation

Service operation name: Nnef_ AIoT_ Notify

Description: The consumer receives notification of the status or results of the requested service operation. If the consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Command service operation, the consumer implicitly subscribes to the results of the requested service operation.

Input, Required: AF Transaction ID, a list of AIoT Device ID(s) or Failure Cause in case of Failure, Information obtained from each target Ambient IoT Device.

Input, Optional: None.

Output, Required: Result indication.

Output, Optional: None.

Editor’s note:  The NEF AIoT service operation definition needs to be revisited to align with AIOTF services.

* * * * End of Changes * * * *