3GPP TSG-SA2 Meeting #167	S2-2502634

Athens, Greece, February 17 – 21, 2025

Title:	[Draft] LS on NR Femto cell shared by PLMN and PNI-NPN

Response to:	-

Release:	Rel-19

Work Item:	5G_Femto, NR_WAB_5GFemto-Core, eNPN

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact person:	Myungjune Youn

m.youn@lge.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

Overall description

SA2 discussed access control description agreed in RAN3, as captured in RAN3 BL CR to TS 38.300 (R3-247936).

- The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

Among above three options, the second option is not aligned with the current specifications for CAG feature introduced to support PNI-NPN.

As described in TS 23.501 clause 5.30.3.4 for PNI-NPN, the AMF and NG-RAN would reject the UE if none of the CAG ID of the cell are part of the UE's Allowed CAG list.

As described in TS 23.501 clause 5.18.1, the MOCN sharing is achieved by different cell IDs with separate configuration, and is different from the concept of the second option above: “For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.”

Additionally, as described below in TS 24.501, the network would also reject the connection request from the UE not supporting CAG.

// From TS 24.501 ##### 5.5.1.2.8 Abnormal cases on the network side The following abnormal cases can be identified: …… j) Based on operator policy, if the initial registration request from a UE not supporting CAG is rejected due to CAG restrictions, the network shall reject the initial registration with a 5GMM cause value other than the 5GMM cause #76 (Not authorized for this CAG or authorized for CAG cells only). ……

SA2 would like to also clarify that there are different ways to realizing PNI-NPN, as introduced in TS 23.501 clause 5.30.3.1, e.g. dedicated DNN and/or S-NSSAIs. The CAG mechanism is only used for the PNI-NPNs to prevent UE(s) which are not allowed to access the NPN via the associated cell(s).

In SA2, the NR Femto support does not require the use of CAG mechanism. A NR Femto cell can act as a normal cell and provide service to the UEs. The CAG mechanism is only used when there is a desire to limit the access to the Femto cell. In such a case, SA2 decided to reuse the CAG feature specified for PNI-NPN and does not change the existing CAG concept.

Therefore, from SA2 perspective, there seems no clear use case on the second option in the RAN3 BL CR for NR Femto, and the reuse of the existing sharing design with different cell IDs as indicated above is preferred.

Actions:

To RAN3:

SA2 kindly asks RAN3 to take the above information into account for their further work.

Date of Next TSG-SA2 Meetings:

SA WG2#168	07 – 11, April 2025	Goteborg, Sweden

SA WG2#169	19 – 23, May 2025	Fukuoka, Japan