# 3GPP TSG-SA WG2 Meeting #168	S2-2502975

Goteborg, SE, 07 - 11 April 2025	(Revision of S2-xxxxxxx)

Title:	[Draft] Reply LS on MWAB-gNB Configuration

Response to:	S2-2502844/S5-251090

Release:	Rel-19

# Work Item:	     AdNRM_Ph3, VMR_Ph2

Source:	SA2

To:	SA5

Cc:	RAN3

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks SA5 for the LS on MWAB-gNB Configuration as quoted below:

# 1	Overall description

SA5 would like to inform SA2 that SA5 has started discussion on management support for MWAB.

SA5 would like to get confirmation on the list of exact (re-)configuration parameters that are required from OAM server to assist the MWAB-gNB providing information used by MWAB-UE for the BH PDU Session(s) management via URSP processing.

SA2 discussed the question and would like to provide the feedback as following:

- SA2 has specified the MWAB configuration in different aspects in clause 5.49. However, there is no requirement in SA2 to configure MWAB-UE URSP via the OAM configuration of the MWAB-gNB.

- TS 23.501 has specified the configuration of MWAB-UE in clause 5.49.2.1. TS 23.501 has specified that OAM can influence the categorization of the traffic (e.g. N2, N3, Xn, OAM) when MWAB-gNB provides the traffic descriptor (via internal communication) to the MWAB-UE. The MWAB-UE can use either URSP or location configuration to associate the upper layer traffic to a proper backhaul PDU sessions. See below text from clause 5.49.1.2 and clause 5.49.2.1 of TS 23.501.

#### 5.49.1.2	Backhaul PDU Session handling for MWAB

The use of multiple BH PDU Sessions for MWAB-gNB N2, N3, Xn interfaces and OAM access is based on configuration from OAM of the MWAB Broadcasted PLMN/SNPN.

The MWAB-gNB requests BH PDU Session(s) from the MWAB-UE by providing corresponding traffic descriptors specified in TS 23.503 [45] and TS 24.526 [110], via the implementation based internal communication between the MWAB-gNB and the MWAB-UE.

The MWAB-UE establishes, modifies BH PDU Sessions based on the received traffic descriptors from MWAB-gNB, configured URSP rules, and local configuration.

NOTE:	The MWAB-gNB, acting as upper layer of the MWAB-UE described in TS 24.526 [110], can categorize the traffic in different ways (e.g. based on different interfaces Xn/N2/N3/OAM, or nature of the traffic (control signalling vs user plane traffic), or slices supported by the MWAB-gNB) and provides corresponding traffic descriptors to MWAB-UE. The internal communication details between MWAB-gNB and MWAB-UE is outside the scope of this specification. For slice information based categorization, see clause 5.49.1.4.

#### 5.49.2.1	Configuration of the MWAB-UE

The MWAB-UE manages the BH PDU Session(s) for the MWAB operation based on the configuration by the HPLMN or Subscribed SNPN of the MWAB-UE. The configuration of the MWAB-UE can be UE Local Configuration, or URSP rules as defined in TS 23.503 [45], for the determination of the proper PDU session parameters, e.g. the DNN, S-NSSAI, the SSC Mode, etc. The URSP rules can be preconfigured or provisioned to the MWAB-UE by its HPLMN using UE Policy delivery as described in clause 4.2.4.3 of TS 23.502 [3].

2. Actions:

To SA5

ACTION: 	SA2 kindly asks SA5 to take above information into consideration.

3. Date of Next SA2 Meeting:

SA2#169   				19 – 23 May, 2025				Fukuoka City, JP

SA2#170   				25 – 29 August, 2025				      Goteborg, SE