SA WG2 Meeting #168	S2-2503153

Goteborg, Sweden, 07 – 11April 2025                              (was S2-250xxxx)

Source:	vivo

Title:	AIOTF/AMF selection and NF profile

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: It proposes the update the RAN selection and introduce AIOTF/AMF selection for AIoT operation.

1 Discussion

Current specification only covers RAN Reader selection. AMF and AIOTF discovery and selection are still missing.

In clause 6.2.2, there is an EN:

Editor's note: How the NEF selects the AIOTF needs further definition.

Proposal1: It is proposed to introduce AIOTF selection as 5.3.X2 and AMF selection in 5.3.X3, remove the AIOTF selection detail from the reader selection clause and remove the EN in clause 6.2.2.

In clause 5.3, there is an EN:

Editor’s note: It is FFS how AIoT RAN transfers and updates its information to the AIOTF in indirect path.

According to LS S2-2502832 (R3-250905), “FFS on RAN3 signalling for the other A-IoT RAN information”. However, for indirect case, it is unclear whether A-IoT RAN information can be OAM configuration in the AIOTF. Event it is possible, the AIOTF doesn’t know whether the NG-RAN has setup NGAP connection with the AMF, e.g. which NG-RAN can be routed by which AMF. Hence, after NGAP connection setup with NG-RAN, the AMF need to notify the NG-RAN ID at least to the AIOTF. Whether the other AIoT RAN information also provided by NG-RAN via AMF need to further coordinated with the RAN WG3.

Figure-1 Different NG-RANs may setup NGAP connection with different AMFs

Proposal2: For indirect path, the AIOTF obtains the NG-RAN ID of the NG-RAN which setup NGAP connection with the AMF via AMF (see clause 6.X AIoT information transfer per RAN node from AMF and AIOTF) and the AIOTF obtains the other AIoT information of the NG-RAN via AMF or by OAM configuration.

2 Proposal

It is proposed to update the following contents in TS 23.369.

START OF CHANGES (All new text)

5.3	Discovery and Selection of AIoT node(s)

5.3.X1 	General

5.3.X2  AIOTF Discovery and Selection

There may be multiple and separately addressable AIOTFs in a network. AIOTF discovery and selection functionality is implemented in NEF, AMF.

When the NEF performs AIOTF discovery and selection for an AIoT service operation request, the following applies:

-	The NEF may query the NRF to discover the candidate AIoT instance(s) by providing the target area information indicated in the AIoT service operation request by the AF, the NRF returns the AIOTF instance(s) of which AIoT serving area matches with the target area information.

-	Alternatively, the AIOTF information may be locally configured in the NEF.

-  The NEF selects an AIOTF instance based on the available AIOTF instances returned from the NRF or locally configured information in the NEF, depending on operator's policies.

For indirect Connectivity via AMF (see clause 4.2), after setup NGAP connection with a NG-RAN, the AMF performs AIOTF discovery and selection in order to provide the AIoT information (at least of the NG-RAN ID) of the NG-RAN to the selected AIOTF (see clause 6.X). When the AMF performs AIOTF discovery and selection, the following applies:

-	The AMF may query the NRF to discover the candidate AIoT instance(s) by providing the AIoT serving area information supported by the NG-RAN, the NRF returns the AIOTF instance(s) of which AIoT serving area matches with the AIoT serving area supported by the NG-RAN.

-  Alternatively, the AIOTF information may be locally configured in the AMF.

-  The AMF selects an AIOTF instance based on the available AIOTF instances returned from the NRF or locally configured information in the AMF, depending on operator's policies.

NEXT CHANGES (All new text)

5.3.X3 	AMF Discovery and Selection

For indirect Connectivity via AMF (see clause 4.2), AMF discovery and selection functionality is implemented in AIOTF.

When the AIOTF performs AMF discovery and selection for an AIoT service operation, the following applies:

-	The AIOTF may query the NRF to discover the AMF instance(s) by providing the target area information indicated in the AIoT service operation request by the AF, the NRF returns the AMF instances of which serving area matches with the target area. The NEF selects an AMF instance based on the available AMF instances returned from the NRF depending on operator's policies.

-	Alternatively, based on the AMF notification of the NG RAN AIoT information as in clause 6.X, after the NG-RAN selection for the AIoT service operation (see clause 5.3.X4), the AIOTF selects the AMF which has association with the selected NG-RAN for the AIoT service operation.

NEXT CHANGES

5.3.X4 	NG-RAN/AIoT Reader Selection

For an AIoT service operation, the AIOTF selects AIoT RAN node(s) and, optionally, a list of the RAN readers. The AIOTF selects the AIoT RAN node(s) based on the target area information, which is the internal area information converted from the AF provide target area information for the AIoT service operation. The serving area of selected AIoT RAN and selected RAN readers should match with the target area of the AIoT Service Operation.

How the NEF converts external target area information into internal target area information s left to the implementation,

Editor’s note:	It is FFS how to handle the case that the AF provides other information than target area information, for the purpose of reader selection. In such case how the NEF and/or AIOTF use it to select the RAN reader.

The AIOTF may have AIoT RAN information and the optional RAN reader information.

- For direct path, the AIOTF obtains the AIoT information from AIoT RAN by the NGAP or OAM configuration.

- For indirect path, the AIOTF obtains the NG-RAN ID of the NG-RAN which setup NGAP connection with the AMF via AMF (see clause 6.X AIoT information transfer per RAN node from AMF and AIOTF) and the AIOTF obtain the other AIoT information of the NG-RAN via AMF or by OAM configuration.

Editor’s note:	AIoT RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note:	The AIOTF or AMF configuration of AIoT RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

NOTE:	RAN reader information is not exposed to the AF.

If an AIoT service request includes AIoT device Identifier(s), the AIOTF may consider the last known serving RAN reader(s) to determine targeted RAN reader(s) directly.

NEXT CHANGES (All new text)

5.Y 	NF profile

The AIOTF registers and update its NF profile to the NRFand the NF profile of the AIOTF includes the following aspects:

-	AIOTF ID/address,

-	NF type,

-	Supported AIoT serving area,

-	A list of NG-RAN information. The NG-RAN information including:

-  The NG-RAN ID (from the AMF in step3a);

-  The AIoT serving area of the NG-RAN;

-  A list of reader information. The reader information includes the reader ID and AIoT serving area of the reader.

The supported AIoT serving area of the AIOTF may be local configured at the AIOTF or constructed by the sum of the AIoT serving area of NG-RANs which have NGAP connections with the AIOTF in case of direct path or is notified by the AMF and have NGAP connections with the AMF in case of indirect path. In case of indirect path, the AMF notify the AIOTF of the AIoT information (at least of the NG-RAN ID) of the NG-RANs which have NGAP connection to the AMF (see clause 6.X).

AIOTF may update its its NF profile to the NRF in case of:

-  NGAP connection setup with new NG-RAN in direct case;

-	Reception of notification of the RAN node information from AMF in indirect case.

NEXT CHANGES

6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices])  service operation request to the NEF.

Editor's note:	The parameters for the inventory service operation need further definition.

2.	The NEF selects the AIOTF to handle the request. The AIOTF Selection is as defined in clause 5.3.X2.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a correlation ID corresponding to this AF service operation request, as well as the AIoT Device Identification information to be included in the paging message sent by AIoT RAN.

AIOTF performs Reader Selection, see clause 5.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the correlation ID, the AIoT Device Identification information to be included in the paging message, and assistance information to the selected AIoT RAN.

Editor's note:	AIoT Device Identification information needs further definition.

Editor's note:	Whether the Inventory Request sent to AIoT RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	The AIoT RAN sends an Inventory Response to the AIOTF with the correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the AIoT Reader(s) will execute the inventory operation.

If an AIoT device matches the AIoT Device Identification information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	AIoT RAN sends one or more Inventory Report messages to the AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the AIoT_Notify message including a list of AIoT Device ID(s).

Editor's note:	Whether only one, or more than one report is generated by the AIOTF to send to the AF needs clarification.

13.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Notify message including the AIoT Device ID(s).

END OF CHANGES