3GPP TSG-SA WG2 Meeting #168	S2-2503269

Gothenburg, Sweden, April 07–11, 2025

Source:	Ericsson

Title:	Architecture assumptions for IMS voice call over NB-IoT (GEO)

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: This contribution proposes architecture assumptions for supporting IMS voice call over NB-IoT (GEO)

# 1.	Discussion

There is one work task in FS_5GSAT_Ph4_ARC regarding the support for IMS voice over NB-IoT (GEO) connecting to EPC, copied below.

- WT-1: IMS voice call over GEO via NB-IoT NTN connecting to EPC NOTE 1: both CP and UP based NB IoT NTN solutions will be studied in the WT-1, checkpoint will be set before the normative work to align with RAN scope. NOTE 2: Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered. - WT-1.2 Whether and how to support QoS for IMS voice service over GEO satellite, - WT-1.3 whether and how to define IMS enhancement to utilize NB IoT as IP-CAN and potential optimization to comply with the relevant SA1 requirements. - WT-1.4 Support of IMS emergency call via NB-IoT NTN. - WT-1.5 How to obtain UE Geo-location for IMS emergency call and other regulatory services. NOTE 3: Solutions should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U). NOTE 4: There will be NO mobility enhancement developed for the NB-IoT NTN in this WT. NOTE 5: Depending on operators’ policies, IMS enhancements defined in the WT-1 may also be used in 5GS with NR NTN, but no additional study will be conducted in this study item.

The work task description has made several architectural assumptions:

- E-UTRA connecting with EPC

- End user data is transferred over SGi, via either control plane or user plane

- No mobility enhancement for NB-IoT

# 2.	 Proposal

It is proposed to update TR 23.700-19 with below changes:

* * * Start of change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

…

[x] 	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[y]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[z]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

…

* * * Next change * * * *

# 4	Architectural Assumptions

For IMS voice call over NB-IoT (GEO), the following architecture assumptions and principles are applied to this study:

-	The EPC architecture as defined in TS 23.401 [x] is used as the only baseline, and only the architecture option where E-UTRA connecting with EPC and end user data over SGi is considered.

-	The IMS architecture defined in TS 23.228 [y] and IMS emergency session architecture defined in TS 23.167 [z] are used as baseline.

-	Enhancement to NB-IoT (GEO) for supporting IMS voice call shall be minimzed to only essential functions.

-	Mobility enhancement within NB-IoT (GEO) or involving a different RAT (3GPP or non-3GPP RAT) are not considered.

* * * End of change * * * *