3GPP TSG-SA WG2 Meeting #168	S2-2503584

Goteborg, Sweden, April 07 - 11, 2025

Source:	vivo

Title:	23.700-19: Key Issue on support of 5G-LAN with satellite access

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/ Rel-20

Abstract of the contribution: this paper proposes a new key issue for FS_5GSAT_Ph4_ARC.

1. Discussion

The Work Task#2.2 of the FS_5GSAT_Ph4_ARC SID is:

- WT-2.2 UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type. NOTE 8: Whether existing mechanism can be reused to support WT-2 needs to be checked. NOTE 9: No UE impact is expected for WT-2.

3GPP TS 23.501 clause 4.4.6 defines the architecture of 5G-LAN, and clause 5.29 specifies the mechanisms for membership management, PDU Session establishment, and user plane (UP) handling for 5G Virtual Network (VN) groups. Following subclauses try to identify the issues for supporting 5G-LAN with UP anchor point on-board satellite.

1.1 Membership management

The membership management is handled by OAM or by AF to configure the 5G VN group date and member UE’s subscription data in UDM/UDR. The NFs related this aspect are not hosted on satellites, so there will be no impact on this aspect.

Observation 1: No protocol and functional impact on membership management.

1.2 PDU Session establishment

If member UEs access the ‌same satellite‌ when establishing PDU Sessions for the 5G VN group, the terrestrial SMF(s) serving these PDU sessions may select ‌satellite-based PSA UPF as the user-plane anchor for all the PDU sessions. The SMF(s) will configure local switching or N19-based switching rules on the PSA UPF(s) per TS 23.501 clause 5.29.4, ensuring seamless operation without protocol impacts.

For scenarios where member UEs access ‌different satellites‌ or mixed access points (satellite/terrestrial) during PDU Session establishment, the SMF(s) must dynamically decide whether to:

-	Select ‌terrestrial PSA UPF(s)‌ (ground-based),

-	Select ‌satellite-based PSA UPF(s)‌, or

-	Use a ‌hybrid approach‌ (e.g., some UPFs on satellites serving multiple UEs).

This decision depends on real-time factors such as satellite coverage, UE distribution, and N19-based switching applicability.

Observation 2: SMF needs to be enhanced in UP anchor selection.

1.3 Mobility handling

When a member UE moves (e.g., switching satellites, transitioning to terrestrial access, or rejoining the same satellite), the SMF(s) must:

-	Re-evaluate UP anchor points‌ for the PDU Session(s) associated with the 5G VN group,

-	Update rules for local/N19-based switching on UP anchor point(s) to maintain service continuity.

The dynamic change of UP anchor points may impact UP handling.

Observation 3: Support change of UP anchor point and update on rules for local/N19-based switching needs to be studied.

Observation 4: UP handling needs to be re-evaluated considering dynamic change of UP anchor points.

2. Proposal

It is proposed to agree the new key issues into TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows

# * * * * First change * * * *

5.X	Key Issue #X: Support of 5G-LAN with satellite access

5.X.1	Description

When member UEs of a 5G VN group access the ‌same satellite‌, during the establishment of PDU Sessions for the 5G VN group, the terrestrial SMF(s) serving these PDU sessions may select ‌satellite-based PSA UPF‌ as the user-plane anchor for all the PDU sessions and apply local switch. For scenarios where member UEs access ‌different satellites‌ or mixed access points (satellite/terrestrial) during PDU Session establishment, the SMF(s) must dynamically decide whether to select ‌terrestrial PSA UPF(s)‌ (ground-based), select ‌satellite-based PSA UPF(s)‌, or use a ‌hybrid approach‌ (e.g., some UPFs on satellites serving multiple UEs). This decision depends on real-time factors such as satellite coverage, UE distribution, and N19-based switch applicability.

When a member UE moves (e.g., switching satellites, transitioning to terrestrial access, or rejoining the same satellite), the SMF(s) must re-evaluate UP anchor points‌ for the PDU Session(s) associated with the 5G VN group, and update rules‌ for local/N19-based switching on UP anchor point(s) to maintain service continuity, the dynamic change of UP anchor points may impact UP handling.

The following aspects need to be studied:

-	How to select satellite/terrestrial PSA UPF(s) during PDU Session establishment for 5G VN groups;

-	How to modify UP anchor points (e.g., satellite-to-ground transitions) during UE mobility while preserving 5G VN group connectivity;

-	Whether and how to enhance the user plane handling in case of dynamic change of UP anchor points;

-	Whether and how to ensure minimal disruption during UP anchor points reselection or rule modifications.

# -	* * * * End of changes * * * *