SA WG2 Meeting #168	S2-2403951

7th – 11st, April, 2025, Stor-Göteborg, SE	(Revision of 3019)

Source:	OPPO

Title:	New solution of IMS call setup procedure simplification for WT#1 GEO voice support

Document for:	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution of IMS call setup procedure simplification for WT#1 GEO voice

1	Discussion

SA1 has concluded the following requirements and KPIs, the solution is to be further studied by Stage-2.

The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [xa].

The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size.

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| …… |  |  |  |  |  |  |  |  |  |
| IMS voice call using GEO (note 7) | [1-3] kbit/s | [1-3] kbit/s | N/A | N/A | N/A | N/A | TBD | Handheld | Call set up time (note 8) ≤30 s (note 9) |
| …… Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [xb]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [xc]; |  |  |  |  |  |  |  |  |  |

In TS23.228 clause 5.7a.2, the procedure for the session establishment without pre-condition is introduced. These clauses present the general end-to-end session flow procedures without preconditions. The flow in clause 5.7a.2 is applicable to services without real-time QoS requirements before session becomes active, and thus do not need to set-up dedicated IP-CAN bearers but can use existing IP-CAN bearers, and to services which do not require that the terminating endpoint obtains a SIP-level notification when the originating endpoint's IP-CAN bearer becomes available.

Thus, procedure in this clause (no pre-condition) is applied to GEO voice scenario, with some further clarification/additions as below.

2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.1.0.

* * * * Start of Changes (All text new) * * * *

6	Solutions

6.x	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |
| 1 |  | X |  |  |

6.x	Solution #X: Establishment of media session without precondition  for IMS voice over GEO satellite

6.X.1	General description

This solution is to address Key Issue #Y: Support of IMS enhancement to utilize NB- IoT.

In TS23.228 clause 5.7a.2, the procedure for the session establishment without pre-condition is introduced. These clauses present the general end-to-end session flow procedures without preconditions. The flow in clause 5.7a.2 is applicable to services without real-time QoS requirements before session becomes active, and thus do not need to set-up dedicated IP-CAN bearers but can use existing IP-CAN bearers, and to services which do not require that the terminating endpoint obtains a SIP-level notification when the originating endpoint's IP-CAN bearer becomes available.

Thus, procedure in this solution is basically to use the procedure in clause 5.7a.2 (no pre-condition), with the following clarification/change to make it apply to GEO voice scenario:

- A codec is pre-negotiated via Register procedure for GEO voice, such as m=audio, Codec=Codec-2/0.7kbps, Ptime=120ms. Then, the P-CSCF serving the UE knows which codec is supported so as to help the P-CSCF to determine whether the codec transcoding is needed during call setup procedure

“Authorizing QoS Resource” is skipped and UE itself reserves a dedicated IP-CAN bearer for media based on the media parameters received in the SDP offer. The Establishment of EPS bearer for IMS voice will be skipped.

NOTE 1: Since the NB-IoT device has at most 2 EPS bearers over UP, and the GBR bearer is not supported over GEO Satellite access, it is proposed to transmit both IMS signalling and Media via one non-GBR EPS bearer. .

Editor’s Note: it is FFS whether and how to establish two EPS bearers for IMS signalling and media data respectively, if required.

3)  Since there is few choice of codec for GEO voice (it is assumed the NB-IoT device only support one Codec (such as Codec-2/0.7kbps), then it is proposed to have one round media negotiation as TS 23.228 clause 5.7a.2, with

-  skipping SIP Offer Response (183 Session Progress), skipping SIP Response Conf. (PRACK), and skipping 200OK (PRACK).

4)  And no Resource Confirmation (SIP UPDATE) is needed, same as TS 23.228 clause 5.7a.2.

Estimation of call setup time:

According to example in 3GPP TS 24.229 clause 7.2.2, the message size would be about

SIP INVITE: 1.4KByte,

200 OK: 0.8KByte.

Given the codec is very simple for GEO voice, the SDP offer will be a basic one, the 1.4KB for SIP INVITE, and 0.8KB for 200 OK should be ok (No 183 in no pre-condition case).

- SIP INVITE (SDP Offer): latency = 1400*8/(1~3)Kbps+3*0.27s=4.868~12.334s

- SIP 200 OK (Offer Response): 800*8/(1~3)Kbps+3*0.27s=2.943~7.21s

NOTE: it is referring the [1-3K] for UL and DL data rate for GEO in SA1 requirements;

NOTE: It is refereeing the 5QI=1 for satellite and the propagation time GEO is 270ms, then the AN-PDB is 3*propagation time.

NOTE: The Ring can be sent in parallel with 200 OK which is not increase the total call setup time.

Given the solution assumes to associate IMS signalling and media data into default bearer, i.e. no establishment during call setup, there is no time consumed for this part. Then roughly estimate the transmission in IMS network side is about 1.5 second. And the transmission time for TN UE side is 2.5 second.

Therefore, the estimation of total latency for

SAT_UE-to-TN_UE = (4.868~12.334s) + (2.943~7.21s) + 1.5s + 2.5s = 11.81~23.54

SAT_UE-to_SAT_UE = 18.62 ~ 42.08s

NOTE: the call setup time can be further optimized if Compress SIP is introduced.

Editor’s Note: it is FFS whether the numbers used to estimate call setup time is reasonable

6.X.2	Procedure

It is assumed a codec mode can be pre-negotiated during IMS register procedure, such as m=audio, Codec=Codec-2/0.7kbps, Ptime=120ms. Then, the P-CSCF serving the UE knows which codec is supported so as to help the P-CSCF to determine whether the codec transcoding is needed during call setup procedure.

Editor’s Note: it needs to further clarify the codec pre-negotiation during IMS register procedure

Editor’s Note: it is FFS how to deal with the case when UE is TN UE and sends the INVITE with pre-condition.

Optimization for IMS call setup is described as following steps:

Step-1

The MO UE sends SIP Invite with the SDP offer only containing one Codec.

It is assumed the offering entity might expect to receive incoming media for any of the offered media as soon as the offer is received by the other endpoint.

Step-2：

The P-CSCF checks the SIP message and decides whether additional codec(s) need be inserted into SIP message based on pre-negotiated codec or SLA. When inserting additional codec(s), the network should be able to indicate the preferred codec order to AGW.

Step-6

The MT P-CSCF checks the SIP message and decides whether additional codec(s) need be inserted into SIP message based on pre-negotiated codec and SLA.

Step-8

MT UE uses a dedicated bearer for media based on the media parameters received in the SDP offer. This means the IP-CAN will not initiates the reservation of required resources and the P-CSCF does not instruct PCRF to authorize the resources necessary for this session.

Step-10,11

Optionally, the MT UE sends 180 Alerting to MO UE.

The MT UE responds final 200 OK.

NOTE: The order of Step-10 and 11 can be switched

Step-12, 15

When receiving the SDP answer, if the P-CSCF checks if the agreed codec belongs to the original offer it received in step-1. If the P-CSCF decides to use transcoding, it configures the AGW to enable the transcoding. Otherwise, the P-CSCF will not invoke the transcoding functionality.

NOTE: The transcoding procedure in this solution is refering to TS23.228 Clause I.3.3. However, transcoding Support for Interworking the transcoding operation can also be realized by alternative way, E.g. based on iFC in the S-CSCF, the S-CSCF can route the INVITE to an B2BUA that needs to modify SDP offer and perform any pre-conditions interactions

Step-17

MT UE reserves a dedicated IP-CAN bearer for media based on the media parameters received in the SDP offer. This means the IP-CAN will not initiates the reservation of required resources and the P-CSCF does not instruct PCRF to authorize the resources necessary for this session

After step-17:

The same procedure as defined in TS23.228.

6.X.2	Impacts

MO/MT UE:

-  Support pre-negotiated Codec

-  Support alerting without receiving 180 Ringing (Optional)

-  UE reserves a dedicated IP-CAN bearer for media based on the media parameters received in the SDP offer (existing feature

IMS

-  support pre-negotiated codec with MO and MT UE

Skip “Authorize QoS resource” during IMS call setup (Depending on the bearer establishment mode selected for the IP-CAN session, existing feature);

-  Skip T-ADS for MT

Editor’s Note: it is FFS whether to enhance IMS HSS for subscription parameters

Editor’s Note: it is FFS what Codec is supported for GEO voice, pending on SA4 discussion.

Editor’s Note: it is FFS how the compressed SIP should be supported.

* * * * End of Changes * * * *