3GPP TSG-WG SA2 Meeting #168	S2-2504043

# Goteborg, SE, 7th April – 11th April, 2025		(revision of S2-2503492)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0717 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and correction on modification of UP connection between UE and LMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the procedure of modification of the User Plane Connection between UE and LMF, there are some aspects to be clarified or corrected. - Since the procedure can also be used to terminate the connection, Step 1a can be used to notify the AMF of the connection termination if the LMF terminates the connection. The AMF releases the context of the UE after receiving the notification. Other steps are not needed in this case. - When performing LMF relocation for the UP connection, the following principles should be applied: 1) The target LMF should be asked first by the source LMF whether it can accept the user plane connection relocation. 2) Based on the target LMF's response, the source LMF notifies the AMF to trigger the target LMF to initiate UP connection establishment. - In SA2 meeting #165, it was agreed to capture the UE identifier for deferred 5GC-MT-LR, where the AMF needs to provide the UE ID to the LMF. The Nlmf_Location_LocationContextTransfer service operation should be updated to cover the UE ID. The message is also used to indicate LMF relocation, the UE Id is also needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Revise the procedure of Modification of User Plane Connection between UE and LMF to make the connection termination logic clearer. - Revise the procedure of Modification of User Plane Connection to support interaction between the source LMF and the target LMF for UP connection relocation negotiation, by using the Nlmf_Location_LocationContextTransfer service operation. - Nlmf_Location_LocationContextTransfer service operation updated to cover UE identifier and additionly support inidiaction of LMF relocation for User Plane Connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Connection termination logic not clear and the principle of modification of User Plane Connection between UE and LMF not align with control plane case. AMF decision would be to force LMF to establish LCS-UPP which is not follow the rule that it shall be UE and LMF decision to establish LCS-UPP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.3, 8.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.18.3	Modification of User Plane Connection between UE and LMF

The figure 6.18.3-1 shows how a secure user plane connection between UE and LMF is modified. The flow describes change of LMF but applies also when source and target LMF is the same. The procedure can also be used to terminate the user plane connection to Source LMF not selecting any Target LMF.

AMF (Target AMF)

UE

2. Store Target LMF Identifier or trigger LMF reselection

Source LMF

Target LMF

6. Terminate connection with UE

3. Nlmf_Location_UPConfig Request

1a. Nlmf_Location_UPNotify

4.Nlmf_Location_LocationContextTransfer Request

5.Nlmf_Location_LocationContextTransfer Response

6a. AMF requests Target LMF initiates a User Plane Connection

5a. Nlmf_Location_UPConfig Response

0.Trigger LMF UP connection update

Figure 6.18.3-1: Connection modification between UE and LMFs

0.  [Conditional] The LMF discovers a need to change LMF or decides to terminate the user plane connection, e.g. after receiving an event report via user plane from UE or after receiving an AMF relocation information from a target AMF (target AMF obtains from source AMF the UE LCS-UP context which indicates UE has maintained LCS-UP connection with the source LMF, the target AMF may inform the source LMF about the AMF change using the Nlmf_Locatoion_UPConfig request).

1a.	[Conditional] The LMF discovers a need to change LMF or re-establish the user plane connection between UE and LMF or terminate the user plane connection, e.g. after receiving an event report via user plane from UE or after receiving an AMF relocation information from a target AMF (target AMF obtains from source AMF the UE LCS-UP context which indicates UE has maintained LCS-UP connection with the source LMF, the target AMF may inform the source LMF about the AMF change using the Nlmf_Locatoion_UPConfig request). [Conditional] The LMF sends an Nlmf_Location_UPNotify message that includes connection move  or termination is required and if connection move is requested then message includes a target LMF identification. The address of the AMF was provided to LMF as a "Notification Target Address" in latest Nlmf_Location_UPConfig message or Nlmf_Location_UP Subscribe message. If the LMF is going to terminate the user plane connection, only step 1a and step 6are performed and other steps are skipped.

If the request is to terminate the user plane connection, the AMF releases the LCS-UP context.

NOTE 1:	The LMF change procedure is independent from the SSC mode of the PDU Session with dedicated DNN used for positioning. For SSC mode 2/3, PSA UPF connection with the LMF can be relocated with UE movement and LMF can discover the need to change LMF to reduce the user plane path latency.

2.	[Conditional] AMF should store the target LMF identification if a target LMF identification received from source LMF for user plane positioning in step 1a, or AMF may perform LMF reselection if a UE established a LCS UP connection with the source LMF to a new location (which may be out of serving area of source LMF and in serving area of target LMF) and select the target LMF for the current UE location based on LMF service area, LMF user plane positioning capability information and other information listed in clause 5.1. LMF needs to be capable to establish a user plane session for positioning with the UE. After AMF relocation, the target AMF may trigger the LMF reselection and LCS-UP connection modification procedure, e.g., if the source LMF does not perform LMF reselection.

3.	The AMF sends an Nlmf_Location_UPConfig Request towards the source LMF.

If this step is to terminate the LCS-UP connection of the LMF, the message includes an indication of  LCS-UP connection termination.. Alternatively, it may include information about AMF reallocation.

If this step is to trigger LMF relocation, the message includes an indication of LCS-UP connection relocation to trigger source LMF to perfom LCS-UP conection movement to a Target LMF. If AMF selected a target LMF in step 2 or source LMF provided a target LMF identifier in step 1a, the message also contains a Target LMF identification, which indicates source LMF to initiate Nlmf_Location_LocationContextTransfer Request to the selected target LMF.

-    Alternatively, it may include information about AMF reallocation.

NOTE 2:	AMF relocation does not necessarily always cause LMF reselection, the AMF relocation information can keep the LMF informed.

4-5.[Conditional] Based on the indication of LCS-UP connection relocation and target LMF identification in the request of step 3, the source LMF invokes an Nlmf_Location_LocationContextTransfer Request service operation towards the target LMF to indicate the movement of LCS-UP connection of the UE, and also provide the current user plane based location context(s) of the UE to target LMF if there is user plane based periodic and triggered UE location events report context(s). If there is only an indication of LCS-UP connection relocation but no target LMF identification in the request of step 3, the source LMF selects a target LMF to perform above actions.

The target LMF informs source LMF of the location context transfer operation results. If target LMF decides to perfom user plane positioning for the UE, the response should also include an indication that target LMF decided to use user plane positioning.

5a. Source LMF responses to AMF (or Target AMF) to confirm the request of LMF relocation for user plane positioning, connection termination or change of AMF.

If this step is used for connection termination confirmation, the AMF will release the LCS-UP context after receiving the response message.

If the response is to confirm LMF relocation for user plane positioning and source LMF selects a tartget LMF in step 4, the response also contains the target LMF identifier.

6a. If step 3 is to inidicate LMF relocation, after AMF received the successful response in step 5a, steps 3-10 of figure 6.18.2-1 are performed between AMF (or Target AMF), UE, and Target LMF with addition that UE also terminate connection to Source LMF.

To guarantee that only Target LMF could send LCS-UPP connection establishment message to the UE who has already LCS-UPP connection, AMF forwards LCS-UPP connection establishment message only from Target LMF to the UE and AMF should reject the request from other LMF with proper failures.

6.	[Conditional] If the user plane connection to source LMF is still active, the source LMF terminates the connection to the UE.

# * * * * Second change * * * *

8.3.2.5	Nlmf_Location_LocationContextTransfer service operation

Service operation name: Nlmf_Location_LocationContextTransfer

Description: Indiactes there is an LCS-UP connection  of the UE and/or transfers location context information for location event reporting for a target UE from the consumer NF.

Input, Required: AMF identity, UE identifier (SUPI), Indication of LCS-UP connection and/or location context information for location event reporting for a target UE including: Location QoS, Supported GAD shapes, Deferred location type, Deferred location parameters, Notification Target Address, Notification Correlation ID, Embedded event report message.

Input, Optional: Event reporting status, UE location information, LOS/NLOS measurement indication, Indoor/Outdoor indication, UE Positioning Capabilities, Scheduled Location Time, the timestamp of the Location, request for user plane reporting to an LCS Client or AF, cumulative event report timer, maximum number of user plane event reports to an LCS Client or AF.

Output, Required: Success/Failure indication of deferred context transfer and/or indication of target LMF decided to use user plane positioning.

Output, Optional: None.

See clause 6.4 for an example of usage of this service operation.

# * * * * Third change * * * *

8.3.2.7	Nlmf_Location_UPConfig service operation

Service operation name: Nlmf_Location_UPConfig

Description: The consumer NF requests to set up, modify or terminate a secure LCS-UP connection for a target UE.

Input, Required: Notification Target Address, Notification Correlation ID, UE identifier (GPSI, SUPI).

Input, Optional: AMF reallocation indication, LCS-UP connection termination indication, LCS-UP connection relocation indication, LCS-UP connection set up request indication, Target LMF identifier.

Output, Required: Success/Failure indication

Output, Optional: Failure Cause (in the case of failure indication provided), Target LMF identifer.

See clauses 6.18.2 and 6.18.3 for an example of usage of this service operation.

# * * * * End of changes * * * *