3GPP TSG SA WG2 Meeting #168	S2-2502948

7th – 11th April, 2025, Goteborg, Sweden	(revision of S2-220xxxx)

Source:	Apple

Title:	Remaining aspects on user consent and privacy for AI/ML-based positioning

Document for:	Decision

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract: This discussion paper analyses the status of open issues for user consent and privacy for AI/ML-based positioning in SA2 and proposes a way forward.

# 1. Discussion

## 1.1	Introduction

During SA2#167 it was discussed whether the UE LCS privacy profile as defined in TS 23.273 needs to be checked when ground truth data (i.e. UE estimated location) is collected from the UE, and the following was captured in the chair’s notes by Dario related to S2-2502257: “It is agreed that the issue of privacy check will be addressed as corrections in next meetings.”

In addition, the following Editor’s Notes are still outstanding in TS 23.273 on clauses related to AI/ML-based positioning:

Editor's note:	Whether user consent for UE positioning calculation is needed apart from the existing GMLC check of the LCS privacy profile is FFS.

Editor's note:	Further details on user consent for data collection for a specific purpose, e.g. for model training and/or model performance monitoring for LMF-based AI/ML Model positioning will be aligned with SA WG3.

This discussion paper proposes resolutions to some of the above outstanding issues.

## 1.2	UE LCS Privacy Profile Check

During SA2#167 it was discussed whether the UE LCS privacy profile as defined in TS 23.273 needs to be checked when ground truth data (i.e. UE estimated location) is collected from the UE, which, according to the LS from RAN1 in R1-2405578, is required for both training and performance monitoring for AI/ML-based positioning. The discussion did not reach an agreement considering that user consent is also checked by LMF during data collection for training and performance monitoring. However, clauses 5.4.2.2.4 and 6.1.2 in TS 23.273 show that when a 5GC NF (e.g. NWDAF) collects the UE location obtained via the 5GC LCS, the UE LCS privacy profile needs to be checked by GMLC, with no dependency on the user consent status. This is also implied in clause 6.2.12.2 of TS 23.288 where it is mentioned that privacy verification may be performed before the UE location is shared with NWDAF by GMLC.

Based on the above, it is proposed to specify as part of Rel-19 that LMF is the NF that checks the UE LCS privacy profile for training and performance monitoring of AI/ML-based positioning, since GMLC need not be involved for training and performance monitoring in AI/ML-based positioning. It is further emphasized that this check is mandatory when LMF shares the ground truth data because model training and/or performance monitoring takes place at NWDAF.

Proposal 1: LMF is the NF that checks UE LCS privacy profile when ground truth data (i.e. UE location) is collected for training and/or performance monitoring. This check is mandatory when when the ground truth data is shared with NWDAF.

## 1.3	User Consent Aspects

During both SA2#1666AH-E and SA2#167 it was discussed whether new user consent purposes were needed in Table 5.2.3.3.1-1 of TS 23.502 to support AI/ML-based positioning (e.g. by introducing a new purpose to support model monitoring), with a majority of companies seemingly preferring not to do it. In addition, TS 29.503 is the Stage 3 specification rigorously documenting all user consent purposes supported in UDM so far, which includes two additional purposes as those documented in TS 23.502 as examples.

While in our view the distinction between model training and model performance monitoring is clear from a user consent point of view, we also understand the system impacts of extending user consent purpose in Stage 2 at this late stage of Rel-19. Therefore, we propose to add a NOTE in Table 5.2.3.3.1-1 of TS 23.502 similar to the one proposed in S2-2502174 but left unhandled, further indicating that any extension to user consent purpose can be left to Stage 3.

Proposal 2: Add a NOTE in Table 5.2.3.3.1-1 of TS 23.502 indicating that model training purpose applies to both model training and model performance monitoring, and any extension to user consent purpose would be left to Stage 3.

In addition, during SA2#166 SA2 sent an LS to SA3 in S2-2412940 asking whether user consent needs to be checked in addition to the privacy profile when performing inference for AI/ML-based positioning, and the related Editor’s Note mentioned in clause 1.1 of this discussion paper was documented in TS 23.273. Unfortunately, SA3 could not reach an agreement on an LS reply at SA3#120 although the discussion is likely to continue at SA3#121.

Given the above, it seems difficult to agree on a resolution of the Editor’s Note at SA2#168. However, with respect to the user consent purpose discussion, the existing analytics purpose is meant to be used when data is collected from a UE to perform inference at NWDAF and derive the analytics. But Rel-19 and current Rel-20 discussion demonstrate that data from the UE may also be collected at the 5GC to perform inference operations for reasons other than deriving analytics (e.g. AI/ML-based positioning). To address this and minimize system impacts, the solution we propose is to leverage the NOTE in Proposal 2 to further indicate that the analytics purpose in Table 5.2.3.3.1-1 of TS 23.502 also applies to other inference operations, and any further details would be left to Stage 3.

Proposal 3: Leverage the NOTE in Proposal 2 to further indicate that the analytics purpose in Table 5.2.3.3.1-1 of TS 23.502 also applies to other inference operations, and any further details would be left to Stage 3.

## 1.4	Documentation aspects

AI/ML-based positioning combines two technologies (AI/ML and positioning) for which different user consent and privacy mechanisms were specified in previous releases, therefore introducing unclarity on how those mechanisms are applied to AI/ML-based positioning and generating controversial discussions in previous SA2 meetings. In addition, SA2 specifications do not describe a user consent framework independent of the 5GC NF that performs the check, but instead an NWDAF-centric framework was specified in cl. 6.2.9 of TS 23.288 during Rel-17 for the specific use case of analytics. While most of the user consent framework description in TS 23.288 is indeed valid for AI/ML-based positioning on us, it is in our view crucial to indicate in TS 23.273 where the framework is described along the fact that it is LMF instead of NWDAF the 5GC NF that checks user consent for AI/ML-based positioning. Furthermore, we propose to include the LMF check on the UE privacy profile since this is another differentiating factor with NWDAF for ground truth data collection.

Because of the above, we propose a new dedicated sub-clause for user consent and privacy aspects of AI/ML-based positioning to enhance the clarity of the specification.

Proposal 4: Add a new sub-clause for user consent and privacy aspects of AI/ML-based positioning.

# 2. Proposals

Proposal 1: LMF is the NF that checks UE LCS privacy profile when ground truth data (i.e. UE location) is collected for training and/or performance monitoring. This check is mandatory when AI/ML-based positioning is performed at NWDAF.

Proposal 2: Add a NOTE in Table 5.2.3.3.1-1 of TS 23.502 indicating that model training purpose applies to both model training and model performance monitoring, and any extension to user consent purpose would be left to Stage 3.

Proposal 3: Leverage the NOTE in Proposal 2 to further indicate that the analytics purpose in Table 5.2.3.3.1-1 of TS 23.502 also applies to other inference operations, and any further details would be left to Stage 3.

Proposal 4: Add a new sub-clause for user consent and privacy aspects of AI/ML-based positioning.