3GPP TSG-WG SA2 Meeting #168	S2-2503483

# Goteborg, Sweden, April 7 – 11, 2025

Source: 	China Mobile

Title: 	New Key Issue: Network service adjustment with considering the energy consumption information

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: Propose a key issue about WT#2: Whether and how to support service adjustments for the UE to enable energy saving.

1	Discussion

This paper is to propose a new key issue according to the WT#2 of FS_EnergySys_Ph2.

The FS_EnergySys_Ph2 for WT#2:

- WT #2: Whether and how to support service adjustments for the UE to enable energy saving.

WT#2.1 Whether and how to support taking the energy attributes (e.g., renewable energy information) into account to determine UE traffic handling (e.g., QoS adjustment)

NOTE 4: Energy attributes will be coordinated with SA5.

WT#2.2	Whether and how to support policy control based on interaction with EIF

WT#2.3	Identify charging impacts caused by service adjustment (in cooperation with SA5)

2 Proposal

It is proposed to update TR 23.xxx on FS_EnergySys_Ph2 as follows.

FIRST CHANGE (all the text is new)

5.X	Key Issue #X: Network service adjustment with considering the energy consumption information.

5.X.1	Description

In Rel-19, as described in 5.51.6 of TS23.501, the AM policy can be determined by PCF with considering the Energy Saving indicator.

When a UE which has limited data rate because of consuming large amounts of energy, moves into an area of which the RAN or UPFs are supplied by renewable energy, the SM policy should be adjusted to increase the traffic data rate, since the NF nodes supplied by renewable energy is supposed to consume less energy or 0 J energy.

In this release, the following aspects should be studied:

-	Whether and how to support UE traffic handling (e.g., QoS adjustment), with taking the energy attributes (e.g., renewable energy information) into account.

NOTE 1: Energy attributes will be coordinated with SA5.

-	Whether and how to support policy control based on the interaction with EIF.

-	Whether and how to support charging impacts caused by service adjustment e.g., policy adjustment.

NOTE 2: Coordination with SA5 is needed when considering charging related update.

End of CHANGE