3GPP SA WG2 Meeting #169	S2-2504679

Fukuoka, Japan, 19-23 May 2025

# Title:	[Draft] LS on issues related to RoHC for support of IMS voice over NB-IoT NTN connected to EPC

# Response to:	-

# Release:	Release 20

# Work Item:	FS_5GSAT_Ph4_ARC

Source:	SA2

To:	RAN1, RAN2, SA4

Cc:	CT1

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

Under the study item FS_5GSAT_Ph4_ARC SA2 is studying the support for IMS voice over NB-IoT NTN connected to EPC. In the context of this study item SA2 is investigating solutions that could potentially use IP or non-IP type PDN connections for the user plane (voice) traffic. When IP PDN connections are used, in order to achieve the target KPIs for support of IMS voice over NB-IoT NTN connected to EPC, it is expected that the support for Robust Header Compression (RoHC) will likely be required. RoHC is now supported between UE and eNB when "user plane" DRBs are used to transport the data or between the UE and MME when "control plane CIoT" and SRBs are used to transport the data.

In order for SA2 to be able to evaluate the viability of using RoHC for support of IMS voice over NB-IoT NTN connected to EPC, SA2 has the following questions:

If enough consecutive packets are lost or erroneously decompressed, the compressor can find itself in the Initialization and Refresh (IR) state during the call, causing a significant, e.g., 500ms (or longer) gap in the transmission of the voice packets. The frequency of this happening depends on the probability of a packet error rate over the NTN GEO channel.

Question 1 (To RAN1): Does RAN1 have any data regarding how often such gaps can be expected in the transmission of 15-25 bytes voice packets due to the typical NB-IoT NTN channel model when using RoHC?

Question 2 (To RAN2): Is it expected that SR/BSR can be used for NB-IoT NTN in order to deliver occasionally uncompressed data packets?

In the context of using "Non-IP" PDN connections, 3GPP likely needs to define its own header compression scheme since no RoHC profile exists that can compress non-IP data. To do that it is necessary to understand which parts of RTP header, e.g., sequence number, timestamp, is essential to allow the decompressor to be able to provide in order delivery of packets to the MMTEL client.

Question 3 (To SA4): What are the essential RTP header fields for the minimum information that need to be provided in RTP for IMS voice over GEO?

More information on the RoHC related issues is described in S2-2504678 that is not endorsed or agreed by SA2.

2. Actions:

To RAN1:

ACTION: 	SA2 requests RAN1 to answer Question 1.

To RAN2:

ACTION: 	SA2 requests RAN2 to answer Question 2.

To SA4:

ACTION: 	SA2 requests SA4 to answer Question 3.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #170		25-29 August 2025	Goteborg, Sweden

TSG-SA2 Meeting #171		13-17 October 2025	China (TBD)