SA WG2 Meeting #169	S2-2504721

19 - 23 May, 2025, Fukuoka, Japan

Source:	Nokia

Title:	pCR to TR 23.700-04: KI#1 High-level UE data collection procedure and interim agreements

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This contribution proposes a high-level solution for UE data collection procedure, together with initial interim agreements.

# 1. Discussion

This contribution proposes a high-level solution for UE data collection procedure, together with initial interim agreements, all proposed to be discussed and added to TR 23.700-04.

---------- Start of change ----------

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue 1> | <Key Issue #y> |
| #X | x |  |

---------- Next change (all new text) ----------

## 6.x	Solution #X: High-level UE data collection procedure

### 6.X.0	High-level solution Principles

This solution is based on the following high-level principles:

-	Training entity OTT server acting as an AF contacts (via NEF) a “UE data collection NF in 5G Core” to request data collection from specific UEs (identified by Type Allocation Codes), for specific measurement Id(s) and possibly in specific area(s).

-	Configuration of the data collection in the UEs is done via Control Plane

-	gNBs are responsible for selecting the UEs for data collection based on Type Allocation Codes provided by Training entity OTT server, taking user consent and local gNB information into account

-	UEs are configured by gNBs for data collection, together with UP information, which then allows UEs to transfer collected data to “UE data collection NF in 5G Core” via User Plane

### 6.x.1 	     Description

This solution addresses Key Issue #1.

A “UE data collection NF in 5G Core” is responsible for handling requests from training entity OTT servers. This NF could be mapped to existing NF, e.g. DCCF, or could be a new NF.

The request from training entity OTT server (acting as an AF) for data collection may contain specific target UE(s), specific geographical area(s), and specific measurement Id(s). The identification of target UE(s) is made via Type Allocation Code (TAC).

The “UE data collection NF in 5G Core” transfers the request to AMFs together with UP information (allowing UE(s) to address the “UE data collection NF in 5G Core”), The AMF then forward the request to appropriate gNBs.

The gNBs are responsible for selecting the UEs for data collection, based e.g. on other procedures running at gNB for the UEs, e.g. avoiding conflict with MDT or taking radio conditions into account. Once the UEs are selected, the gNBs provide the request to the UEs, together with the UP information.

The UEs then start data collection and data transfer to the UP entity provided in UP information.

### 6.x.2	Procedures

### 6.x.2.1	Initiating UE data collection

The procedure is shown below:

AF

NEF

SMF

AMF

RAN

UEr

UE data collection NF in core

0. TAC and user consent info provided to gNB

1. Request for UE data collection

2. Request for UE data collection

3. Request (+ UP information)

4. UEs selection

5. RRC Reconfiguration (+ UP information)

6. PDU session establishment

7. UE training data transfer

8. UE training data check

9. UE training data transfer

Figure 6.x.2-1: High-level UE data collection initiation procedure

0.	During UE context setup, AMF provides RAN with Type Allocation Code and user consent information for the UE.

1.	Training entity OTT server request for UE data collection is provided to “UE data collection NF in 5G Core” via NEF. The request contains Type Allocation Code(s) for the UEs to collect data from, geographical area(s) or cell(s) where to collect data, and measurement Id(s) indicating the type of measurements expected. The NEF checks whether the AF is authorized for this request. The NEF maps the geographical area(s) into a list of TAIs.

2.	 The “UE data collection NF in 5G Core” transfers the request to appropriate AMFs, together with UP information indicating the intended destination of the collected data.

3.	The AMF forwards the request to appropriate gNBs. This uses non-UE associated NGAP signaling. The AMF provides the UP information to the gNB. The UP information provides enough information to the UE to be able to establish the PDU session in step 6, and they are transparently forwarded by gNB to the selected UEs in step 5.

4.	The gNBs are responsible for selecting the UEs based on information received in step 3 and information received in step 0, also taking into account local information at gNB. For example, the gNBs may take into account potential conflict with on-going MDT procedures or radio conditions associated with the specific measurement Id.

5.	The gNB provides radio measurement configuration to the UE, together with the UP information.

6.	(if such PDU session is not already established) The UE establishes a PDU session for data collection taking the received UP information into account . This may use URSP.

7.	The UE starts collecting data and provides data to the “UE data collection NF in 5G Core” over user plane.

8.	The “UE data collection NF in 5G Core” can check collected data, if configured by the operator.

9.	The “UE data collection NF in 5G Core” transfers the collected data to the training entity OTT server.

### 6.X.3	Impacts on services, entities and interface

Editor’s note:	Details to be added.

---------- Next change (all new text) ----------

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The UE selection will eventually be done at gNBs, based on information provided by Core Network and based on local information at gNB.

The training entity OTT server provides Type Allocation Codes for the UEs to collect data from, possibly together with geographical area(s), and measurement Id(s).

## 7.2	Topics for further consideration

### 7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future release.

Whether “UE data collection NF in 5G Core” can be mapped to existing NF or should be a new NF.

---------- End of changes ----------