3GPP TSG-SA2 Meeting #169 	S2-2504790

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025			      (revision of S2-2503071)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3905 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on S&F Wait Timer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The S&F Wait Timer behaviour at UE is not clearly described when the S&F Wait Timer is received in a NAS message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification text added on UE behaviour when S&F Wait timer is running |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing clarity in UE behaviour of handling S&F Wait Timer |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# FIRST CHANGE

### 4.13.9	Support of Store and Forward Satellite operation

#### 4.13.9.1	General

The Store and Forward Satellite operation applies in E-UTRAN with satellite access and is suitable for delay-tolerant communication services (e.g. CIoT/MTC, SMS, etc.). If the satellite does not support Store and Forward Satellite operation and there is no feeder link connection to the ground network, then the satellite cannot provide any service to any UEs. When both the service link and feeder link are available, all services can be provided. To support Store and Forward Satellite operation, eNodeB and some network functionalities need to be deployed on the satellite.

Example deployments are described in Annex O.

Figure 4.13.9.1-1: Store and Forward Satellite operation

NOTE 1:	In Figure 4.13.9.1-1, the NTN Gateway belongs to the satellite deployment and is not subject to 3GPP specifications.

In Store and Forward Satellite operation, the end-to-end exchange of signalling/data traffic is handled in a sequence of steps reflecting the intermittent availability of the service link when the satellite can exchange data with the UE and of the intermittent availability of the feeder link when the satellite can exchange data with the core network. This is depicted in Figure 4.13.9.1-1.

For example, a simple sequence of events for the transmission of data from the UE to a server on the ground may involve two steps: firstly, when service link is available (e.g. location L1 in Figure 4.13.9.1-1) the UE sends the data to the satellite. Subsequently (e.g. location L2 in Figure 4.13.9.1-1), the satellite carrying the payload connects to the ground network and delivers the UE data to the network.

Downlink data can be stored onto the same or a different satellite and provided to the UE later using the first step and the process is repeated.

When an on-board MME supports Store and Forward Satellite operation and is operating in S&F Mode, the on-board eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5], which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in S&F Mode and if a UE is enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

NOTE 2:	A satellite operating in S&F mode implies that the on-board eNodeB deployed on it is operating in S&F mode and is connected with an on-board MME that is also operating in S&F mode.

NOTE 3:	Some deployments cannot support all system features, for example user plane establishment and user plane data transfer cannot be supported, when a satellite is operating in S&F Mode with the split MME deployment described in Annex O.

The MME may adapt periodic update timer, mobile reachable timer and Implicit Detach timer to the fact that the UE is served by an MME operating in S&F Mode.

For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode:

-	If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to Store and Forward Satellite operation.

-	If the UE is rejected with a reject cause indicating it is due to S&F operation, the UE's EMM state shall remain unchanged.

-	The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting a NAS procedure or rejecting a NAS procedure with S&F reject cause.

NOTE 4:	How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g., based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, how the network is deployed (see Annex O for examples), etc.

-	If the UE receives the S&F Wait Timer in Attach Accept, Service Accept or TAU Accept, the UE shall start the S&F Wait Timer immediately and can continue to access and perform NAS procedures with the same satellite. When the S&F Wait Timer is running, the UE shall not attempt to access any other satellite in S&F Mode in the same PLMN, and shall not perform any NAS procedures while the S&F Wait Timer is running.

-	If the UE receives the S&F Wait Timer along with S&F reject cause code in Attach Reject, Service Reject or TAU Reject, the UE shall start the S&F Wait Timer immediately and shall not attempt to access the same satellite or any other satellite in S&F Mode in the same PLMN, and shall not perform any NAS procedures while the S&F Wait Timer is running.

-	When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause.

-	The S&F Wait timer doesn’t restrict the UE to attempt TAU procedure whenever it moves out of the TAI list.

-	The MME may indicate to the UE that it should stop a running S&F Wait Timer in any NAS accept message.

NOTE 5:	When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions.	The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from.

-	The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s).

NOTE 6:	The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode.

NOTE 7:	How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

-	The MME may indicate to the UE an Estimated S&F UL Delivery Time in a NAS accept messages (i.e. Attach Accept, TAU Accept or Service Accept messages).

NOTE 8:	The Estimated S&F UL Delivery Time is an estimate of the time required to deliver the data or signalling sent by the UE to the ground. The Estimated S&F UL Delivery Time is associated with the satellite that provides it and how UE uses this information is left for UE implementation.

The MME may provide S&F Monitoring List to the UE as part of detach procedure.

If the UE did not indicate Store and Forward capability in NAS signalling, if the network determines to reject the UE, the network shall reject the UE request with a cause non-specific to S&F Mode.

The MME may trigger an Update Location procedure with the HSS along with the authentication procedure to fetch subscription information from the HSS. The MME may indicate the timestamp information to HSS during the Update Location procedure. This timestamp information shall be used by the HSS to ensure that newer location for that UE is not cancelled. If MME makes an Update Location Request before the completion of the authentication procedure, it shall include an indication that this Location Update is provisional, i.e. the HSS shall not consider the UE as registered until it receives the final Update Location Request (without indication that the Location Update is provisional). The timestamp information is the time when the on-board of satellite MME part has received the NAS procedure from the UE. The HSS compares the timestamp received in the Update Location Request with any stored timestamp of a previous Update Location Request and determines whether to accept or reject the request. If the received Update Location Request does not include a timestamp, the HSS assumes the present time as the timestamp of the received Update Location Request. The HSS shall reject the Update Location Request, if the timestamp associated with this request is older than the stored timestamp. If the HSS accepts the Update Location Request, the HSS shall store the timestamp associated with the latest Update Location Request. If the HSS does not support the timestamp, the MME shall reject a UE Attach request for S&F Mode.

NOTE 9:	The timestamp information is used by the HSS to ensure that interactions with MMEs for a UE are handled in the correct order.

NOTE 10:	The absence of timestamp in the Update Location Request can occur e.g. when the UE is connecting from a network that does not operate in S&F mode.

The EPS may expose whether a UE is served by an MME operating in S&F Mode and provide to the SCS/AS related timing information to guide the SCS/AS decision when to try to contact the UE. This is further described in clause 5.6.3.10 of TS 23.682 [74].

# End of CHANGEs