3GPP TSG-SA2 Meeting #168	S2-2504840

19 - 23 May, 2025, Fukuoka, Japan

Source:	China Telecom

Title:	KI#3, New Sol: Sensing Entity and Sensing Function Discovery and (Re-)Selection

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This paper proposes solution to address Key Issue#3 Sensing Entity/Entities and Sensing Function(s) discovery and (re-)selection.

1. Introduction

According to TR 23.700-14, Key Issue #3 will study how to perform the Sensing entity(ies) and Sensing Function(s) discovery and (re-)selection. In particular, this KI will address:

-	When required, for the discovery and (re-)selection of the Sensing entity(ies), the study needs to address the following aspects:

-	Which entity(ies) performs the discovery and (re-)selection of the Sensing entity(ies);

-	What the information and/or criteria are taken into account for the discovery and (re-)selection of the Sensing entity(ies); and

-	The procedure(s) for the discovery and (re-)selection of the Sensing entity(ies).

-	When required, for the discovery and (re-)selection of the Sensing Function(s), the study needs to address the following aspects:

-	Which entity(ies) performs the discovery and (re-)selection of Sensing Function(s);

-	What the information and/or criteria are taken into account for the discovery and (re-)selection of Sensing Function(s); and

-	The procedure(s) for the discovery and (re-)selection of the Sensing Function(s).

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |
| --- | --- |
| Solutions | 3 |
| X | X |

# * * * * Second change (All Text New) * * * *

6.X	Solution #X: Discovery and selection of the Sensing Entity/Entities and Sensing Function(s)

6.X.0	High-level solution Principles

The high level principles of the solution are:

- The Sensing Function(s) need to report their own Sensing Capability to the NRF.

- The Sensing Entity/Entities need to report their own Sensing Capability to the Sensing Function(s).

- The NEF discovers and selects the appropriate Sensing Function(s) based on the Sensing Capability in the NRF, the supported sensing service type, sensing service requirement, and sensing area information.

- The Sensing Function(s) discovers and selects the appropriate Entity/Entities based on the Sensing Capability of the Sensing Entity/Entities, the supported sensing service type, sensing service requirement, and sensing area information.

6.X.1	Description

In 5GS, not all RAN nodes and/or UEs are able to perform sensing data measurement, and multiple Sensing Function instances may be deployed in the network for area or service-type-specific management purposes.

For the discovery and (re-)selection of the Sensing Function(s), the following information may be considered by the NEF:

-	Sensing Capabilities of the Sensing Function(s) (e.g., the supported calculation accuracy)

-	Supported sensing service type.

-	Supported sensing service requirement.

-	Supported sensing area information.

For the discovery and (re-)selection of the Sensing Entity/Entities, the following information may be considered by the Sensing Function(s):

-	Sensing Capabilities of the Sensing Entity/Entities (e.g., the measurement capability of RAN entities)

-	Supported sensing service type.

-	Supported sensing service requirement.

-	Supported sensing area information.

6.X.2	Procedures

Figure 6.X.2.1-1: Procedure of Sensing Capability Report

1.	The Sensing Function(s) need to report their own Sensing Capability to the NRF. If the capability changes, the Sensing Function(s) can update its capability information to the NRF or deregister itself to the NRF.

Therefore, after the NEF authorizes the sensing service request(s) from the AF, the NEF can perform the discovery and (re-)selection of the Sensing Function(s) based on the Sensing Capability profile in the NRF, the supported sensing service type, sensing service requirement, and sensing area information.

2.	When received the Sensing Capability report of the Sensing Function(s), the NRF store / update the Sensing Capability profile.

3. 	The NRF sends the Sensing Capability report response to the Sensing Function(s).

4. 	The Sensing Entity/Entities (e.g. the RAN entities) need to report their own Sensing Capability to the Sensing Function(s).

Therefore, after received the authorized sensing service request(s), the Sensing Function(s) can perform the discovery and (re-)selection of the Sensing Entity/Entities based on the Sensing Capability of the Sensing Entity/Entities, the supported sensing service type, sensing service requirement, and sensing area information.

NOTE 1: Whether and how to perform AMF selection for the discovery and (re-)selection of Sensing Entity/Entities can be discussed later.

6.X.3	Impacts on services, entities and interfaces

NEF:

- Support the Sensing Function(s) discovery and (re-)selection

NRF:

- Support the Sensing Function(s) capability registration/update/deregistration.

Sensing Function(s):

- Support the Sensing Entity/Entities discovery and (re-)selection

# * * * * End of changes * * * *