SA WG2 Meeting #169	S2-2505004

# Fukuoka, Japan, May 19 – May 23, 2025                        (revision of S2-25xxxxx)

Source: 	CATT

Title: 	Update on Device Permanent Identifier and Filtering Information

Document for: 	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper proposes update on Device Permanent Identifier and Filtering Information to TS 23.369.

1	Discussion

This paper proposes to resolve the following ENs on Device Permanent Identifier and Filtering Information.

Editor's note:	The reference in NOTE 3 needs to be updated, when the appropriate stage 3 document is identified.

Resolution: This EN can be removed as AIoT Device Permanent Identifier will be defined in TS 23.003 by stage 3.

Editor's note:	Whether the Domain Information can be empty needs to be clarified.

Resolution: This EN can be removed as normative text specifies “The Domain Information includes none, one or more of the following: …”.

Editor's note:	Whether and how to secure the filtering information is up to SA WG3.

Resolution: This EN can be removed as it depends on SA3 conclusion.

2	Proposal

It is proposed to update the TS 23.369 as followings.

# * * * * First change* * * *

5.7.2	AIoT Device Permanent Identifier

In order to support the AIoT feature in 5G System, a globally unique AIoT Device Permanent Identifier shall be allocated to each AIoT Device. An AIoT Device Permanent Identifier is assigned either by an operator or by a third party. The AIoT Device Permanent Identifier is used to identify an AIoT Device and locate the entity where the AIoT Device related information is stored.

NOTE 1:	How to configure an AIoT Device with the AIoT Device Permanent Identifier is out of scope of this specification.

The AIoT Device Permanent Identifier includes the following components:

-	The ID Type, including:

-	Information indicating whether a PLMN ID is included.

-	Information indicating whether a NID is included.

-	Information indicating whether a third party identifier is included.

-	Identification Information Type, indicating whether the Identification Information contains an  EPC or unstructured information.

-	The Domain Information includes none, one or any combinations of the followings:

-	A PLMN Identifier (i.e., MCC and MNC) as specified in TS 23.003 [6] when the information in the ID type indicates it is included

-	A Network Identifier (NID) as specified in TS 23.003 [6] when the information in the ID type indicates it is included.

-	A third party identifier used to identify a third party when the information in the ID type indicates it is included.

-	The Identification Information is used to distinguish different AIoT Devices within the scope identified by Domain Information (if available) and can contain either of the followings:

-	An EPC, as defined in clause 14 of GS1 TDS Release 2.1 [7].

-	Unstructured information, where the contents is defined by the allocator.

Figure 5.7.2-1: AIoT Device Permanent Identifier Structure

An operator allocated AIoT Device Permanent Identifier should include the identifier of the network for the operator. The identifier of the network is present as either a PLMN Identifier, NID or both in the AIoT Device Permanent Identifier.

A third party allocated AIoT Device Permanent Identifier may include none of the following information or include any combination of at least one kind of the following information: a PLMN Identifier, NID or the third party identifier.

NOTE 3:	The length of ID Type, PLMN Identifier (if present), NID (if present) and the third party identifier (if present) components is fixed. The length of the Identification Information is variable. The details are specified in TS 23.003 [6].

The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits.

NOTE 4:	The encoding for the length of the Identification Information enables additional shorter or longer fixed lengths to be supported in the future.

# * * * * Second Change * * * *

5.8	Filtering Information

The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components of the AIoT Device Permanent Identifier as defined in clause 5.7.

The filtering information includes a list of filtering element information. The filtering element information includes:

-	Information indicating which component of the AIoT Device Permanent Identifier is used to match the bitstring.

-	A bitstring which is used to compare with the component.

Each bitstring is corresponding to one component of the AIoT Device Permanent Identifier.

NOTE:	The bitstring match can start at any position in the corresponding component.

To determine whether an AIoT Device Permanent Identifier matches the filtering information, it is compared with every filtering element information within filtering information by comparing the bitstring in a filtering element information with the indicated component of its AIoT Device Permanent Identifier. If all the compared bitstrings match the AIoT Device Permanent Identifier then an AIoT Device Permanent Identifier matches the filtering information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the filtering information.

# * * * * End of Changes * * * *