3GPP TSG-SA WG2 Meeting #169	S2- 2505194

19 - 23 May, 2025, Fukuoka, Japan

Title:	23.700-14: New Solution for Key Issue#5: Sensing Result Exposure

Source:	ETRI

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract:	This contribution proposes a solution to support Key Issue #5: Sensing Result Exposure.

Proposal

The following Solution is proposed for inclusion in TR 23.700-14 to address the solution for the key issue #5 which addresses the exposure of sensing results to support sensing services. The aspects to be studied in this key issue include:

-	Study what types of sensing results with or without the sensing contextual information defined in TS 22.137 [2] could be exposed from the Core Network to the sensing service consumer.

-	How the Core Network exposes sensing results to the sensing service consumer (e.g., one time, periodic or event trigger).

* * * First Change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #5> | <Key Issue #y> |
| #1 |  |  |
| #2 |  |  |
| #X | X |  |

6.X	Solution #X: Sensing Result Exposure for Sensing Service Consumers

6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Clause(s) may be added to capture details.

This solution addresses Key Issue #5: Sensing Result Exposure by specifying how sensing results—enriched with contextual information—are exposed to authorized sensing service consumers. These consumers may include trusted or untrusted Application Functions (AFs).

The sensing function:

- Collects and processes sensing data from distributed sensing entities (e.g., NG-RAN nodes).

- Produces sensing results (e.g., object detection, environmental measurements, and motion recognition).

- Exposes sensing results enriched with sensing contextual information (e.g., timestamp, location, velocity, area, confidence).

NOTE:	The sensing function may correspond to the network function responsible for sensing result exposure, based on the outcome of the study on Key Issue #1: System Architecture to Support Sensing.

Sensing results exposure supports the following reporting modes:

- One-time delivery (on-demand request)

- Periodic reporting (e.g., every 10s, 1 min)

- Event-triggered reporting (e.g., object movement, area entry/exit)

The sensing function exposes these results using:

- Direct interface (Nsf_EventExposure) for trusted AFs.

- Indirect interface (Nnef_EventExposure) via NEF for untrusted AFs.

Sensing assistance information, as described in TS 22.137 [2], may be provided by trusted third parties (e.g., area of interest, target parameters, velocity threshold) to improve sensing performance.

6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

This clause describes the procedures for sensing service request, subscription, and event notification between the sensing function and service consumers.

Table 6.X.2-1: Sensing Function Exposure Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsf_SensingService | Request | Request/Response | NEF, AF |
|  | Response | Request/Response | NEF, AF |
| Nsf_EventExposure | Subscribe | Subscribe/Notify | NEF, AF |
|  | Notify | Subscribe/Notify | NEF, AF |
|  | UnSubscribe | Subscribe/Notify | NEF, AF |

(1) Sensing Service Request from AFs

A trusted AF subscribes to sensing services using the Nsf_EventExposure_Subscribe to the sensing function. An untrusted AF initiate subscriptions via the NEF using Nnef_EventExposure_Subscribe, which forwards the request to the sensing function. Each request includes:

- Sensing type (e.g., motion, object, environment, etc)

- Reporting mode (one-time, periodic, or event-triggered)

- Sensing assistance information (e.g., target area, position, velocity)

(2) Sensing Result Exposure to AFs

Upon receiving a valid subscription, the sensing function configures sensing operations among the relevant entities. The sensing function sends sensing results to the trusted AF using Nsf_EventExposure_Notify or to the NEF, which then uses Nnef_EventExposure_Notify to deliver them to the untrusted AFs.

Each notification includes:

- Sensing result payload (object type, location, velocity, geometric information, etc.)

- Contextual information: e.g., timestamp, source ID, accuracy, detection confidence, and serving area

Figure 6.X.2-1: Sensing Service exposure procedure for sensing result

- Sensing Service Request: AF issues request (direct or via NEF if untrusted).

- Data Collection: Sensing function gathers data from sensing entities as configured.

- Sensing Service Response: Sensing function response with sensing result.

- (Un)Subscribe Request: AF subscribes to updates or event triggers.

- (Un)Subscribe Response: Acknowledged by sensing function.

- Event Notification: Sensing result exposed to AF as per configuration.

6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interface

Services:

Introduction of exposure services for sensing: motion tracking, object/environment detection.

Entities:

Sensing Function: Function responsible for result processing and exposure.

Sensing Service Consumer: Consumes sensing results for vertical applications.

NEF: Optionally mediates result exposure with policy enforcement and access control.

Interfaces:

Nsf_SensingService

Nsf_EventExposure

* * * End of Changes * * * *