# 3GPP TSG-WG2 Meeting #169	S2-2505314

# Fukuoka, Japan, 2025-05-19 – 2025-05-23

Title:	[Draft] Reply LS on Paging ID Length

Response to:	(S2-2504515/R2-2503197) LS on Paging ID Length

Release:	Rel-19

Work Item:	Ambient_IoT_solutions, AmbientIoT-ARC

Source:	[Ericsson, will be] SA2

To:	RAN2, RAN3, CT4, SA3

Cc:	-

Contact person:	Robbie Ling

robbie (dot) ling (at) ericsson (dot) com

Send any reply LS to:		3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

SA2 thanks RAN2 for the LS on the Paging ID Length.

SA2 discussed the LS, and agreed on the following response:

The AIOTF delivers the Paging ID towards NG-RAN node via “A-IoT Device Identification Requested” IE in “Inventory Request Transfer”.  The “A-IoT Device Identification Requested” IE should be flexible to include the Filtering information which targets to a group of AIoT devices, or the temporary ID which targets to an individual AIoT device.

SA2 assumes the NG-RAN node can get the length of “A-IoT Device Identification Requested” IE and inform the reader about the paging ID length to be used in the A-IoT paging message. SA2 understands such information element definition is under the remit of RAN3. SA2 respectfully asks RAN3 to provide feedback, if any.

For the possible length of Filtering Information, SA2 estimated the maximum length to be 262 bits. SA2 also understands that the length of the Filtering Information varies. It depends on the components included as well as the length of the bitstrings. As the stage 3 work of the Filtering Information is under the remit of CT4, SA2 kindly asks CT4 to continue the stage 3 work, provide feedback on the estimated value or provide a more accurate one.

For the possible length of temporary ID, SA2 understands it is under the remit of SA3, and SA2 cannot provide the estimation now. SA2 would like to respectfully request SA3 to provide feedback.

SA2 would like to kindly request RAN2 to take the above information into account.

# 2	Actions

To RAN3

ACTION: SA2 respectfully asks RAN3 to provide feedback, if any.

To RAN2

ACTION: SA2 kindly asks RAN2 to take the above information into account.

To CT4

ACTION: SA2 respectfully asks CT4 to continue the stage 3 work on Filtering Information, provide feedback on the estimated value or provide a more accurate one.

To SA3

ACTION: SA2 kindly asks SA3 to provide the feedback on the estimation of possible length of the temporary ID.

# 3	Date of next TSG SA WG SA2 meetings

3GPPSA2#170				2025-08-25 – 2025-08-29			Goteborg, Sweden

3GPPSA2#171				2025-10-13 – 2025-10-17			TBC, China