3GPP TSG-SA2 Meeting #169	S2-2505742

Fukuoka, Japan, May 19 – May 23, 2025                                (Revision of S2-2505492, 04681)

# Title:	[DRAFT] Reply LS on RTP retransmission

# Response to:	S2-2504566 / S4-250739

# Release:	Rel-19

# Work Item:	5G_RTP_Ph2, XRM_Ph2

Source:	InterDigital [to be SA2]

To:	SA4

Cc:	RAN2

Contact Person:	Michael Starsinic

Michael.starsinic@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

- Overall description

SA2 thanks SA4 for their LS on RTP retransmissions. In the LS, SA4 asked the following question of SA2:

Question to SA2: Considering the options identified above, are there any benefits to providing application-layer retransmission information to the 5GC when PDU Set based handling is enabled? If so, SA4 would like to receive feedback on what type of application-layer retransmission information would be beneficial for PDU Set based handling in the 5GC.

SA4 also provided in the LS the following information.

- “According to RFC 4588, source and retransmission PDUs associated to a media flow are transmitted in separate RTP streams”.

- “Retransmission PDUs have stricter delay requirements to ensure meeting the decoding deadlines of corresponding ADUs at the receiver”.

SA2 discussed the matter and has identified no benefit for the 5GC in receiving an explicit indication that a flow corresponds to a retransmission stream. Additionally, SA2 would like to provide the following information.

- If two types of traffic (i.e. source stream flows and retransmission stream flows) have different delay requirements, then the following options are already realized by the existing specifications:

  - The AF can provide one set of requirements to the 5GC for both source and retransmissions (i.e. requirements that are based on strictest delay requirement which is the PDB of the retransmission flow). The PCF/SMF would then map both types of traffic to one QoS flow. OR

  - The AF can provide two sets of requirements to the 5GC (e.g. one set of requirements for each SSRC value or port number). The PCF/SMF could then choose to map the flows to different QoS flows or the same QoS Flow.

- It is already possible to map RTP streams to different QoS flows, e.g., based on SSRC as the flow description information can include an SSRC value (see IP Packet Filter Set Rel-19 extensions in clause 5.7.6.2 of TS 23.501). SA2 would like to refer SA4 to the procedure for Setting up an AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502. Please also see clause 6.1.3.27.7 of TS 23.503.

- Actions

To SA4:

ACTION: 	SA2 kindly asks SA4 to consider the information above.

# 3	Dates of next TSG SA meetings

3GPP TSG SA2#170	  	25th August – 29th August, 2025		      Goteborg, SE

3GPP TSG SA2#171	             13th October – 17th October, 2025	      China