3GPP TSG-SA WG2 Meeting #169	S2-2505863

19-23 May 2025 Fukuoka Japan                                                  (was_S2-2505612_4631)

Agenda item:	20.4.1

Source:	NEC

Title:	KI#2 new solution: UE energy saving service

WI/Release:	FS_EnergySys_Ph2 /Rel-20

Document for:	Approval

1. Introduction

This contribution proposes a new solution for KI#2 for the Rel-20 FS_EnergySys_Ph2 study.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

***** First change  *****

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |
| --- | --- | --- | --- |
|  | 1 | 2 | 3 |
| X |  | X |  |

***** End of first change  *****

***** Second change  *****

## 6.x	Solution #X: UE energy saving service

This solution applies to Key Issue #2.

### 6.x.0	High-level solution principles

-	It is assumed that a user is subscribed to one of the available UE energy saving services via the service provider AF or alternatively by the operator, based on the SLA agreement between the operator and the service provider.

-	It is assumed that the PCF holds energy saving policy, for example operator defined, per each available UE energy saving service. Two UE energy saving services, UE energy saving service A and UE energy saving service B, are defined by this solution.

-	It is also assumed that the AF can monitor on the application level the communication with the UE.

Based on certain criteria within the AF,  the AF may requests activation for the UE subscribed UE energy saving service.

Note 1: UE energy saving service is an Energy Saving Subscription as specified in 6.x.1 that can be subscribed by the user.

### 6.x.1	Description

With the development of mobile networks the 3gpp network is expected to accommodate more services like Augmented Reality (AR), Extended Reality (XR) called immersive services which can cause higher energy consumption at the device side. To achieve providing immersive service experience, the 3GPP system is required to provide a long-time high data rate data transmission with low energy consumption in the device. To achieve energy efficiency, the operator will need to introduce the concept of energy as a service and to provide different levels of energy saving services for subscribers. This solution proposes two new UE energy saving services:

- UE energy saving service level A (high price subscription): the network provides and maintains high service experience (e.g., low latency, high throughput).

- UE energy saving service level B (low price subscription): the network provides high service experience initially however, the network may reduce the service experience when this UE service level is activated by the AF (e.g. the network performs QoS degrading).

### 6.x.2	Procedure

Figure 6.x.2-1: UE subscribed UE energy saving service activation

- The AF subscribes for UE energy saving service by invoking Nnef_EnergySavingService_Subscribe Request service operation which includes the UE_Id and the UE energy saving service parameter, i.e. UE energy saving service level A or UE energy saving service level B.

Note 2: It is up to the operator’s policy what UE energy saving methods will be deployed by the operator as part of UE energy saving service level A/B.

Editor’s Note: It is FFS whether the UE energy saving service level A and B can be provided by the UE by implementation.

- The NEF subscribes for UE energy saving service invoking the Nurd_EnergySavingService_Subscribe Request service operation which carries the UE_Id and the UE energy saving service parameters, e.g. UE energy saving service level A or B.

- The UDR stores the UE energy saving service parameter as UE subscription information. Alternatively, the UE energy saving service parameter can be provided by the operator as a result of SLA agreement with the service provider AF in which case the steps 1 to 5 are not required.

- The UDR acknowledges the subscription for the UE energy saving service.

- The NEF acknowledges the subscription for the UE energy saving service.

- The AF may monitor the communication with the UE over the application and based on certain criteria the AF may request for the UE subscribed UE energy saving service to be activated.

- The AF invokes the Nnef_EnergySavingService Request service operation which includes the UE_Id and the UE energy saving service activation parameter.

- The NEF forwards the request from the AF to the UDR via the Nurd_EnergySavingService_Request service operation.

- If the UE is subject to energy saving and the UE has a subscription for a specific UE energy saving service, the UDR indicates the UE_Id and the UE subscribed UE energy saving service parameter, e.g. UE energy saving service level A or B to the PCF.

- The UDR invokes the Nudr_DM_Notify service operation in which the UDR includes the UE_Id and the UE subscribed UE energy saving service parameter.

- The PCF holds UE energy saving policy based on each UE energy saving service, i.e. UE energy saving service level A and UE energy saving service level B. The PCF activates the UE energy saving policy related to the UE subscribed UE energy saving service notified by the UDR.

### 6.x.3	Impacts on existing services, entities and interfaces

AF/NEF:

-	UE energy saving service parameter handling and provision to the UDR.

UDR

-	UE energy saving service subscription information handling.

PCF

- 	activates the requested UE energy saving service subscription policy.

*****  End of second change  *****