3GPP TSG-SA2 Meeting #169	S2-2504664

19- 23 May 2025, Fukuoka, Japan

Source:	China Telecom

Title:	23.700-19: New solution to support ground fallback of UE-Satellite-UE communication

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper propose solution to support ground fallback of UE-Satellite-UE communication, to ensure session continuity.

1Discussion

After the UE-Satellite-UE communication is established, due to satellite or UE mobility the UE-Satellite-UE communication may not be able to continue, e.g. if the the ISL between the UPFs on target satellites is not available, then the UE-Satellite-UE communication should falls back to the ground. During the ground fallback, zero packet loss can be achieved for session continuity. This paper propose a soultion to ensure zero packet loss by using existing procedures for Removal of additional PDU Session Anchor and Branching Point or UL CL.

2 Proposal

It is proposed to agree the solution of support of ground fallback of UE-Satellite-UE communication into TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows

* First change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | Y |
| X |  |  |  |  | X |

* Second change (all New)*

6.X	Solution #X: ground fallback of UE-Satellite-UE communication for KI#Y

6.X.0	High-level solution Principles

The communication path between UEs is changed from between L-UPFs onboard satellite to between PSA UPFs on ground, by using exising procedures for Removal of additional PDU Session Anchor and Branching Point or UL CL through coordinating steps at two UE side.

6.X.1	Description

This solution assumes the PSA UPF is on the ground and UL CL/L-UPF is inserted onboard satellite to support UE-Satellite-UE communication.

When ongoing UE-Satellite-UE communication needs to fall back to the ground, the SMF performs the procedures for removal of UL CL/L-UPF of peer UE, then performs the procedures for removal of UL CL/L-UPF of local UE after the handover completion, in order to ensure zero packet loss during the transition. The UEs can be controlled by single SMF or SMF set.

6.X.2	Procedures

6.X.2.1 Ground fallback of UE-Satellite-UE communication

Figure 6.X.2.1-1: Ground fallback of UE-Satellite-UE communication

- UE-SAT-UE communication is already established between L-UPF1 of UE1 on SAT1 and L-UPF2 of UE2 on SAT1 and UE2. The PSA UPFs for PDU sessions of UE1 and UE2 are on the ground.

- When UE2's serving satellite changes from SAT2 to SAT3, e.g. due to satellite mobility, the exiting procedures for N2 or Xn based handover is performed.

- During the handover the SMF is informed with the target satellite ID(SAT3 ID). The corresponding detail procedure is standardized in R19 UE-SAT-UE IMS call. Then the SMF determines to perform ground fallback, e.g. SMF knows the ISL(s) between SAT1 and SAT3 is not available based on implementation. This coresponds to the scenario already studied in R19 for UE-SAT-UE IMS call.

- The SMF configures PDR&FAR and N6/N19 routing in PSA1 and PSA2 for UE1 and UE2 communication, if they are not configured before. The configuration uses existing 5G VN mechanism. Although the actual traffic from UE2 to UE1 is not routed between PSA2 and PSA1 before step 7 is performed, it can be pre-configured to shorten duration of transition from satellite to ground.

- For UE1, the SMF updates eNB1 for uplink traffic towards PSA1 and updates PSA1 for downlik traffic towards eNB1. The detailed procedures follows step 4-5 of clause 4.3.5.5 of TS 23.502.

After step 4, the traffic paths between UE1 and UE2 are below:

- If UE2's handover procedure is not completed, e.g. if it is N2 based handover, the traffic from UE1 to UE2 is depicted in step 5a, the traffic from UE2 to UE1 is depicted in step 6a.

- If UE2's handover procedure is completed, the traffic from UE1 to UE2 is depicted in step 5b, the traffic from UE2 to UE1is depicted in step 6b.

7.  After the handover completion, the SMF removes ULCL/L-UPF2 of UE2, following step 4-7 of clause 4.3.5.5 of TS 23.502.

8.  The UE-GROUND- UE communication path is established. UL CL/L-UPFs on satellite are not in the communication path.

9.  The SMF removes ULCL/L-UPF1 of UE1, following step 6-7 of clause 4.3.5.5 of TS 23.502.

6.X.3	Impacts on Services, Entities and Interfaces

The following impacts are foreseen by this solution:

SMF:

- Determine to perform ground fallback based the unavailability of ISL(s) between satellites.

- Remove UL CL/L-UPF of peer UE and configure PDR&FAR and N6/N19 routing in ground PSA UPF(s) during the satellite handover of local UE.

- Remove UL CL/L-UPF of local UE after satellite handover of local UE

* End of change *