3GPP TSG-SA2 Meeting #169	S2-2504668

# Fukuoka, Japan, 19 – 23 May 2025

Source: 	SK Telecom, Boost Mobile Network, China Mobile, Deutsche Telekom, KDDI, KPN N.V., NTT DOCOMO, Rakuten Mobile, Verizon

Title: 	New Solution for KI#2: AI/ML-assisted UP traffic pattern and behaviour analysis

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#2 to support AI/ML-assisted UP traffic pattern and behaviour analysis.

1	Discussion

The UPF is responsible for managing substantial volumes of uplink and downlink traffic originating from UEs' applications or application servers, it must be robust and resilient against abnormal data traffic, e.g. large amount of anormal or malicious IP packets, flows, bursts, syn-ack signalling. UPF can generate and collect such information as UP pattern, which can be exposed to NWDAF using AI/ML training and inferencing, and UPF and other NFs can receive analytics output to perform mitigations to improve efficient performance of user plane as well as UPF's robustness.

This paper proposes a solution to the Use-Case#1/Key Issue #2: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane according to clause 5.3 and 5.3.1 in TR 23.2700-04 v0.1.0.

“Use of AI/ML can be studied to provide analytics on user plane traffic patterns and how to use the analytics to improve user plane performance. In this context, it is proposed to study:

-	How to provide analytics (e.g. enhancing existing in TS 23.288 [5] or introducing new) including the aspects of required inputs/outputs to improve performance of user plan.

-	Which consumers can subscribe/request the analytics and how the consumers use the analytics to take actions to improve user plane performance.

-	Whether and how to train ML models and perform the inference for the user plane analytics, considering solutions that minimize the load on Control Plane NFs.”

2. Proposal

It is proposed to add the following contents to TR 23.700-04.

Start of Change

6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 |  | x | x | x |
| #2 |  |  |  |  |

Next Change (all new text)

6.X	Solution #X: AI/ML-assisted UP traffic pattern and behaviour analysis

6.X.1	High level principles

The high level principles of the solution are:

- The consumer (e.g. UPF and SMF) subscribes/requests "UP Traffic Pattern" analytics to the NWDAF.

- The NWDAF provides Output Analytics from existing and new Input Data from UPF and SMF about the UP pattern data collection based on observed UPF's packet handling.

- Based on the Output Analytics, the consumer executes mitigation adjustments.

- To reduce the load on Control Plane NFs due to excessive reports from UPF, consumers should utilise Analytics Filters, reporting intervals, measurement duration, and thresholds when subscribing/requesting to NWDAF. Additionally, UPF should collect and generate UP pattern data based on threshold limits and the maximum number of anomaly and application traffic information.

6.X.2	Description

This solution is for Key Issue#2 for AI/ML-assisted UP traffic pattern and behaviour analysis which the NWDAF is used to train and infer AI/ML models from UPF data and, to provide analytics on user plane traffic patterns, to apply mitigation strategies using analytics, e.g. avoid abnormal or malicious traffic, so as to improve efficient performance of user plane as well as UPF's robustness.

Since UPF processes all UP packets, it has the ability collect and generate a UP pattern data and report to NWDAF based on what is monitors and observes most efficiently accordingly to the PFCP rules and its local configurations.

- The UP pattern data is a collection of packet information within the measurement duration, e.g. it attempts to capture any potential anomalies locally observed during the measurement period based on primitive thresholds or rules configured by operator. If at least one anomaly is supposedly found, it should list such related information, e.g. unexpected flows and packets, and also obtain those related Application(s) made up of n-tuple IP flow(s).

- Individual per IP flow information is obtained such as total number of packets, volume, directionality, basic stats, packet discarded, buffered information along with associated UPF's resource. Optionally, UPF may have AnLF features to lightly perform an inference to aid UP pattern data.

UPF and SMF as consumer, receives a response or notification from the NWDAF's Output Analytics of statistics or predictions. For prediction example, it contains severity or risk level about UP anomalies predicted, along with a list of predicted anomalies of the UP pattern data, each with pattern type, e.g. DDoS, burst, overload type, e.g. Interface queue full in 5s, and suggested UPF configuration for mitigation adjustments.

- Mitigation for the SMF shall use SMF's control on UP Management such as (re)selection. Mitigation to UPF shall use UPF's configuration related to packet processing, e.g. buffer capacity, packet drop, rate-limit, resource allocation, load reductions, etc that are not controllable or available from the SMF.

- It should also reveal which Application caused such predicted anomalies so that UPF and SMF can use this information to adjust configuration. NWDAF should provide some pre-defined configured local rules or process for the UPF.

- Load on Control Plane NFs should be monitored and have a separate resource allocation and thresholds for this Analytic, e.g. Analytics Filter, reporting periods, maximum number of UP pattern data’s Anomalies, including, e.g. reporting thresholds, receiving only top-N most severity ones, etc.

6.X.3	Procedures

6.x.3.1 UP pattern data collection from UPF as Input Data to NWDAF

Figure 6.x.3.1-1: UP Pattern data collection by the NWDAF for analytics

1.	The consumer NF, e.g. UPF and SMF subscribe to or sends a request to NWDAF for the "UP traffic pattern" analytics using either Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation. The request additionally includes thresholds such as confidence level.

-	The Analytics ID is set to "UP Traffic Pattern" analytics. The target for analytics reporting is set to be any UE or group of UEs. Analytic Filter, e.g. AoI and/or S-NSSAI and/or DNN, can be provided to reduce Control Plane signalling.

-	The consumer NF can request statistics or/and predictions for a given Analytics target period.

2.	The NWDAF retrieves Input data from UPF and NFs, using Nnf_EventExposure_Subscribe, or from NRF using Nnrf_NFManagement_NFStatusSubscribe service operation.

3.	The NWDAF derives the required analytics with the collected data.

3-a. (Optional) Depending on the derived result, the NWDAF may request to further collect historical or additional UP data to DCCF and NFs to increase accuracy of statistics and prediction

3-b. (Optional) The NWDAF derives the required analytics with the collected data.

4.	The NWDAF invokes Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response to the consumer NF for the Output data analytics.

5.	The consumer (e.g. UPF or SMF) upon receiving the statistic and prediction may execute based on the mitigation mechanisms.

-	To reduce load on Control Plane NFs, UPF can take certain autonomous actions without waiting for instructions from the SMF or AF to optimize UP handling such as protecting from anomaly traffic based on UPF's internal logic.

-	Mitigation mechanisms for the UPF autonomously to achieve performance enhancement and robustness can be, e.g. adjustment of UPF buffer capacity, packet drop, rate-limits, resource allocation, load reduction.

The NWDAF collects the Input Data for the UP Pattern data as listed in Table 6.x.3-1.

Table 6.x.3-1: UP Pattern data collected by NWDAF

| Information | Source | Description |
| --- | --- | --- |
| UE ID | SMF, UPF | Identifier of the UE (e.g. SUPI) associated for the UP pattern data |
| UE IP Address | SMF, UPF | UE IP address associated for the UP pattern data |
| S-NSSAI | SMF, UPF | Network Slice associated for the UP pattern data |
| DNN | SMF, UPF | Data Network Name |
| PDU Session ID | SMF | PDU Session identifier |
| UPF Node ID | UPF | IP address or FQDN of the UPF |
| F-SEID | SMF, UPF | SMF or UPF side of Fully Qualified SEID |
| UP pattern data | SMF, UPF | User Plane pattern data collection |
| > Measurement duration | SMF, UPF | The measurement duration for the UP pattern (timestamp of start and end time) |
| > UPF interface info | SMF, UPF | Identifier and address of the UPF interface where UP pattern is observed (e.g. N3 for RAN side, N6 for DN side). |
| > PFCP session info | SMF, UPF | Summary of PFCP session context (e.g. PDR/FAR/QER/URR/BAR) status and any PFCP errors per clause 7.6 of TS 29.244 for the UP pattern. |
| > Delay info | SMF, UPF | Delays observed at N6 (N6 Delay), N3/N9 (GTP-U Path), N4 (PFCP HB) Interface per clause 5.33.8, 5.24.5, 6.2.2 of TS 29.244 for the pattern. |
| > Sampled info | UPF | Information related to the traffic sample rate, if sampling is used. |
| > Anomaly info (0..max) | UPF | List of any anomalies locally observed for the UP pattern during the measurement period based on thresholds or rules configured by operator at UPF (each entry describes one anomaly observation). |
| >> Unexpected flow info | UPF | Anomaly information regarding a traffic persisting unusually long/sort or at very high/low throughput relative to expected usage patterns​. For examples, Long duration >60 secs, Short duration: <1 sec, High rate: > 5000pps, Slow rate: <0.1pps. It can also be a high/low frequency of new application traffic initiations or service access attempts by the UE or server, e.g. DDoS behaviour, unexpected traffic access outside of its normal usage. |
| >> Unexpected packet info | UPF | Anomaly information regarding traffic targeting an unexpected or unauthorized packet address (e.g. IP, protocol, port, prefix, domain) and packet string (e.g. headers, fields, options) not typically accessed by the UE.​ |
| > Application traffic info (1..max) | UPF | List of observed Application traffic information communicating with the UE based on thresholds or rules configured by operator at UPF (each entry describes one flow observation). |
| >> Per-flow info | UPF | IP flow represented and aggregated by n-tuples, e.g. SrcIP, DstIP, SrcPort, DstPort, Protocol, uni/bi-directionality, URL, total number of packets, their total volume in size and duration. |
| >> Per-flow stats info | UPF | Quartiles and statistics such as average packet size. |
| >> Per-flow discarded packet info | UPF | Discarded packets including total number of packets, discarded reasons. |
| >> Per-flow buffered info | UPF | Buffered or queued packets including total number of buffered packets, average buffered time, buffered reasons. |
| >> Per-flow temporal behaviour | UPF | Temporal packet behaviour including inter arrival time, jitter, latency related information. |
| >> Per-flow resource info | UPF, NRF | Resources related to load, e.g. CPU, Memory, Energy, processing delay, associated for processing the flow. |
| > OPTIONAL traffic inference information | UPF | Analytics-derived insights provided by a UPF with embedded analytics logic (AnLF) if the UPF has AI/ML capabilities or models from NWDAF to classify traffic. |

Editor's Note: Input Data collections are to be further updated during the normative phase.

The NWDAF notifies/responses the Output Analytics to the consumer NF as listed in Table 6.x.3-2 and 6.x.3-3.

Table 6.x.3-2: Output Analytics of UP Traffic Pattern statistics

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Severity | Identified severity/risk level, e.g. Information, Warning, High, Critical |
| > Anomaly info (0..max) | Identified list of detected anomalies analysed in the UP pattern data, each entry describes one identified anomalies analysed. |
| >> UP pattern type | UP pattern type, e.g. DDoS, misbehaving server/UE, burst, etc |
| >> Overload type | Detected overloads, e.g. CPU overloaded, NIC queue full, etc |
| >> UPF configuration | Configuration used for the collection of the statistics, e.g. buffer capacity, packet drop, rate-limits, resource allocation, load reduction adjustments. |
| >> Application traffic info (0..max) | Identified list of detected Application traffic information in the UP pattern data. |
| >>> IP packet filter set | IP packet filter set |

Table 6.x.3-3: Output Analytics of UP Traffic Pattern predictions

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Predicted severity | Predicted severity/risk level, e.g. Informative, Warning, High, Critical |
| > Predicted anomaly info (0..max) | Predicted list of anomalies analysed in the UP pattern data, each entry describes one identified anomalies analysed. |
| >> UP pattern type | Predicted UP pattern type, e.g. DDoS, misbehaving server/UE, burst, etc . |
| >> Overload type | Predicted overloads, e.g. CPU overloads in 20s, queue full in 5s, etc. |
| >> UPF configuration | Configuration suggestions to mitigate, to improve performance and/or reliability of the UPF, e.g. buffer capacity, packet drop, rate-limits, resource allocation, load reduction adjustments. |
| >> Predicted application traffic info (0..max) | Predicted list of Application traffic information in the UP pattern data. |
| >>> IP packet filter set | IP packet filter set |
| > Confidence | Confidence of this prediction. |

Editor's Note: Output Analytics (statistics and predictions) are to be further updated during the normative phase.

6.X.4	Impacts on services, entities and interfaces

UPF:

-	Collects UP pattern data upon request from NWDAF.

-	Sends the UP pattern data as Input Data for reporting to NWDAF.

NWDAF:

-	Supports providing subscriptions and/or requests of the "UP Traffic Pattern" analytics.

-	Supports deriving statistics and/or predictions for the consumer NF's requests.

Consumer NF:

-	Supports subscribing /requesting "UP traffic pattern" analytics to NWDAF and receives notification/response from the NWDAF.

-	Supports executing the UPF performance enhancement based on the Output Analytics from NWDAF.

End of Changes