3GPP SA WG2#169	S2-2505042

Fukuoka, Japan, 19-23 May, 2025

Source:	Tejas Networks Limited

Title:	Enhancements to A-IoT Inventory Service

Spec:	3GPP TS 23.369

Agenda item:	19.14.2

Work Item / Release: AmbientIoT-ARC / Rel-19

Document for:	Discussion and decision

Abstract of the contribution: This contribution is to discuss the enhancement of the semantics of the AIoT Inventory Procedure for enabling more efficient operation.

1. Introduction

3GPP TS 23.369 defines two AIOTF services viz., the Inventory service and Command service. Inventory service is used to discover the list of operational AIoT devices that are present in the network and obtain their identification (i.e., AIoT device permanent IDs). Once the list of operational devices is obtained by AIOTF, Command procedure may be used to perform a device-specific operation such as read, write and disable. However, these procedures need to be enhanced to improve the usage of radio resources and enable error handling. To better understand the need for these enhancements, let us consider the following scenario illustrated in Figure 1., wherein an AIOTF wishes to alter the configuration of two specific devices namely, Device 1 and Device2 for an Inventory cum follow-on command procedure.

Figure 1: Example scenario for Inventory procedure with a follow-on command.

The following set of actions occur within the network for processing this request. The AIOTF sends an Inventory Request with a Follow-onCommand as per the procedure described in clause 6.2.3 of 3GPP TS 23.369.

- As per this procedure, the AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Device Permanent Identifier, and assistance information to the selected NG-RAN node.

- On receiving the Inventory Request message, the RAN readers associated with the NG-RAN transmit a paging message that includes the AIoT Device Permanent Identifier. If the AIoT Device Permanent Identifier in the paging message matches that of Device 1 and Device 2, they respond to the NG-RAN paging request and send an Inventory Report (NAS message) including their device IDs to the AIOTF.

- On receiving the Inventory Report from these two devices, the AIOTF can proceed to send out a Command Request with a “write” operation.

We can make the following observation from the above scenario:

[Observation-1]]: While the main objective of changing the configuration of specific devices has been achieved by AIOTF, the Readers are still sending out an Inventory Requests with a follow-on command to the remaining devices. This is due to the fact that the Inventory Request procedure is not yet deemed to be complete as it is yet to aggregate the response from X% (X can be 90%, 95%, 99% based on the application/use case requirement) of the devices in the network.

This results in resource wastage as the remaining “Inventory Report(s)” are not relevant to the AIOTF, especially when the number of devices in the network are large. Moreover, since RAN readers can process only one request at a time, they are blocked from processing other priority requests.

[Proposal1]: Extend the semantics of the Inventory service to introduce an “Inventory Cancellation” Procedure illustrated Figure 2 in comprising an “Inventory Cancellation Request” and “Cancellation Status” messages.

- As per this procedure, the AIOTF sends an “Inventory Cancellation” message including the Correlation ID, Application Function Id (which had requested the procedure) to the selected NG-RAN node as shown in Figure 2.

- The NG-RAN, halts any further paging and sends “Cancellation Status” message comprising status information (whether the cancellation was successful or not), Criticality diagnostics (in-case of unsuccessful cancellations), Correlation ID, Application Function Id back to the AIOTF.

Figure 2: Inventory Cancellation Procedure- Proposal 1

[Proposal2]: Extend the semantics of the Inventory service to introduce an “Inventory Cancellation” Procedure illustrated in Figure 3 comprising “Inventory Cancellation Request” and “Cancellation Acknowledge” messages.

- As per this procedure, the AIOTF sends an “Inventory Cancellation” message including the Correlation ID, Application Function Id (which had requested the procedure) to the selected NG-RAN node as shown in Figure 5.

- The NG-RAN, halts any further paging and sends “Cancellation Acknowledgement” message comprising status information (whether the cancellation was successful or not, Correlation ID, Application Function Identifier back to the AIOTF.

Figure 3: Inventory Cancellation Procedure-Proposal 2.

Note: The format of the messages for Inventory Cancellation would be decided by RAN3.