SA WG2 Meeting #169	S2-2505169

19 - 23 May, 2025, Fukuoka, Japan	(revision of S2-25xxxxx)

Source:	ZTE, Futurewei, Nokia, LG Electronics, Lenovo, InterDigital Inc., CATT, China Mobile

Title:	pCR to TS 23.369: AIoT Service Operation Result Aggregation

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper adds AIoT service operation result aggregation principal.

1. Discussion

The TR 23.700-13 has concluded the aggregation of AIoT operation responses received from multiple AIoT Devices can be performed by NG-RAN based on the aggregation assistance information received from the AIOTF. This paper adds AIoT service operation result aggregation principal as a high level feature in clause 5 of TS 23.369.

2. Proposal

It is proposed to capture the following changes into TS 23.369.

* * * * Start of Changes * * * *

5.X	AIoT Service Operation Result Aggregation

An AIoT service operation with a specific Correlation ID may involve many AIoT Devices. The NG-RAN may perform AIoT service operation result aggregation based on the aggregation assistance information received from the AIOTF as specified in clause 5.4.

The AIOTF determines the aggregation assistance information based on the request from the AF or local configuration, which includes:

-	Time interval: the fixed time interval for which NG-RAN collects multiple AIoT Devices’ operation responses before reporting the aggregated AIoT response to the AIOTF. The reporting based on time interval may potentially happen multiple times until the NG-RAN completes the request operation.

If the AF has provided a time interval, then the AIOTF should signal a time interval to the NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

If the AIOTF does not provide aggregation assistance information, the aggregation process in the NG-RAN may be determined by implementation.

The AIOTF may also perform AIoT service operation result aggregation for the service results that share the same correlation identifier before sending the result to the NEF or the trusted AF.

* * * * Next Change * * * *

5.4	Assistance information provided to NG-RAN node

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests:

a)	AIoT service type (e.g. Inventory, Command).

b)	Optionally, approximate number of AIoT devices based on AF request.

c)	Optionally, approximate D2R message size based on AF request.

d)	Optionally, time interval as AIoT aggregation assistance information.

Editor's note:	Other assistance information may be added later if necessary.

If not provided by the AF, bullets b) and c) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

The assistance information is used by the NG-RAN for performing service operations, e.g. radio resource allocation by using bullets b) and c), AIoT Device responses aggregation by using bullet d).

* * * * End of Changes * * * *