# 3GPP TSG-WG SA2 Meeting #169															S2-2505353

# Fukuoka, Japan, May 19 – 23, 2025

Source:	OPPO

Title:	Discussion on routing ID and LCS-UP connection mapping for multiple LCS-UP connections

Agenda item:	9.6.2 (5G_eLCS_Ph3)

Document for:	Decision

1. Introduction

As agreed in S2-2504088, in Rel-19 multiple LCS-UPP connections are supported, but how to map the routing ID and the proper LCS-UP connection is not resolved.

Currently two alternative ways forward are identified, one is to make the assumption that the UE can only sends Uplink LCS-UP packet towards LMF within the asme LCS-UP connection when LMF firstly sends Downlink LCS-UP packet towards the UE, the other is to notify the UE about the correlated routing ID or deferred routing ID with correlated UP connection identifier. The details and impacts of the two alternatives are discussed as follows.

2. Discussion

As specified in 6.18.1 and 6.18.2 of TS 23.273, both LMF and UE can decide to utilize the user plane connection for positioning to transfer LPP messages with the following descriptions:

“If LMF or UE determines to utilize the user plane connection for positioning and the secure user plane connection is established, LPP messages are transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data.”

“After the secure user plane connection is established, if LMF determines to utilize the user plane connection for positioning after receiving a positioning request from AMF, or UE determines to utilize the user plane connection for positioning, LPP messages are transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data.”

Thus it is possible that the UE receives routing ID or deferred routing ID via control plane DL NAS message and decides to utilize the user plane connection to send uplinke positioning messages. This is supported in CT1’s specification TS 24.572 clause 7.3.2.2 with the following descriptions:

“The UE initiates the uplink LCS-UP transport procedure by sending the UL LCS-UP TRANSPORT message to the LMF, as shown in figure 7.3.2.2.1.

In case a) in subclause 7.3.2.1, the UE shall:

-	set the LCS-UP payload type IE to "LTE Positioning Protocol (LPP) message";

-	set the LCS-UP payload IE to the LPP message(s) payload; and

-	set the LCS session identity IE to the routing identifier or the LCS session identity received in the DL LCS-UP TRANSPORT message.

In case b) in subclause 7.3.2.1, the UE shall:

-	set the LCS-UP payload type IE to "LCS supplementary services message";

-	set the LCS-UP payload IE to the LCS supplementary services message payload; and

-	set the LCS session identity IE to:

1)	the routing identifier or the LCS session identity received in the DL LCS-UP TRANSPORT message for the PeriodicTriggeredInvoke operations; or

2)	the deferred routing identifier or the LCS session identity received in the DL LCS-UP TRANSPORT message for the MSCancelDeferredLocation operations and the EventReport operations.

”

Note that for Rel-18, since only one LCS-UP connection is supported at the UE side, when the UE decides to utilize user plane to transmit the positioning messages, the UE just set the LCS session identity IE to routing ID or deferred routing ID and sends the LCS-UP messages via the only LCS-UP connection. This feature is natually supported in Rel-18.

Observation #1: It is natually supported in Rel-18 to allow UE decide and intiate uplink LCS-UP messages for the on-going control plane positioning sessions.

However, for Rel-19, since the UE can have multiple LCS-UP connections with different LMF, when the UE decides to utilize user plane to transmit the postioning messages, the UE cannot choose the proper LCS-UP connection towards the proper LMF based on current specification.

Observation #2: In Rel-19, the UE cannot choose the proper LCS-UP connection towards the proper LMF based on current specification.

There are two approaches to resolve this issue for Rel-19:

- Make the assumption that the UE can only sends Uplink LCS-UP packet towards LMF within the same LCS-UP connection when LMF firstly sends Downlink LCS-UP packet towards the UE. In this approach, the UE should not decide to utilize user plane itself. The UE should always send the UL traffic within the same LCS-UP connection after the UE receives the DL LCS-UP traffic. Based on this assumption, TS 23.273 should be updated to clarify that in Rel-19 only LMF can decides to utilize the user plane positioning and the UE should follow the LMF’s decision. CT1’s TS 24.572 should also be updated to remove the “routing identifier” in clause 7.3.2.2 and update the “deferred routing identifier” to “LCS correlation ID” since this information can only come from the LMF via user plane without going through AMF (routing ID and deferred routing ID is only used in NAS messages and LMF uses LCS correlation ID).

- When establishing LCS-UP connection, the LMF sends the existing correlated LCS correlation ID along with the LMF UP information (e.g.IP address or LMF identifier) towards the UE and after that if the UE decides to utilize User plane positioning, the UE can select the proper LCS-UP connection using the correlated LCS UP information for the Uplink LCS-UP messages.

Based on the information above, we can have the observations as follows:

Observation #3: Approach 1) will drop the feature in Rel-19 and it is not future proof.

Observation #4: For Approach 2) additional LCS correlation ID should be carried with the LMF UP information for UE to map routing ID and LCS-UP connection. This approach is future proof.

Conclusion: Based on the observations and information above, it is proposed to use approach 2) as way forward.

3. Proposal

Update clauses 6.18.1 and 6.18.2 of TS 23.273 to specify that the LMF provides the UE with LCS-related User Plane information and the correlated LCS Correlation Identifier, enabling the establishment of mapping between the Routing Identifier (LCS correlation ID) and the LCS-UP connection to support UE-initiated Uplink LCS-UP Traffic Transmission in Release 19.

Proposals above are provided in S2-2505355