3GPP TSG-SA WG2 Meeting #169	S2-2505547

19 – 23 May, 2025, Fukuoka, Japan 	S2-2505161

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6256 | rev | 6 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Energy consumption information for access type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Samsung, NTT DOCOMO], Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is not clear how energy consumption information collected for the different access type. The 3GPP system support the following scenario with the N3GPP Acess type: -single access PDU session established ver N3GPP Access type - Handover of 3GPP/N3GPP for Single access PDU session - MA PDU which may have 1 leg on 3GPP access and 1 leg on N3GPP Access In this relase of specification we intend to consider only the Energy Consumption related to the 3GPP Access, i.e. when the service node is a gNB. In case of single Access PDU session the SMF needs to reports the traffic related to the PDU session when the service access node is the gNB, while when is a N3IWG or TNGF the serving node and the related DV shall be not reported to the EIF. In case of MA PDU the scenario is more complicated since the following case can occur: - only 3GPP leg is active, so the DV and the serving gNB shall be reported - only N3GPP leg is active, so the DV and the serving gNB shall be not reported - both 3GPP and N3GPP legs are active, so only the so the DV transmitted over the N3 interfacer toward the serving gNB shall be reported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Ensure that the energy consumption information is collected only for the 3GPP access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.1, 5.51.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.51.2.1	General

The Energy Information Function (EIF) is defined to:

-	collect the UE related Energy Consumption information;

-	calculate the Energy Consumption information at UE, S-NSSAI, PDU Session and Service Data Flow (e.g. per UE per application) granularity; and

-	expose the Energy Consumption information to the authorized consumer NF(s) (AF/NEF or 5GC NF).

Procedures for energy consumption information collection and exposure are defined in clause 4.29 of TS 23.502 [3].

In this release of the specification, collecting and calculating of UE related Energy Consumption information is supported only for the UP resources of the 3GPP access type serving the UE in the 5GS.

# * * * * Next change * * * *

##### 5.51.2.2.2	Energy Consumption information collection from SMF

The serving SMFs are retrieved by the EIF from the UDM of the UE based on the provided input parameters including the UE ID and (S-NSSAI, DNN).

The EIF invokes Nsmf_EventExposure_Subscribe as defined in TS 23.502 [3] with the required granularities (UE ID, DNN/S-NSSAI, application information (e.g. Application Identifier, or Packet Filters)) to retrieve the information from SMF, which is shown in Table 5.51.2.2.2-1.

The SMF receives the data volume of the required granularities from the PSA UPF. The SMF then sends the collected data volume, along with the serving gNB ID(s) and (I-)UPF ID(s) to EIF for energy consumption calculation. And the information collected from SMF by EIF, is shown in Table 5.51.2.2.2-2. The SMF shall report the energy related information only for the PDU session  carried over the 3GPP Access and the EIF shall consider the lack of reporting equivalent to UL/DL Data volume in the measurement period equal to 0 (zero) therefore the corresponding EC is 0 (zero).

NOTE 1:	When the PDU session is in the measurement period is carried over 3GPP access for a period of time and on Non-3GPP over a period of time due to handover, the SMF reports  the energy related information corresponding to the period of time for which the PDU session was on 3GPP access.

NOTE 2: 	In case of MA PDU session, the SMF does not report the the energy related information related to the leg over Non-3GPP access,.

Table 5.51.2.2.2-1: Information to SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE ID | SUPI. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| IP 5-Tuple | IP-5-tuple. |

NOTE 1:	The user-plane energy consumption information reporting interval from the SMFs is the PLMN-wide configurable starting time and interval T.

Table 5.51.2.2.2-2: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI. |
| S-NSSAI | Network Slice applicable to a UE |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| Packet Filters | Packet Filters as in clause 5.7.6 for IP or Ethernet traffic. |
| Application Identifier | Identification for the traffic of the service data flow. |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time interval. |
| > UL/DL Data Volume | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > gNB ID | Identifier of the gNB serving the UE. |
| Reference to Time Interval | Indicate the time interval of the collected information (e.g. time stamps). |

NOTE 2:	Each entry of List of Data Volume information represents the serving gNB and UPF(s) corresponding to the same Data Volume regarding the required granularities. A new entry is added when the gNB or UPF(s) is changed.

# * * * * End of changes * * * *