SA WG2 Meeting #169	S2-2505559

Fukuoka, Japan, 19-23 May, 2025

# Title:	D2R message size for inventory response

# Source:	InterDigital Inc.

# Document for:	Approval

Agenda Item:	19.14.2

Abstract:	The pCR proposes to align with RAN2 assumption by changing “D2R message size of inventory response” in assistance information to mandatory.

1. Discussion

In the LS S2-2504517(R2-2503199), RAN2 has indicated that they assume inventory response size can always be provided by the core network to the reader. This is necessary as inventory response message is not to be segmented so the reader needs to know the size of the message so enough resource can be allocated.

The main content of the inventory respose is the device identifier. According to SA3 conclusion, Temporary ID will be used for privacy protection. Though the mechanism to generate the Temporary ID is still under the discussion of SA3 group, it can be trusted that the CN (AIoTF) will be able to know the size of the Temporary ID. Thus, SA2 should accept RAN2’s assumption and align its specifications with this assumption.

2. Proposal

It is proposed to adopt the proposed change in TS 23.369.

* * * * First Change (All new text) * * * *

## 5.4	Assistance information provided to NG-RAN node

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests:

a)	AIoT service type (e.g. Inventory, Command).

b)	 D2R message size for inventory response.

c)	Optionally, approximate number of AIoT devices based on AF request.

d)	Optionally, approximate D2R message size based on AF request.

e)	Optionally, time interval as AIoT aggregation assistance information.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e. in this clause or in another clause.

If not provided by the AF, bullets c) and d) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

If the AF has provided a time interval, then the AIOTF should set bullet d) to NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

The assistance information is used by the NG-RAN for performing service operations, e.g. radio resource allocation by using bullets b), c) and d), AIoT Device responses aggregation by using bullet e).

* * * * End of Change * * * *