3GPP TSG-SA WG2#169	S2-2505576

Fukuoka, JP, 19 - 23 May 2025	(was S2-2504592)

Source:	Qualcomm Incorporated

Title:	TR 23.700-14: FS_Sensing_ARC new solutions on sensing service operations

Document for:	Discussion/Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposed a new solution on the sensing service operations.

# 1.	Discussion

## 1.1 scope of the solution

According to the agreed TR 23.700-14v0.1.0 from SA2#168, there are six key issues to address for the architecture enhancement to support sensing. In this contribution, a solution is proposed to support the sensing service using the SBA like operation models. It has relevance to KI#1 (system architecture), KI#3 (sensing entity and sensing function discovery and (re)selection), KI#4 (Sensing data and the associated information collection and transport), and KI#5 (sensing result exposure).

## 1.2 Issues to address

Based on the existing defined entities for sensing operations, the architecture needs to support efficient interactions between the Sensing Entities (i.e. the Sensing Transmitter or Receiver) and the Sensing Function(s) in the core network to accomplish a sensing service operation. The examples of such interactions include, for example the discovery and configure the sensing entity (as in KI#3 and KI#6), collection of the sensing data (as in KI#4), or exposure of the sensing results (when the result is directly provided by Sensing Entities), etc.

Among the tasks, the collection of sensing data (related to KI#4) would require some architecture enhancements. Without assuming any sensing mode or system architecture, the Sensing Entities would reside in RAN or being a UE, and the Sensing Function(s) would be in the CN. Therefore, the traditional architecture would require their interactions to go through some control plane nodes, e.g. in 5G system via the AMF.

However, for sensing service operations, the sensing data (and the associated information) is too heavy for such control plane based interactions. Although, there was no design for ISAC from 3GPP RAN WGs, some estimations can be made based on other known sensing technologies. For example, if similar measurements results were to be collected as that of IEEE802.11bf based sensing [1], the sensing data can be up to 3.2 Mbps per antenna (e.g. assuming to track a 100 Hz micro-doppler using the CSI report as defined in Table 9-129K). For any Sensing Entity that uses more than one antenna, the data rate would multiply. If we assume some other type of (processed) sensing data from the Sensing Entity for a different application, e.g. the point clouds for automotive use, the data rate can be even higher. Based on [2], an automotive LiDAR can generate 9.6M points/second point cloud. Assuming 4 Bytes per point (Range, Azimuth, Elevation, and Doppler), that is equivalent to ~300Mbps. Even with different compression methods applied, the equivalent data rate to be supported still exceeds the capacity of the normal control plane connections.

In addition, when there are different sensors to be supported in the same area for a particular sensing service operation, e.g. multiple RAN nodes were used to track the UAVs, the amount of the data to be sent to the same set of Sensing Function(s) would also multiply.

As there was no QoS mechanism to differentiate the control plane traffic, the sensing data would interfere with other control signalling for the communication services, e.g. mobility management, PDU session management, etc. Even if the control plane is enhanced to support larger data volume, the latencies for the signalling for UE or RAN management may increase significantly. Such impacts to the basic network operations are not acceptable.

Besides the transportation load, the sensing data also causes additional processing burden on the control plane nodes. Different from a user plane node that may forward traffic simply based on IP routing information, the control plane nodes need to process each of the packet according to the control plane headers. This design was not meant to handle large volume high speed data. For example, an AMF would need to handle gigabits of data per second if there are more than 10 Sensing Entities in its controlled region. Such design is not scalable.

Observation 1: Sensing data is too heavy for control plane connection (e.g. NGAP) and control node (e.g. AMF), which was designed to handle only control signalling.

In view of the above issues, it is desirable to support a direct interface between the Sensing Entity (i.e. the source of sensing data) to the Sensing Function(s).

Proposal 1: Direct connections between Sensing Entities (sensing data source) to the Sensing Function should be supported.

The details of the proposal to support the sensing operation are described below.

## 1.3 Solution proposals

### 1.3.1 General principles of the solution

As established above, in order to avoid impacts to the general network operations and other type of services, the Sensing Entity and the Sensing Function(s) (in the core network) should use a direct interface for their interactions, e.g. sensing data collections.

This direct interface needs to support the following requirements:

-  	It needs to support transporting large amount of data without affecting other services for UEs. For example, this may require a separate bearer if a UE is involved, or a separate tunnel if a RAN node is involved.

-	It needs to satisfy the QoS requirements for the sensing service operation. For example, the sensing service operation may have a delay requirements for the result generation. The direct interface needs to have a mechanism to control and ensure the fulfillment of such requirements.

-	It needs to allow the reuse of the interface for different Sensing Functions. For example, the Sensing Entity may be utilized by different Sensing Functions in different Sensing service sessions.

-	It needs to support the Sensing Function(s) to control and configure the Sensing Entity.

-	It needs to provide security protection for the communications between the Sensing Entity and the Sensing Functions.

Based on the requirements, the solution is designed with the following general principles:

-	The Sensing Function performs a selection of the Sensing Entity(ies) based on the sensing service request.

-	The Sensing Function triggers each of the selected Sensing Entities to establish a direct user plane connection, if the connection was not yet available.

-	The Sensing Function uses the direct user plane connection to request the sensing service operation at the Sensing Entity(ies), and obtaining the Sensing Data and Associated information from the Sensing Entity(ies).

### 1.3.2 Establishment of the direct user plane connections

Obviously, for different types of Sensing Entities, the direct user plane connection establishment would be different. For example, when a Sensing Entity is a UE, there were already examples of user plane based control interfaces, e.g. the LCS-UPP as defined in TS 23.273 for location services. Similar mechanism for the user plane interface establishment and management can be used for sensing services.

When the Sensing Entity is a RAN node, there is a need to define a new mechanism to establish the direct interface between the Sensing Entity and a core network Sensing Function. It should be noted that the new interface does not have to support UE associated control, and therefore could be designed in a future proof manner.

Additionally, given that the study is to focus on the generic solution, the solution should also consider potential future implementation options, and not limited to the existing system architecture. With that in mind, the solution proposed to cover the three different options of the interfaces between the Sensing Entity and the Sensing Functions, i.e. a point-to-point protocol similar to that of NRPPa; a service based interface provided by the Sensing Entity; and a service based interface provided by the Sensing Function.

Depending on the system supporting the sensing services, different options above could be implemented for the sensing operations.

Reference:

[1] R. Roriz, J. Cabral and T. Gomes, "Automotive LiDAR Technology: A Survey," in IEEE Transactions on Intelligent Transportation Systems, vol. 23, no. 7, pp. 6282-6297, July 2022, doi: 10.1109/TITS.2021.3086804.

# 2.	Text proposal

It is proposed to agree the following changes to TS 23.700-14:

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[x]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[y]	3GPP TS 37.355: "	LTE Positioning Protocol (LPP)".

[z]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

# >>>>NEXT CHANGE<<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| #X | X |  | X | X | X |  |

# >>>>NEXT CHANGE<<<<

## 6.X	Solution #X: Support of Sensing operations with direct interfaces

### 6.X.0	High-level solution Principles

The solution is based on the following general principles to support sensing service:

-	The Sensing Function performs a selection of the Sensing Entity(ies) based on the sensing service request.

-	The Sensing Function triggers each of the selected Sensing Entities to establish a direct connection with the Sensing Function, if the connection was not yet available.

-	The Sensing Function uses the direct connection to request the sensing service operation at the Sensing Entity(ies) and obtain the Sensing Data and Associated information from the Sensing Entity(ies).

It addresses the KI#1, KI#3, KI#4, and KI#5.

### 6.X.1	Description

In this solution, in order to avoid impacts to the general network operations and other services, the Sensing Entity and the Sensing Function(s) (in the core network) establish a direct interface for their interactions, e.g. sensing data collection, via a direct connection.

This direct interface between the Sensing Entity and Sensing Function supports the following requirements:

-  	It needs to support transporting large amount of data without affecting other services for UEs. For example, this may require a separate bearer if a UE is involved, or a separate tunnel if a RAN node is involved.

-	It needs to satisfy the QoS requirements for the sensing service operation. For example, the sensing service operation may have a delay requirements for the result generation. The direct interface needs to have a mechanism to control and ensure the fulfillment of such requirements.

-	It needs to allow the reuse of the interface for different Sensing Functions. For example, the Sensing Entity may be utilized by different Sensing Functions in different Sensing service sessions.

-	It needs to support the Sensing Function(s) to control and configure the Sensing Entity.

-	It needs to provide security protection for the communications between the Sensing Entity and the Sensing Functions.

Based on the requirements, the solution follows the below general principles:

-	The Sensing Function performs a selection of the Sensing Entity(ies) based on the sensing service request.

-	The Sensing Function triggers each of the selected Sensing Entities to establish a direct connection, if the connection was not yet available.

-	A direct interface between the Sensing Function and Sensing Entity is established over the direct connection. The Sensing Function uses the direct interface to manage the sensing service operation at the Sensing Entity(ies), and obtaining the Sensing Data and Associated information from the Sensing Entity(ies).

NOTE: The protocols for the direct connection and direct interface can be determined based on coordination with RAN and CT WGs.

### 6.X.2	Procedures

#### 6.X.2.1 General Sensing Service operation procedure

Figure 6.x.2.1-1 illustrates the general operation sequence based on the solution principles described in 6.X.1.

Figure 6.x.2.1-1.	Sensing service operation procedure

0.	 A Sensing Service operation is initiated towards the Sensing Function. This may be triggered from a Sensing Service Consumer, or by another entity, e.g. a Sensing Service initiator that is different from the Sensing Service Consumer. The Sensing Service initiator can be also a network function, or a UE. The Sensing Service may be initiated via a Gateway Function, and corresponding service authorization operation (related to KI#2 as described clause 5.2) and Sensing Function discovery and selection (related to KI#3 in clause 5.3) may be carried out.

1.	The Sensing Function discovers and selects the appropriate Sensing Entities based the Sensing Service requested. This may make use of any compatible solution addressing the KI#3 as describe in clause 5.3.

2.	The Sensing Function sends a trigger for the Sensing Entity to set up the direct interface over a direct connection with it, if the interface was not yet established. This is achieved by providing for example the user plane address, transport protocols and required security information to the identified Sensing Entity. The signaling between the Sensing Function and the Sensing Entity can make use of any of the available connections between them, e.g. the control plane signaling or an already established user plane connection. This direct interface establishment may be triggered by the Sensing Entity in some prior signalling not shown in the figure.

3.	Based on the information received in step 2, the Sensing Entity either establishes a direct interface over the new connection or update an existing connection with the Sensing Function.

4.	The Sensing Function sends a Sensing Service request to the Sensing Entity. This needs to include the corresponding configurations for the Sensing Entities to carry out the sensing operations. It may also include the specific information on how the sensing data/results should be reported, e.g. the periodicity, or the criteria for an event report. Within the Sensing Service request, the Sensing Function may also indicate if the Sensing Entity needs to process the sensing data and generate sensing result or partial result in the report. Based on the situation, the step 4 may be signalled together with step 3, e.g. when a direct connection already exists.

5.	The Sensing Entity(ies) performs the sensing operations according to the configurations received in step 4. This may also involve necessary coordination between the involved Sensing Entities.

6.	Based on the sensing service request indicated in step 4, the Sensing Entity(ies) uses the direct connection to provide the sensing data/result to the Sensing Function.

7.	(optional) The Sensing Entity(ies) may use the direct connection to provide additional reports based on the configurations received in step 4, e.g. for a periodical sensing operations or event triggered sensing operations.

8.	The Sensing Function performs sensing result calculation or compiles the sensing results based on the report from the Sensing Entity(ies).

9.	The Sensing Function provides the sensing results to the Sensing Service Consumer based on the sensing service request received in step 0. This may make use of any solutions addressing the KI#5 as clause 5.5.

Based on the configuration from the Sensing Function, the Sensing Entity may provide the Sensing Data/Result, corresponding to step 6 and 7, to a location different from the Sensing Function, e.g. a Sensing Consumer, directly.

#### 6.X.2.2 Establishment of the direct connection and direct interface

With reference to the step 2 and 3 of the operations described in clause 6.X.2.1, there may be different operation options for the establishment of the direct connection and direct interfaces, depending on the type of the Sensing Entities and the network architectures.

For example, if the Sensing Entity is a UE, the step 2 goes through a core network to UE signalling interface, e.g. NAS. When the solution is applied to 5GS, the Sensing Function invokes the Namf_Communication_N1N2Transfer service and provide the signalling message towards the UE. The UE can use the provided information to establish a user plane connection with the Sensing Function, e.g. setting up a TLS connection. Similarly, URSP can be used for controlling the PDU Session(s) used for the interaction, and QoS Flows could be established to separate the treatment of the sensing operation traffic, e.g. control signalling and sensing data reports, and other communication service traffic.

The UE based Sensing Entity and the Sensing Function may establish a direct interface over this user plane connection for the sensing service operation, e.g. initiates and manage a sensing service operation, and reporting sensing data or results to the Sensing Function, etc.

If the Sensing Entity is a RAN node, the step 2 can go through a core network to RAN interface. When the solution is applied to 5GS, the signalling can be realized by the Sensing Function invoking the Namf_Communication_N1N2Transfer service. The Sensing Entity can initiate a direction connection establishment towards the address provided by the Sensing Function in step 2, e.g. establish a TLS connection to a provided IP address. A direct interface is established over the connection, without going through other core network control functions, e.g. AMF.

NOTE 1: The exact protocol for the direct connection can be either signalled within the step 2, or be defined by stage 3 based on the study conclusions. LCS-UPP as defined in TS 23.273 [x] is an example of how of user plane connection managed and utilized for non-data communication related services.

NOTE 2: 	Stage 3 can decide if the same protocols can be used for the direct connection and interface between the Sensing Function and UE based Sensing Entity and RAN based Sensing Entity.

#### 6.X.2.3 Sensing service operations over the direct interface

The direct sensing interface established in step 2 and 3 of clause 6.X.2.1 needs to support the sensing service operations as described in step 4 to 7.

If the Sensing Entity is a UE, a point-to-point direct interface between the Sensing Entity and the Sensing Function needs to be defined. The direct interface needs to allow the Sensing Entity to register to the system, to be discovered, and to be managed for the sensing operations. There can be other general interface functions utilized for the sensing service-related control or management, e.g. configurations or provisioning. The protocols used for this direct interface need to be defined based on RAN input or by RAN WGs directly.

NOTE 1:  LPP for location services defined in TS 37.355 [y] is an example of how such protocols defined by RAN WGs can be utilized by the system defined in SA2.

If the Sensing Entity is a RAN node, there may be different options to realize the direct interface for sensing service operations:

-	define a point-to-point protocol over the direct connection for sensing.

NOTE 2: The NRPPa defined in TS 38.455 [z] is an example of such end to end protocol, and it needs to be adapted to the direct connection between the Sensing Function and the Sensing Entity.

-	define a service based interface for the Sensing Entity, so that the Sensing Function can access over the direct connection;

-	allow the Sensing Entity to access the service based interface offered by the Sensing Function over the direct connection.

The examples of how the sensing service operation can be support for the two latter options are described in the following subclauses.

NOTE 3: Whether or which option of the direct interface is applicable can be decided based on RAN work scope alignment.

#### 6.X.2.4 Examples of the SBI based direct interface for sensing service

##### 6.x.2.4.1 General

To illustrate how the direct interface between the Sensing Function and the Sensing Entity can be utilized for sensing service, a few examples of the direct interface when the Sensing Entity is a RAN node were shown in the following subclauses.

Depends on the network architecture support and operator configurations, one of the options can be implemented.

##### 6.X.2.4.2 Sensing service operations based on the SBI of Sensing Entity

In this option, the Sensing Entity offers a set of services over an SBI interface, e.g. Nse_SensingService_Request/Response/Notify.

The operation example is illustrated in Figure 6.X.2.4.2-1.

6.X.2.4.2-1. Sensing operations using SBI of the Sensing Entity

1-2. The Sensing Entity registers itself to the NRF (or via the Sensing Function) after establishing the direct connection that allows the use of Nnrf_NFManagement_NFRegister service. In some options, such registration can be achieved via the Sensing Function, i.e. the Sensing entity registers to the Sensing Function, which in turn adds it to the NRF.

3.	Sensing Function receives a sensing service request. This corresponds to the step 0 of Figure 6.x.2.1-1.

4 -5. The Sensing Function discovers the corresponding Sensing Entity(ies) needed for a sensing service by querying the NRF, e.g. using Nnrf_NFDiscovery_Request service.

6. Sensing Function invokes the Sensing Entity service, e.g. Nse_SensingService_Request.

7.	Sensing Entity(ies) performs the sensing operations. This corresponds to step 5 of  Figure 6.x.2.1-1.

8. Sensing Entity provides sensing data/results via a response: e.g. Nse_SensingService_Response.

9. (optional) If an event report was triggered based on the criteria configured in step 6, the Sensing Entity reports the event to Sensing Function, e.g. Nse_SensingService_Notify.

##### 6.X.2.4.3 Sensing service operations based on the SBI of Sensing Function

In this option, the Sensing Entity access the services of the Sensing Function over by the SBI interface of the Sensing Function via the direct connection established in step 3 of Figure 6.x.2.1-1.

The operation example is illustrated in Figure 6.X.2.4.3-1.

Figure 6.X.2.4.3-1. Sensing operations using SBI of the Sensing Function

1.	The Sensing Entity is triggered by the Sensing Function to establish the direct connection as in step 2 of Figure 6.x.2.1-1. Alternatively, it might be redirected to another Sensing Function due to interactions with NRF.

2 -3. 	The Sensing Entity uses Nsnf_SensingEntity_Register service to make itself known to the Sensing Function.

4.	The Sensing Entity subscribe to the Sensing Function, according to the sensing services it would provide, e.g. Nsnf_SensingEntity_ServiceEvent_Subscribe

5.	The Sensing Function received a sensing service request, as step 0 and 1 of Figure 6.x.2.1-1.

6.	The Sensing Function to use Nsnf_SensingEntity_ServiceEvent_Notify to trigger and configure sensing entity for a sensing service.

7. 	Sensing Entity(ies) performs the sensing operations. This corresponds to step 5 of  Figure 6.x.2.1-1.

8-9. The Sensing Entity(ies) invokes the Nsnf_SensingEntity_ServiceReport to report sensing data/result to the Sensing Function.

### 6.X.3	Impacts on Services, Entities and Interfaces

A generic system architecture supporting Sensing Service needs to be defined.

A new direct interface between the Sensing Entity and Sensing Function needs to be defined, with the supporting signaling and operation procedures.

Editor's Note: The impacts to the existing system architecture and interfaces need to be reviewed after alignment of scope with RAN WGs.

# >>>>END OF CHANGES<<<<