3GPP TSG-SA WG2 Meeting #169	S2-2505645

Fukuoka, JP, 19th May 2025 – 23rd May 2025			         was S2-2505309

# Title:	Reply LS on Ethernet MA PDU session using MPQUIC-E steering

# Response to:	S2-2504573/C4-250554, LS on Ethernet MA PDU session using MPQUIC-E steering

# Release:	Rel-19

# Work Item:	MASSS

Source:	SA2

To:	CT4

Cc:	CT1

Contact person:	Pallab Gupta

pallab.gupta@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.502 CR5396, CR5421

1. Overall Description:

SA2 thanks CT4 for their LS on Ethernet MA PDU session using MPQUIC-E steering functionality, highlighting the potential gaps in the stage 2 specification about handling MA PDU Session with MPQUIC-E steering functionality when ROHC is required.

SA2 have discussed the questions raised by CT4 and would like to provide the below feedback.

Q1: Can SA2 clarify the handling of MA PDU sessions using MPQUIC-E with a 3GPP access leg in EPS with respect to the PDN connection type to be signaled to the eNB?

[SA2 Answer]: SA2 agrees with CT4 observation that there will be further impacts in the procedures for PDN Connection Establishment of a 3GPP access leg in EPS (when a non-3GPP access leg has been established in 5GC) and mobility procedures to handover the 3GPP access leg of a MA PDU session from 5GS to EPS. However, SA2 has discussed and decided to not impact the EPC. Accordingly, below way forward is agreed in the attached CR5421:

- MPQUIC-E steering functionality can be supported with no changes when EPC does not support Ethernet PDN.

- When EPC supports Ethernet PDN, MA PDU Session using the 3GPP access leg in EPC will not be allowed.

Q2: Can SA2 confirm CT4 assumption regarding the handling of MA PDU sessions using MPQUIC-E for HR PDU sessions and PDU sessions with an I-SMF.

[SA2 Answer]: SA2 agrees with CT4 observation that for HR PDU sessions the V-SMF transfers the ATSSS rules from the H-SMF transparently to the UE. SA2 would like to inform CT4 that for Rel-19 SA2 has decided to limit the feature to non roaming and LBO scenario only and accordingly agreed the attached CR5396.

SA2 would also like to point out that I-SMF is not supported for MA PDU Sessions (please see the specification text highlighted below). Therefore, no corresponding change is required to support PDU sessions with an I-SMF.

TS 23.501 clause 5.32.1 specifies below:

In this Release of the specification, support for ATSSS assumes SMF Service Areas covering the whole PLMN or that a MA PDU Session is released over both accesses when the UE moves out of the SMF Service Area.

TS 23.502, clause “4.23.5.1           PDU Session establishment procedure” also specifies as below:

“If the service area of the selected SMF does not include the current UE location and the UE does not request for a MA PDU Session, the AMF selects an I-SMF that serves the area where UE” camps.

“If the service area of the selected SMF does not include the current UE location and the UE requests a MA PDU Session, then the AMF rejects the MA PDU Session Establishment procedure.”

2. Actions:

To CT4.

ACTION: 	SA2 kindly asks CT4 and CT1 groups to take into account the above information and update stage 3 specification accordingly.

3. Date of Next SA2 Meetings:

SA2 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-S2.htm