3GPP TSG SA WG2 Meeting #169 	S2-2505701

# 19-23 May, 2025, Fukuoka , Japan  						 				     Revision of S2-2505458)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1551 | rev | 5 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on procedure and service of DC interworking via DC AS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, Hisilicon, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current procedure of DC interworking has some issues to be corrected: - In step 6a and step 7, the response to Nimsas_SessionEventControl_Notify service operation is sent twice. However there should be only one response and should be sent after all the media resources have been reserved. - Currently the DC AS is only notified of DC interworking and UE#2 information, but the DC AS has not been notified which DC connection is used for interworking. - Step 8 is incorrect, which should be Nimsas_SessionEventControl_Notify response instead of reINVITE request. To solve the issues, the followings are proposed: - Use Nimsas_MediaControl_MediaInstruction service operation in step 6a with the indication of DC interworking simultaneously with P2A media resource reservation, to trigger the notification from the IMS AS to DC AS; - Enhance the existing procedure of P2A DC establishment to indicate DC AS that the DC connection being established is for DC interworking. - UE#2 information is included by IMS AS in the notification to DC AS, not notified by DCSF. - Correct step 8 with Nimsas_SessionEventControl_Notify response. - Add interworking indication in Nimsas_MediaControl_MediaInstruction service operation. - Fix some Editorial issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Use Nimsas_MediaControl_MediaInstruction service operation in step 6a with the indication of DC interworking simultaneously with P2A media resource reservation, to trigger the notification from the IMS AS to DC AS; - Enhance the existing procedure of P2A DC establishment to indicate DC AS that the DC connection being established is for DC interworking. - UE#2 information is included by IMS AS in the notification to DC AS, not notified by DCSF. - Correct step 8 with Nimsas_SessionEventControl_Notify response. - Add interworking indication in Nimsas_MediaControl_MediaInstruction service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.9.3, AA.2.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change****

### AC.7.9.3	Signalling Procedure of Application Data Channel Interworking via DC AS

Figure AC.7.9.3-1 depicts a signalling flow diagram for establishing an application data channel from DCMTSI UE to DC application server, providing interworking of application data channel, supported by the DC AS, to a MTSI UE via the DC application server.

In this case, the originating DCMTSI UE sends the data channel application media to the DC AS, which performs the necessary interworking actions on the application media for the terminating MTSI UE.

Figure AC.7.9.3-1: Application Data Channel set up signalling procedure for IMS data channel interworking with MTSI UE via DC application server

The steps in the call flow are as follows:

1.	DC AS subscribes via the NEF for the event “Application Data Channel interworking required” to be notified when any application DC for a DC application that requires interworking is established.

1a.	The DC AS subscribes to the NEF by using Nnef_ImsEE_Subscribe Request (event ID: Application Data Channel interworking required, DC Interworking Information). The DC Interworking Information includes the application ID(s) of the DC application which requires interworking with MTSI UE.

1b.	After performing the necessary authorization, the NEF responds to DC AS via Nnef_ImsEE_Subscribe Response.

1c.	The NEF locates the IMS AS(s) via NRF.

1d.	The NEF subscribes to all the IMS ASs as requested by DC AS in step 1a, via Nimsas_ImsEE_Subscribe Request (Event Filter including the application ID(s)).

1e.	The IMS AS responds to NEF via Nimsas_ImsEE_Subscribe Reponse.

2.	IMS session and bootstrap data channel have been established. Selected DC application(s) have been downloaded to UE#1.

The DCSF knows the UE#2 or terminating network does not support IMS DC based on procedure in clause AC.7.9.1. Hence, the bootstrap data channel is established only between the UE#1 and the originating network.

3-5.	The UE#1 initiates a P2P application DC. Steps 1-2 of clause AC.7.2.1 apply.

6.	After receiving the session event notification from the IMS AS, the DCSF determines that IMS DC interworking for the requested DC application is to be performed based on the application information, i.e. application id. the remote network or UE#2 not supporting IMS DC, or not willing to use IMS DC. The DCSF changes P2P application DC to P2A application DC.

6a.	The DCSF instructs the IMS AS that the P2P application DC needs to be changed to P2A application DC. The DCSF also instructs the IMS AS that DC interworking is required to trigger the IMS ASnotification to the DC AS in step 6b.

The IMS AS modifies the endpoint type of the application data channel to the MDC2 endpoint received in the instruction.

NOTE: 	If the IMS AS cannot change SDP attributes different with the UE requested, the IMS AS can reject the P2P application data channel establishement request and the UE#1 may initiate P2A application data channel establishment. The detail are described in TS 24.229[10a].

6b.	The IMS AS notifies the NEF that the DC Interworking Required event is detected with the UE#2 information by using Nimsas_ImsEE_Notify Request. The UE#2 information may include the SIP URI or Tel URI of the UE#2.

6c.	The NEF responds to the IMS AS via Nimsas_ImsEE_Notify Response.

6d.	The NEF notifes the DC AS that the DC Interworking Required event is detected accompanied by the UE#2 side Information via Nnef_ImsEE_Notify Request.

6e.	The DC AS responds to the NEF via Nnef_ImsEE_Notify Response.

7.	The IMS AS instructs the MF to establish the media resource of the P2A application DC and the DC connection is established between UE and the DC AS. Steps 7-13 of clause AC.7.2.2 applyies with the enhancement that in step 9 of clause AC.7.2.2 the IMS AS indicates the DC AS in the connection establishment request that this connection is for DC interworking.

8-9.	The DCSF sends Nimsas_SessionEventControl_Notify response to IMS AS and IMS AS sends the re-INVITE to remote network side and UE#2, which does not include bootstrap data channel and application data channel media components in the SDP.

10.	Steps 17-26 of clause AC.7.2.2 apply with the differences that there is no any data channel media components being negotiated with terminating network and the SDP answer to originating UE is generated based on the P2A DC media. The SDP answer for application DC sent to the UE#1 indicates that P2A application data channel establishment is accepted.

11.	UE#1 uses the established P2A application data channel to provide DC application specific information e.g. sends the file (e.g. a photo) to the DC AS, which will be sent to UE#2 from DC AS.

12.	DC AS performs interworking actions based on the notification received in step 6d and the data received in step 11.

NOTE:	How the DC AS performs interworking actions to provide DC application specific information to the MTSI UE is out of scope of this specification. The DC AS can for example generate a hyperlink associated with the file received in step 11 and sends the hyperlink to UE#2 through a SMS, where the user of UE#2 can download the file using the hyperlink if he/she wants.

**** Second Change****

#### AA.2.4.3.2	Nimsas_MediaControl_MediaInstruction service operation

Service operation name: Nimsas_MediaControl_MediaInstruction

Description: This operation provides instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event, and that may require resource reservation in media resource e.g. MF.

For the case when a specific media flow needs to be terminated in MF media resource (i.e. termination of a Data Channel media descriptor offered by one of the IMS subscribers) or originated by the MF (i.e. origination of a data channel media descriptor to be offered towards one of the IMS subscribers), the consumer must provide a complete MF media specification including information required by MF to know how to terminate or originate the media flow.

Inputs, Required: Session ID, Media instruction set

Session ID specifies the IMS session for which the MediaInstruction operation applies.

Media instruction set includes a set of instructions for each media flow to control. Each instruction includes:

-	media ID: used by the producer (IMS AS) and the consumer for referencing purposes. The consumer reuses the media ID it received from the IMS AS for referencing the same media. This field will be null for instructions related to originating new media.

-	Media resource capability: Identify the Media Resource capabilities the Media instruction is intended for (e.g. DC, AR).

-	Media instruction: includes instructions to the producer (IMS AS) for handling the media. The following instructions are supported:

-	"TerminateMedia": Terminate the offered media descriptor of the UE in the mediaResource, i.e. this media descriptor will not be exposed to the other UE.

-	"OriginateMedia": Originate and offer a media descriptor from the mediaResource to the UE. The media ID representing the new media flow will be provided by the IMS AS in the response.

-	"TerminateAndOriginateMedia": Terminate the offered media flow in the mediaResource from the UE and originate the same media flow from the mediaResource to the other UE, i.e. the offered media descriptor of the originating UE will be replaced by the mediaResource provided media descriptor, which is sent towards the other UE.

-	"UpdateMedia": Update a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia"

-	"DeleteMedia": Delete a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia".

-	"RejectMedia": Remove an offered media flow, i.e. the offered media descriptor will be removed both from the offer sent to the remote UE and from the answer returned to the initiator of the offer.

-	DC Media Specification: Description of additional media specification information needed for data channel media stream from application layer, which includes:

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	MDC1/MDC2 media endpoint address of the application layer.

-	Replacement HTTP URL per stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	DC Interworking indication indicating the DC media is for DC interworking via DC AS when procedure in clause AC.7.9.3 applies.

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies how the media stream should be processed.

-	Avatar Media Specification: Description of additional media specification information needed for avatar communication services from application layer, which includes:

-	Resource URL: URL that allocated by DC AS for MF to retrieve the avatar representation to perform network rendering.

-	Media Processing Specification: It specifies how the media stream should be processed.

NOTE:	DC Media Specification and AR Media Specification are optional depending on the scenario and media type.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: operation result indication.

Outputs, Optional: media resource information set:

This set includes entries corresponding to each instruction in the received media instruction set. Each entry in the set includes the media ID (same as received or new for new media) and includes the media type according to SDP. Each entry may optionally contain Avatar Media Specification, DC Media Specification and/or AR Media Specification. The DC Media Specification shall exist only with media type "DC" and contains additional media resource information, e.g. allocated media address for MDC2 interface when an offer must be provided to Data Channel Application Server. The AR Media Specification contains media processing instruction for MF on how to process the media. The Avatar Media Specification contains media processing instruction for MF on how to perform network rendering for avatar communication.

**** End of Changes ****