3GPP TSG-SA WG2 Meeting #169	S2-2505832

Fukuoka, Japan, May 19–23, 2025                                                                   (revision of S2-2505606)

Source:	NEC

Title:	KI3 new solution: UPF discovery based on energy related information

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#3 for the Rel-20 FS_EnergySys_Ph2 study.

1. Introduction

This contribution proposes a new solution for KI#3 for the Rel-20 FS_EnergySys_Ph2 study.

-	Whether and how to perform NF (re)selection based on energy related information, including:

-	Whether and how to extend NF profile (e.g., include energy related information) to support NF (re)selection.

-	Which energy related information is to be considered for NF (re)selection.

-	Whether and how to enhance NF (re)selection procedure.

2. Proposal

It is proposed to agree the following key issue for the KI#3 in the TR23.700-67.

* * * Start of change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |
| --- | --- | --- | --- |
|  | 1 | 2 | 3 |
| X |  |  | X |

* * * End of change * * * *

* * * Start of change * * * *

## 6.X	Solution #X:  UP path adjustment based on energy related information

### 6.X.0	High-level solution principles

The high-level principles of the proposed solution are as follows:

- EIF is able to collect energy consumption data from OAM.

- SMF adjusts the user plane path by relocating the UPF based on energy related information provided by the EIF.

### 6.X.1	Description

This solution applies to Key Issue #3.

As noted in clause 4.2.18.1, TS23.501, the EIF requests energy related information of the targeted UPF(s) from OAM by creating management objects defined in TS 28.622 [x] using procedures defined in TS 28.532 [x]. It is expected to have the report as stream data, file-based or threshold crossing etc. Based on the service, this solution addresses the kye issue #3 for the UP path adjustments for the UE based on the energy attributes and energy related information from EIF.

### 6.X.2	Procedures

- With the steps in clause 4.17.6.2, TS23.502[x], UPFs may be associated with UPF Provisioning Information in the NRF. The UPF Provisioning Information additionally consists of new attribute:

-	an indication that the energy related information for the UPF can be collected by OAM

- As noted in clause 4.2.18.1, TS23.501, the EIF requests energy related information of the targeted UPF(s) from OAM by creating management objects defined in TS 28.622 [x] using procedures defined in TS 28.532 [x]. It is expected to have the report as stream data, file-based or threshold crossing etc.

- In the PDU establishment, SMF retrieves the subscription data on UDM in clause 4.3.2, TS23.502 [3]. The subscription may include information that indicates if the subscriber allows the operator to use energy efficient resource or not. If allowed, the network operator is allowed to use energy efficient network resources. The information may, for example, indicate that the subscriber allows the network operator to use energy efficient network resource when the network is in a condition of off-peak (e.g. workload become less such as in night-time.).

- The SMF invokes Neif_EventExposure_Subscribe to be notified with energy related information from the discovered UPF(s).

- The EIF computes energy related information based on the reported energy consumption data.

- The EIF invokes Neif_EventExposure_Notify to report the outcome of the computation. As described in clause 5.51.2.2.3, TS23.501[2], it is expected to report UPF energy consumption and UPF data volume. The reported UPF energy consumption and UPF data volume may optionally be stored by SMF.

- In order to optimize the energy saving and efficiency whilst maintaining a high service quality along with the time-of-the day, SMF determines to initiate UPF relocation.

- Based on the reported or stored UPF energy consumption and UPF data volume, SMF selects targeted UPF(s) for UPF relocation.

- The same procedures in clause 4.3.5.1, TS23.502[3], can be reused to relocation UPF.

Figure 6.x.2-1: UP path adjustments for the UE based on energy related information

### 6.X.3	Impacts on services, entities and interfaces

EIF:

- Notify energy related information to SMF

NRF:

- Support an indication that the energy related information for the UPF can be collected

SMF:

- Determine when UPF relocation should be initiated

- Selects the UPF based on energy related information

UDM:

- Support information that indicates if the subscriber allows the operator to use energy efficient resource or not

* * * End of change * * * *