3GPP SA WG2 Meeting #170	S2-2506461

26-29 Aug 2025, Goteborg, SE

Title:	[Draft] LS on maximum DRBs for support of IMS voice over NB-IoT NTN connected to EPC

Response to:	-

Release:	Release 20

Work Item:	FS_5GSAT_Ph4_ARC

Source:	SA2

To:	RAN2

Cc:	RAN1

Contact Person:

Name:	Heng Nie

Tel. Number:

E-mail Address:	nieheng@chinatelecom.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 is studying the support for IMS voice over NB-IoT NTN via GEO, in the study item FS_5GSAT_Ph4_ARC. Given the GEO IMS voice can apply to mobile phone, it is a resonable demand that a UE using IMS voice over NB-IoT NTN should also simultaneously use other services, e.g. Internet service, VPDN, XCAP at the Ut reference point for Supplementary Services, which may use user plane for transmission efficiency.

As per TS 36.331 clause 5.3.1.4, depending on its capability a NB-IoT UE supports maximum 2 DRBs. In TR 23.700-19-v020 which documents SA2 study, there are several solutions proposing to use DRB(s) to support IMS voice. e.g. one DRB for IMS signalling and one DRB for voice data, one SRB for IMS signalling and one DRB for voice data. SA2 also discussed the solution described in S2-2504662 which is not endorsed or agreed. In this solution, when an UP bearer is needed to be activated for voice data and the existing number of activated UP bearers has reached 2, it proposes to temporarily release an existing UP bearer for other service and then activate UP bearer for voice data. At the end of the call the UP bearer voice is released then the previously released UP bearer for other service is activated again. This approach can ensure the priority of voice service based on the existing DRBs restriction for a UE, but other service may be suspended during the call.

If SA2 makes the conclusion to use DRB to support IMS voice, based on the existing limitation of maximum 2 DRBs SA2 thinkes it is not possible to simultaneously support IMS voice and other services, at least during the call. If RAN can support multiple DRBs in R20, then the UE can support simultaneous voice and other services even during the call.

Question: Is it feasible to support more than 2 DRBs for a UE accesing NB-IoT in R20? If the answer is yes, what is maximum number of DRBs supported?

2. Actions:

To RAN2:

ACTION: 	SA2 kindly requests RAN2 to answer the Question.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #171		13-17 October 2025	Wuhan, China

TSG-SA2 Meeting #172		17-21 November2025	Dallas, USA