SA WG2 Meeting #170	S2-2506575

25 – 29 Aug, 2025, Goteborg, SE

Source:	CATT

Title:	[WT#5] Data Framework for 6G System

Document for:	Agreement

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution is intended to justify and describe the study scope and the potential Key Issues of data framework under WT#5 as defined in S2-2506096.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

S2-2506096 indicates WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

Regarding the WT#5, this paper suggests to consider the following inputs.

Architectural Motivation: The core objective of the mobile communication system defined by 3GPP (e.g., 5G System, 5GS) is to enable User Equipment (UE) to access the Data Network (DN) and to be able to use the data services provided by the network. To this end, 5GS defines Control Plane (CP) to support user access and control the signalling interactions within the system, and User Plane (UP) to support data transmission and QoS guarantee for user traffic data.

For 6G, SA1 defines the 6G System Data as follows. 6G System Data, such as, Sensing data, AI data, NF context, is different from user traffic data. This type of data not only requires the 3GPP system to deliver application level data transparently between the UE and the DN, but presents some new requirements.

TR22.870 indicates 6G System Data: the data that is generated, and controlled by 6G system.

NOTE 1: 6G system data is different from traditional user traffic data which is application level data being transmitted through the 3GPP system for user related services.

Examples of use cases for different data are given below:

- AI data：

  - AI data: data related to AI models/algorithms, which can be used to improve network performance and/or provide AI services (e.g. AI inference) for applications.

  - AI data including: e.g. raw data or processed datasets for training/testing/validating AI models, AI models and parameter data, AI inference input data and output data (e.g. AI analytic results).

  - Use cases: support AI model training data collection from UE, RAN, CN NFs (e.g. UPF, ADRF, other NFs) to Core Network for further processing to generate AI data set, AI model training, AI inference to generate analytic results, and/or storing for future use [refers to TR22.870 clause 5.9.2].

- Sensing data：

- Sensing data: data related to object or environment of interest for sensing purposes, which can be derived from 3GPP radio signals and also can be provided by non-3GPP devices.

- Sensing data including: e.g. sensing measurement data generated by sensing entities with sensing capabilities (e.g. UE, BS, and Non-3GPP devices), and sensing results generated after processing sensing data within the 6G system.

- Use cases: support sensing measurement data collection from UE, RAN to Core Network for further processing to generate sensing results and/or storing for future use [refers to TR22.870 clause 5.9.2, and clause 5.4.6].

- NF context：

  - NF context: the “context” storage resource of the NF Service, which is fully or partially decoupled from the “compute” resource of the NF Service, e.g. UE context in 5G AMF, SM context in 5G SMF.

  - Use cases: support data sharing between different manufacturers or different places of the same manufacturer to avoid data silos; support trusted exposure of the context data, such as expose network status data to trusted third parties or UEs.

For the above different use cases, a common data framework needs to be considered at the beginning of 6G architecture design.

Gap Analysis: consider the following two aspects.

- Case-by-case data handling in 5GS. Beyond traditional connectivity services, some new beyond connectivity services emerging, such as sensing services and AI services, brings a large amount data (called 6G System Data)  and some new data handling requirements.

  - 6G System Data requires that the 6G system supports flexible and efficient data collection and processing for the data from different data sources (e.g., UEs, RANs, CN NFs) based on data consumer requirements. However, traditional CP, designed for signalling delivery, which is not suitable for large amount of data transmission. UP as “pipeline” supports the data transmission between UE-UPFs for user plane traffic, and cannot well support the new transmission topology requirements (e.g., direct data transmission between RAN-CN) and on-demand data processing requirements.

  - Current 5G data handling methods are varied and across different use cases (e.g., NWDAF-based for AI data handling, LPP-based UE positioning data handling, BS Sensing data handling is under discussion in R20), which introduces complexity to the architecture design. At the same time, the case-by-case solutions have poor scalability support for future new services.

- Data storage architecture in 5G.

  - For supporting "stateless" NFs, 5G support the "compute" resource is decoupled from the "storage" resource of the NF Service. However, the compute/storage separation of 5G is not thorough enough, for example, the context data in 5G AMF/SMF is still unstructured, and a large amount of data is still dispersed in different 5GC NFs, which leads to insufficient data sharing among different manufacturers and inefficient data collection methods.

  - 5G utilizes UDRs and UDSFs to support the storage of network data, such as in UDM/PCFs and AMF/SMFs, and ADRFs to support the storage of the collected and analytic data in the core network. For 6G, more data such as AI and Sensing will be generated and controlled by system, the 5G data storage architecture needs to be redesigned to facilitate data monetization via unified data management and control.

Therefore, to support multiple use cases and avoid diverse solutions while enabling data collaboration between different data types, it is recommended to design a common data framework for 6GS. This framework needs to be easily scalable to support future new services. The data framework should enable operators to manage and control 6G System Data, and further support including: e.g. improving data collection efficiency and avoiding duplication data collection, efficient and flexible data transmission between UE, RAN, and CN NFs (e.g. transmission between UE-CN, RAN-CN, UE-RAN, different CN NFs, as well as supporting data providers and data consumers join or leave flexibly at any time), on-demand data processing of different service logics, efficient data storage for future use, data exposure to enable data monetization.

Technical Impact: How would this WT/KI influence:

- Overall architecture, including dedicated NF(s) and interfaces could be required.

- RAN-CN interaction and UE-network interaction may be impacted.

- Interactions between Network Functions.

**** First Change ****

Annex A.X. WT#5 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#5 Scope remains unchanged.

**** Second Change ****

5.X. Key Issue #X: Data Framework

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This key issue aims to study the data framework for 6G System Data with the considerations of different use cases (e.g. AI, Sensing services, and future other services) and the data-related service requirements from SA1.

The following aspects will be studied:

- Whether and how the 6G system architecture to support the data framework, including e.g. defining new NF(s) and interfaces.

- Whether, how and which data to be collected and coordinated, e.g., unified control to support reuse and avoid duplicated data collection.

- Whether and how to support efficient and flexible data transmission involving UE, RAN, CN NFs, e.g. Data Session between UE<->CN, RAN<->CN, UE<->RAN, different CN NFs, and how to support data providers and data consumers join or leave flexibly.

- Whether, how and which data processing functionalities are supported, e.g. data pre-processing for AI service, data aggregation for Sensing service, collaboration of multiple data types.

- Whether, how and which data is stored and retrieved efficiently, e.g. unified data storage architecture.

- Whether, how and which data needs to be exposed to the authorized third-parties and the UEs, with consideration of access control/user consent and privacy.

- Whether and how to support data framework for roaming case.

NOTE 1: Access control/user consent and privacy aspects will be coordinated and aligned with SA3.

NOTE 2: Charging and OAM aspects will be coordinated with SA5.

NOTE 3: Radio interface, RAN-CN interaction aspects will be coordinated and aligned with RAN WGs.

**** End of Changes ****