3GPP TSG-WG Meeting #170	S2-2507193

Goteborg, SE, 25-29 Aug, 2025

Source:			CATT

Title:			WT#1.2, 6G Policy Framework – Study proposal for 6G Policy Framework

Document for:			Approval

Agenda item:			20.6.1.2

Work Item / Release:			FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the study of 6G Policy framework under WT1.2.

1．Introduction

The Policy Control is a very important function for the whole 4G and 5G systems. In the 4G/EPS, the policy is mainly on the QoS and Charging on the Session Management, but in the 5GS, the policy function is expanded to include Access and Mobility Policy (in AMF), UE PDU Session Selection Policy (URSP),  UE Access Selection Policy (ANDSP) and service related policy UE V2X Communication Policy(V2XP), ProSeP, A2XP, RSLPP.

2. Gap Analysis

For the new 6G connectivity services (e.g., avatar-based multimodality immersive communication services), whether new policy requirements and new policy rules are to be defined in 6GS. For the avatar-based immersive communication services, the voice, avatar model data, user gesture data, and user facial expression data are critical multi-modality data to support the service, and these multi-modality data have different traffic characteristics, and how to support these multi-modality data coordinately is not well defined in 5GS. The Jitter is also a very important QoS characteristic in the policy, but it is not defined in the 5G.

In 6GS, it is assumed that the Policy Control function will continue to evolve to include QoS and Charging for the SM and AM Policy for MM and UE related Policies. After the beyond-communication services are defined in 6G, it is assumed that a new data plane framework will be defined in 6GS, in such a case, the policy on the managements of the data framework and beyond-communication services in the UE and 6G NFs needs to be studied.

The computing function is to be studied in 6GS,  in such a case, whether and how the policy control can be used on the computing framework in the UE and 6G NFs also need to be studied.

The current 5G PCF produces the rules to UE and 5G NF based on the pre-defined rules in the PCF or the UDR. However, such pre-defined rules are not accurate and cannot match well with the dynamically changing services.  The 6G policy rules need to support producing dynamic and more accurate policy rules based on received dynamic requests and or the received dynamic event reports via the AI in the 6GS.

3．Proposed Changes

* * * First Change * * * *

Annex A.X. WT#(1.2): Scope: Support of 6G Policy Framework

Study how 6G supports the policy framework, including

-	How to support policy in the UE and 6G NF(s) for the 6G connectivity services, e.g., avatar-based multimodality immersive communication service.

-	Whether and how to support the policy in the UE and 6G NF(s) for the 6G data framework and beyond-connectivity services;

-	Whether and how to support the policy in the UE and 6G NF(s) for the computing functions in 6GS.

-	Study whether and how to enable AI functionalities in 6G to support and improve the policy for 6G connectivity services and 6G beyond-connectivity services, and computing in 6G.

NOTE 1: The policy on data framework, computing will be coordinated and aligned with WT#5/6.

* * * Next Change * * * *

5.X. Key Issue #X (1.2.X): Support of 6G Policy Framework

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This key issue (KI) will study how 6G supports policy control for the 6G connectivity services and 6G beyond-connectivity services and computing functions.

-	How to support policy in the UE and 6G NF(s) for the 6G connectivity services, e.g., avatar-based multimodality immersive communication service.

-	Whether and how to support the policy in the UE and 6G NF(s) for the 6G data frame and beyond-connectivity services;

-	Whether and how to support the policy in the UE and 6G NF(s) for the computing functions in the 6GS.

-	Study whether and how to enable AI functionalities in the 6G to support and improve the policy for 6G connectivity services and 6G beyond-connectivity services, and computing in 6G.

NOTE 1: The policy on data framework, computing will be coordinated and aligned with WT#5/6.

* * * End of Changes * * * *