3GPP TSG SA WG2#170	S2-2507196

Goteborg, Sweden, August 25 – 29, 2025	(was S2-241xxxx)

Source:	Lenovo

Title:	Proposed Key Issue for WT3: Study a Distributed Intelligence Framework in 6G

Document for:	Approval

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Release 20

Abstract of the contribution:

# 1	Discussion

The current wording for WT3 is as follows:

Study how to support and enable use of AI in 6G (e.g. AI agent, framework).

NOTE 4: The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

The 5G System introduced the NWDAF framework, which enables data collection and providing historical / predictive analytics to other network functions. The architecture supports distributed deployments (e.g., central, regional, edge) and collaborative model training (using FL), which are critical elements for network optimization.

While the NWDAF provides the necessary analytics, the 6G system requires a deeper integration of intelligence to achieve full, real-time autonomy. The goal is to evolve from a network that is analytics-driven to one that is intent-driven and cognitive. This requires a framework where intelligence is not just a service to be consumed, but a native capability with the power to act autonomously.

The NWDAF framework successfully distributes the analytics capability. However, the decision to act on these analytics still resides with the consuming network functions (NFs) or the OAM system. This separation between "analysis" and "action" can introduce latency and complexity, limiting the network's ability to react in real-time to dynamic conditions.

To meet the 6G goals of a lean, efficient, and fully autonomous network, the architecture must support "autonomous agency". This means embedding intelligent entities (e.g., AI Agents) directly into the core network architecture that can perceive the network environment, reason, and execute actions to achieve a high-level goal (an "intent").

The required evolution from 5G to 6G intelligence can be summarized as shown in the table below.

Table 1: Evolution from 5G to 6G intelligence

| Aspect | NWDAF in 5G/5GA | Distributed Intelligence Framework in 6G |
| --- | --- | --- |
| Primary role | Provides Analytics-as-a-Service. Other NFs consume analytics and then decide how to act. | Provides Intelligence-as-a-Native-Function. AI Agents are architectural components with the autonomy to act directly. |
| Operational model | Analytics-driven. The action is separated from the analysis. Intelligence is an "add-on" capability. | Intent-driven. An AI Agent receives a high-level goal and autonomously orchestrates NFs to achieve it in a fast, closed loop. |
| Architectural Focus | Focuses on distributing the data analytics capability. | Focuses on distributing the autonomous agency capability. |

Frameworks have been defined in order to facilitate interaction of agents with external systems/data sources through the use of context oriented communication such as the Model Context Protocol as well as facilitate interaction between agents, or inter-agent communication, through the use of A2A, ACP and other protocols. One aspect to study is how the 3GPP system is involved for facilitating communication between AI agents and/or between AI agents and the 3GPP system.

In addition, to support a Distributed Intelligent Framework in 6G a framework is required to be studied to support AI/ML operations such as Model Training, Data Collection (which will be covered as part of WT5) and Model Provisioning whilst taking into account the procedures defined for the NWDAF framework. Such procedures are required to support AI agent functionality, e.g., how AI agents are trained to make decisions in dynamic environments. ML model training techniques such as online learning and reinforcement learning can be leveraged to support such AI agent functionality.

This contribution proposes a Key Issue to study the evolution from the 5G analytics framework to a 6G Distributed Intelligence Framework (DIF), which enables autonomous, intent-driven operations.

# 2	Proposal

The following are proposed to be included in 6G study TR

**** First Change ****

# Annex A.X. WT#3 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

The following are within WT#3 scope:

-	Study support of a fully autonomous network supporting an "autonomous agency". This means embedding intelligent entities (e.g., AI Agents) directly into the core network architecture that can perceive the network environment, reason, and execute actions to achieve a high-level goal (an "intent").

**** Second Change ****

5.Z. Key Issue #Z: Key Issue for WT3: Study support of a Distributed Intelligent Framework in 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

Introduction

The 5G System introduced the NWDAF framework, which enables data collection and providing historical / predictive analytics to other network functions. The architecture supports distributed deployments (e.g., central, regional, edge) and collaborative model training (using FL), which are critical elements for network optimization.

While the NWDAF provides the necessary analytics, the 6G system requires a deeper integration of intelligence to achieve full, real-time autonomy. The goal is to evolve from a network that is analytics-driven to one that is intent-driven and cognitive. This requires a framework where intelligence is not just a service to be consumed, but a native capability with the power to act autonomously.

As part of this key issue it is proposed to study the evolution from the 5G analytics framework to a 6G Distributed Intelligence Framework (DIF), which enables autonomous, intent-driven operations.

Motivation

The NWDAF framework successfully distributes the analytics capability. However, the decision to act on these analytics still resides with the consuming network functions (NFs) or the OAM system. This separation between "analysis" and "action" can introduce latency and complexity, limiting the network's ability to react in real-time to dynamic conditions.

To meet the 6G goals of a lean, efficient, and fully autonomous network, the architecture must support "autonomous agency". This means embedding intelligent entities (e.g., AI Agents) directly into the core network architecture that can perceive the network environment, reason, and execute actions to achieve a high-level goal (an "intent").

The required evolution from 5G to 6G intelligence can be summarized as shown in the table below.

| Aspect | NWDAF in 5G/5GA | Distributed Intelligence Framework in 6G |
| --- | --- | --- |
| Primary role | Provides Analytics-as-a-Service. Other NFs consume analytics and then decide how to act. | Provides Intelligence-as-a-Native-Function. AI Agents are architectural components with the autonomy to act directly. |
| Operational model | Analytics-driven. The action is separated from the analysis. Intelligence is an "add-on" capability. | Intent-driven. An AI Agent receives a high-level goal and autonomously coordinates NFs to achieve it in a fast, closed loop. |
| Architectural Focus | Focuses on distributing the data analytics capability. | Focuses on distributing the autonomous agency capability. |

As part of this key issue for WT3 the following aspects should be studied:

-	Study how an AI Agent should be defined in the 6G architecture (e.g., as a new, standalone NF, or as an embedded capability within other NFs).

-	Study the functions and responsibilities of an AI Agent.

-	Study the necessary interfaces for an AI Agent to interfact with other NFs for the purpose of coordinating NFs to achieve a desired goal.

-	Study how an AI Agent receives intents (e.g., from authorized third parties).

-	Study support of AI/ML operations for AI agents to achieve a requested intent

-	Study whether and how an ML model used by an AI agent is trained to support a requested intent. As part of this objective study how reinforcement learning and online learning are supported within the 6G core network

-	Study whether and how a trained ML model to support a specific intent is provisioned to an AI agent

NOTE x:	Data collection aspects are covered by WT5

NOTE y:	The AI/ML operations defined for the NWDAF framework will be used as a basis.

-	Study the impact on 6G NFs, e.g., how the SMF or PCF would need to evolve to support interaction with an AI Agent.

**** End of Changes ****