SA WG2 Meeting #170	S2-2507320

# August 25 - 29, 2025, Goteburg, Sweden

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: Fast call set up from the idle mode using suspend-resume for CP/UP bearer in 1 PDN connection

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#1.

1	Introduction

This contribution proposes a solution for KI#1 on IMS voice service over GEO satellite, in particular addressing call setup time reduction from the idle mode.

This solution assumes to use a CP bearer for SIP messages and a UP bearer for voice media in 1 PDN connection.

With this solution, call setup time from the idle mode becomes shorter, since AS security procedure and RRC reconfiguration do not need to be performed.

2	Proposal

It is proposed to add the following text to TR 23.700-19 v020.

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #1 | X | X |  |  |
| #2 | X |  |  |  |
| #3 | X |  |  |  |
| #4 | X | X | X | X |
| #5 | X |  |  |  |
| #6 | X |  |  |  |
| #7 | X | X |  |  |
| #8 | X |  |  |  |
| #9 | X |  |  |  |
| #10 | X |  |  |  |
| #11 | X |  |  |  |
| #12 |  | X |  |  |
| #X | X |  |  |  |

********** Next Change (All new text) **********

6.x	Solution #x: Fast call set up from the idle mode using suspend-resume for CP/UP bearer in 1 PDN connection

6.x.1	High level principles

This solution addresses KI#1, in particular call setup time reduction from the idle mode for the scenario where a CP bearer and a UP bearer in 1 PDN connection are used.

In this solution, the suspend/resume procedures are utilized; the UE AS context in eNB is retained when UE goes idle. In this solution, when the CP bearer needs to be resumed, the UP bearer is also resumed. Then, RRCConnectionResumeComplete message may contain SIP INVITE.

6.x.2	Description

The assumptions of this solution are as follows:

-	It is assumed that a CP bearer and a UP bearer in 1 PDN connection are used. The CP bearer transfers SIP messages and the UP bearer transfers voice media.

-	It is assumed that the CP bearer and the UP bearer are already established before SIP INVITE is sent (by using one of other solutions).

The principles of this solution are as follows:

-	The suspend/resume procedures are utilized with the following addition to allow S-GW activating and deactivating two bearers simultaneously.

-	During the suspend procedure, upon reception of Release Access Bearers Request, S-GW releases both S1 eNodeB F-TEID and S11-U MME F-TEID.

-	During the resume procedure, MME sends to S-GW the Modify Bearer Request containing both S1 eNodeB F-TEID and S11-U MME F-TEID.

NOTE:	This behavior of MME and S-GW might not be precluded in the existing specifications.

-	During the resume procedure, UE may include SIP INVITE in RRCConnectionResumeComplete message.

The advantages of this solution are as follows:

-	The AS security mode command procedure and the RRC connection reconfiguration procedure are skipped, which leads to the shorter call set up time from the idle mode.

-	SIP INVITE in RRCConnectionResumeComplete may contribute to even shorter call set up time from the idle mode.

6.x.3	Procedures

Figure 6.x.3-1: Suspend

0.	eNB recognizes to use the suspend/resume procedure either by an indication in Initial Context Setup request that the eNB has received from MME or by local pre-configuration.

1-2. eNB and MME proceed with the suspend procedure as per the existing specifications.

3.	S-GW releases both S1 eNodeB F-TEID and S11-U MME F-TEID. S-GW sends a Release Access Bearers Response to MME.

4-5. MME and eNB proceed with the suspend procedure as per the existing specifications.

Figure 6.x.3-2: Resume

1.	UE and eNB perform the RRC connection resume procedure as per the existing specifications. RRCConnectionResumeComplete may contain a NAS message containing SIP INVITE.

2.	eNB sends UE Context Resume request to MME as per the existing specifications.

3.	MME sends to S-GW a Modify Bearers Request containing a Bearer Contexts to be modified that contains S11-U MME F-TEID and another Bearer Contexts to be modified that contains S1 eNodeB F-TEID.

4.	S-GW modifies the bearer contexts both for the CP bearer and the UP bearer. S-GW sends a Modify Bearers Response to MME.

5.	MME sends UE Context Resume response to eNB as per the existing specifications.

6.	When RRCConnectionResumeComplete that eNB has received at step 1 contains a NAS message containing SIP INVITE, eNB sends to MME Uplink NAS Transport containing the NAS message.

6.x.4	Impacts on Services, Entities and Interfaces

MME:

-	Capability of setting both S1 eNodeB F-TEID and S11-U MME F-TEID in Modify Bearer Request, if the existing specifications do not allow it.

S-GW:

-	Capability of releasing both S1 eNodeB F-TEID and S11-U MME F-TEID upon reception of Release Access Bearers Request, if the existing specifications do not allow it.

UE:

-	Optionally, adding SIP INVITE in a NAS message in RRCConnectionResumeComplete.

********** End of Changes **********