3GPP TSG-WG SA2 Meeting #170	S2-2507439

Goteborg, SE, 25th Aug – 29th Aug, 2025

Source:	HONOR

Title:	Interim agreements for UE reader authorization (WT#1)

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: This contribution proposed the agreed principles for UE reader authorization.

1. Introduction

This contribution is proposed to address the editor’s notes related to the UE reader authorization in TR 23.700-13 and proposed the agreed principles for WT#1 the support of AIoT services in Topology 2 in SP-250834.

In Rel-19, the TR 23.700-13 has concluded some interim conclusions related to the UE authorization aspect:

8.1.3.1	General

-	Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

-	The UE Reader is assumed to request radio resources for Reader operation (e.g. taking into account assistance information received from the AIoTF) from the gNB.

NOTE 1:	Assistance information is further clarified in other clauses.

NOTE 2:	The details of UE Reader radio resource management will be defined by RAN WG2.

8.1.3.4	Interim Conclusion for authorization

The following interim conclusion are agreed for the UE Reader authorization in Topology 2:

-	The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader.

-	The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader.

And also, the following editor’s notes related to UE Reader authorization are remained in TR 23.700-13:

Editor’s note:	Whether and how to enable authorization to the UE is FFS.

Editor's note:	How to authorize the UE Reader will be determined later.

Based on the interim conclusions for UE reader radio resource management, the UE cannot perform any AIoT service as an AIoT reader before it is authorized by the network. According to the related solutions in the TR 23.700-13, most of solutions support that the AMF enables the AIoT reader authorization to the UE during the UE registration procedure. Therefore, we proposed the following proposal:

Proposal: During the UE registration procedure, the UE provides its AIoT capability to the AMF, which then determines whether the UE is authorized to work as an intermediate node for the AIoT service based on UE's AIoT capability and the UE subscription.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-30.

# * * * * First change * * * *

7	Interim agreements

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#Y

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following interim agreed principles are related to the UE Reader authorization in Topology 2:

-	The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader.

-	The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader.

-	During the UE registration procedure, the UE provides its AIoT capability to the AMF, which then determines whether the UE is authorized to work as an AIoT reader based on the UE's AIoT capability and the UE subscription.

# * * * * End of changes * * * *