3GPP TSG SA WG2 Meeting #170			S2-2507964

Goteborg, Sweden, 25th – 29th August, 2025	(revision of S2-2507022)

# Source:	Samsung, Verizon, Oracle, Tejas Networks Limited, CEWiT

# Title:	New WID on providing per-subscriber Allowed MAC addresses from UDM

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	WID on providing per-subscriber Allowed MAC addresses from UDM

Acronym:	TEI20_MACSUB

Unique identifier:		TBD

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

The support for allowed MAC addresses handling in 5GS for Ethernet PDU Sessions is currently rather limited. As specified in TS 23.501, clause 5.6.6 and 5.6.10.2:

- 	There is support for a DN-AAA Server to provide a list of allowed MAC addresses to SMF (for a maximum of 16 MAC addresses). This allows the UPF to allow packets to/from a PDU Session only if it has one of the allowed MAC addresses. Ethernet frames with other MAC addresses will be dropped.

As the DN-AAA server is invoked only when secondary authentication and authorization is enabled for the corresponding DNN using which UE has triggered PDU session or based on SMF policies which may require DN authorization without Secondary authentication/authorization, support for SMF/UPF to allow/block MAC addresses with different values per subscriber is thus not available.

An appropriate place for per-subscriber MAC address handling information is in SM subscription data in UDM as was done for TEI19_VLANSUB for per-subscriber VLAN handling and the impact will be limited to SMF and UDM.

# 4	Objective

The following aspects will be specified:

WT 1:	Add support for allowed MAC addresses in SM subscription data in UDM/UDR.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT 1 |  | 0.5 | No | None |

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Support of allowed MAC addresses in SM subscription data in UDM | TSG#XX |  |
| TS 23.502 | Support of allowed MAC addresses in SM subscription data in UDM | TSG#XX |  |

# 6	Work item Rapporteur(s)

Ashok Kumar Nayak, Samsung (ashok.nayak @ samsung.com)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

Samsung

Verizon

Oracle

Tejas Networks Limited

CEWiT