SA WG2 Meeting #170	S2-2507355

25 – 29 Aug, 2025, Goteborg , SE

Source:	Xiaomi

Title:	[WT#5] KI on data framework in 6G

Document for:	Agreement

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: The clarification on WT#5 on data framework in 6G.

# 1. Justifications

The contribution provides clarification on WT#5 as agreed in in the SID of FS_6G_ARC(SP-250806).

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination.

1.1	Architectural Motivation

In TR 22.870 clause 5.9.2, the requirements of efficient data collection and consumption for 6G system are proposed, and there are the Potential New Requirements as following:

[PR 5.9.2.2-1] The 6G system shall support mechanisms for 6G System Data collection and consumption minimizing the impact to 6G services.

[PR 5.9.2.2-2] Subject to operator’s policy, regulation and user consent, the 6G system shall support processing of 6G System Data.

NOTE: examples of data processing are use case dependant, e.g. data fusion, data anonymization and data analysis.

[PR 5.9.2.2-3] Subject to user consent, regulation and operator's policy, the 6G system shall support secure means to expose 6G System Data to authorized trusted third-party, authorized network function or authorized UE.

[PR 5.9.2.2-4] The 6G system shall support security and privacy protection of the 6G System Data.

[PR 5.9.2.2-5] The 6G system shall be able to provide charging and accounting mechanisms for 6G System Data.

1.2	Gap Analysis-Data handling within 5G architecture

1) 5G SA2 Data handling for beyond connectivity services provided by operator

To support sensing, AI/ML, Ranging/Sidelink Positioning and eLCS/Positioning, multiple NFs have to be introduced into the network, which then use different procedures and mechanisms to support the related data handling within the network architecture requiring similar functionalities on different NFs as below.

2) 5G SA5 data management

In 5G, SA5 has designed a data management system for Management data. The Management data is classified into 5G performance measurements as defined by TS 28.552, TS 28.554 and TS 32.423. The Management data is within the remit of SA5 belongs to management domain for network operation, with different characteristics comparing with the service data managed along with the network services, at least on the following aspects: purpose, consumer, granularity & time scale, typical data types, data privacy and lifecycle management.

Observation#1: Considering above, to support efficient data collection and consumption for 6G system, common data framework for multiple services/features should be studied in SA2 6G.

1.3	Technical Impact

As the common data framework is for multiple services/features by avoiding overlapping network functionalities 6G system which can be applied by different services, so this WT/KI influence:

- Overall system architecture

- All other WTs involving data handling

- Interworking or migration from 5G to 6G

Proposal

It is proposed to add the following to the TR 23.801-01 “Study on Architecture for 6G System”

**** First Change ****

Annex A.X. WT5 Scope

WT#5 focuses on common data framework under remit of SA2 e.g. for service data handling for operator service.

The WT Studies data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

**** Second Change ****

5.X. Key Issue #5: common data framework

To support efficient data collection and consumption for 6G system, common data framework for multiple services/features should be studied. The following issues need to be address during the study on the common data framework in 6G:

-	what data scope to be involved, e.g. ISAC data, AI data, positioning data;

NOTE 1:	This WT discusses generic mechanism for data management, data management for services covered by other WTs should be collaborated with related WTs before conclusion.

-	whether and how to define an end-to-end architecture for data framework, e.g. which functional nodes/entities of data source and data handling functionalities involved and the interfaces among the functional nodes/entities;

-	whether and how to support data collection, data processing and data storage;

-	whether and how to support data access and data exposure;

NOTE 2:	Coordination with SA3 considering user consent requirement, and RAN WGs are required for data collection, data access and data exposure

-	whether and how to improve mechanisms for data exchange among involved functional nodes required for the data handling, e.g. via enhanced control plane, enhanced user plane or any new plane.

NOTE 3:	The UE-CN interaction has dependency on WT#1.1. The user plane, if used for data exchange in this WT, has dependency on user plane architecture on WT#1.2.

**** End of Changes ****