SA WG2 Meeting #S2-170	S2-2507623

25 - 29 August, 2025, Goteburg, Sweden

Title:	Draft LS on Average Window for Alternative QoS

Response to:	S2-2506109/ R3-252397

Release:	REL-18

Work Item:	XRM

Source:	SA2 (CATT)

To:	RAN3

Cc:	-

Contact person:	Chunshan Xiong (chunshan.xiong@cictmobile.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

SA2 thanks for the LS from the RAN3, SA2 has the following answer for the LS:

- Understanding 1: NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled.

- Understanding 2: The Alternative QoS Parameter sets may contain different averaging window configurations. It means that the NG-RAN will use different durations to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled. This represents a new requirement to NG-RAN.

RAN3 could not reach consensus on introducing averaging window in the alternative QoS Profile, and respectfully requests SA2 to provide clarification on introducing averaging window in the alternative QoS Profile.

[SA2 Answer:] SA2 has discussed the LS and thinks that both Understanding 1 and 2 are not aligned with SA2's understanding. In the PDU Session Modification procedure of TS23.502, the SMF can provide a different QoS Profile with a different averaging window for the same QoS Flow. Also the NG-RAN can provide the available data rate without indicating the average window value to the AF for the QoS Flow.

SA2 kindly asks RAN3 to check whether the NG-RAN can use the averaging window configuration of the current activated QoS Profile to provide correct measurement of the GFBR/MRFR for an alternative QoS Profile with a different averaging window(i.e., to determine which alternative QoS Profile can be used as reference if the current activated QoS Profile is not fulfilled). If Yes, RAN3 can implement the GFBR/MFBR measurement based on the averaging window of the current activated QoS Profile in the related TS. If No, please provide further information.

2. Actions:

To RAN3

ACTION: 	SA2 kindly asks RAN3 to take the above information into account and provide further information if needed.

3. Dates of Next SA2 Meetings:

SA2#171		13th – 17th October 2025 		 Wuhan, CN

SA2#172		17th – 21st November 2025 		 Dallas, US