SA WG2 Meeting #170	S2-2507908

25 – 29 Aug, 2025, Goteborg , SE

Source:	NEC

Title:	[WT#7] Work task #7 Key issues on support of 6G RAT for NTN

Document for:	Approval

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the Key Issue description of WT# 7 on 6G NTN.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Work task 7 scope as agreed in 6G System Study item SP-250806:

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 8:	The detailed scope for WT#7 will be coordinated and aligned with RAN

2. Documents proposing WT#7

The following documents proposes KI descriptions

| Tdoc | Title | Source |
| --- | --- | --- |
| S2-2506226 | 6G system architecture to support of NTN | THALES |
| S2-2506437 | [WT#7] Scope and New Key Issues to Support 6G RAT for NTN | Nokia, T-Mobile USA, China Telecom, TNO, Thales, Vivo, ESA, IRDM, EchoStar, Boost Mobile Network, Viasa |
| S2-2506591 | [WT#7, NTN] Scope and New Key Issue to Support 6G RAT for NT | LG Electronics |
| S2-2506740 | [WT#7, all topics] Study proposal for NT | MediaTek Inc. |
| S2-2506873 | [WT#7, Service Continuity] Scope and Key Issue for Service Continuity | Futurewei |
| S2-2506940 | [WT#7, all topics] Study proposal for NTN | Ericsson |
| S2-2507231 | [WT#7, NTN] Key Issues for 6G NT | Xiaomi |
| S2-2506520 | [WT#7, NTNs] Non-Terrestrial Network | InterDigital Inc. |
| S2-2506978 | [WT#7, NTN] Clarification on 6G NTN work task scope | CSCN |
| S2-2507080 | Discussion and proposal on key issue for 6G satellite access | CATT, Thales, TNO, Novamint, vivo |
| S2-2507179 | Key Issue on NTN in 6G - Study proposal for Service Continuity aspects | NEC |
| S2-2507144 | [WT#07, all topics] Scope and key issue on NTN | ZTE |
| S2-2507308 | [WT#7, NTN] 6G support of NTN | Huawei, HiSilicon |

Below follows the proposals per document on the Key Issue description.

S2-2506226(Thales)：

Null.

S2-2506437(Nokia etc.)：

5.Y. Key Issue #Y (7.1): Support of 6G RAT for NTN in 6GS

This key issue (KI) will be studied to cover the following aspects:

- Study architecture enhancements needed for NTN, if any, based on the 6G RAT specified by 3GPP RAN.

NOTE 1:	The detailed scope will be coordinated and aligned with RAN.

NOTE 2:	Whether and how to handle interworking with EPS/5GS system/RAT will be studied in key issue(s) related to WT#2.

5.X. Key Issue #X (7.2): Support of Service Continuity for NTN

This key issue (KI) will be studied to cover the following aspects:

-	Whether and how to enhance mobility and service continuity when a UE moves from a TN to an NTN and vice versa.

-	Whether and how to support UE mobility between satellites at different types of NTN orbits, as well as whether and how to enhance the service continuity when a UE moves between the different types of NTN orbits (LEO, MEO, GEO).

S2-2506591(LG)：

5.Y	Key Issue #Y: Support 6G RAT for NTN

The support of TN with NTN in 6G system can provide ubiquitous connectivity and service continuity with the consistency of user experience in all locations.

This Key Issue aims to study 6G system to support 6G RAT for NTN, specifically:

-	How to support service continuity between (N)TN and NTN

NOTE 1:	This KI requires consideration of relationship and dependency with Key Issue #X for migration and interworking.

NOTE 2:	This KI requires coordination with RAN WGs.

S2-2506740(MediaTek)：

5.X. Key Issue #X: Support of NTN in 6G

This key issue (KI) will study to cover the following aspects:

-	 Study how to support 6GR for NTN, based on RAN decision, and support service continuity aspects.

-	Whether and how to enhance mobility and service continuity when a UE moves from a TN to an NTN and vice versa for inter-PLMN.

NOTE 1:	The detailed scope will be coordinated and aligned with RAN.

NOTE 2:	How to handle interworking with legacy system/RAT will be studied in key issues related to WT#2.

S2-2506873(Futurewei)：

5.X. Key Issue #X: Enhanced Session Continuity

KI #7.X Enhanced Session and Service Continuity:

Study whether and how to enhance session continuity mechanisms and procedures to provide seamless service continuity with support for high bandwidth and reliability for a service as connections/sessions of TN and NTN change due to user mobility or other network conditions.

S2-2506940(Ericsson)：

5.X. Key Issue #X: Support for 6G satellite access

5.X.1 Description

The RAN Study Item Description includes an objective to “Aim at a harmonized 6G Radio design for TN and NTN, including their integration.”. This KI studies the corresponding system level and core network aspects.

Commonalities between TN and NTN should be maximalized, i.e. the main features available for TN in 6G should be used also to support NTN in 6G, with NTN-specific adjustments done only if strictly needed to address NTN-specific challenges.

NOTE: Coordination and alignment with RAN are needed.

This KI will study:

-	How 6G satellite access is supported in 6G system, considering the progress on the overall 6G architecture:

-	Support for satellite access in the 6G system.

-	Support for session continuity between TN and satellite access.

-	Whether and how to re-use the 5G satellite specific functions (e.g. earth fixed tracking areas, satellite specific RAT types etc) in 6G, considering progress on the overall 6G architecture:

-	Specific handling due to quasi-earth-fixed (LEO) cells, if needed

-	Location verification

-	Support for policy/charging differentiation between TN and satellite access

Features that require CN functionality on-board the satellite are not included in the scope.

S2-2507231(Xiaomi)：

5.X. Key Issue #7.X: Study the support of 6G RAT for NTN

This Key Issue (KI) study covers the following aspects:

Study how 6G system supporting a new 6G RAT for NTN if it is designed by RAN WGs.

5.Y. Key Issue #7.Y: Study the support of service continuity for using NTN

This KI study covers the following aspects:

Study how 6G system supporting the service continuity when UE moves between TN and NTN

Study how 6G system supporting the service continuity when UE moves among satellites that are deployed in differ orbits.

S2-2506520(InterDigital)：

5.X. Key Issue #X: Support of NTN

This key issue focuses on how to support NTN in 6GS. The key issue includes:

- Study how to support 6G RAT for NTN

Note: this is based on RAN decision

- Study whether and how to enable Service Continuity for various cases: TN-NTN, NTN-TN, NTN-NTN (across various orbits)

- Study whether and how to enable deployment and selection of NFs on-board satellites

- Study whether and how to enhance network/orbit (re)selection and cell (re)selection in multi-orbit deployments

- Study whether and how to set up and maintain ISLs for multi-orbit deployments

S2-2506978(CSCN)：

Null.

S2-2507080(CATT etc.)：

5.X. Key Issue #X: Support of 6G satellite access

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

For the support of 6G satellite access, following aspects shall be considered and studied:

-	Architecture to support satellite access in 6G

-	How to support both transparent and regenerative payload based satellite access

-	How to support RAN node (in regenerative payload mode) mobility efficiently.

-	Whether and how to support selected Core Network functions onboard satellites.

-	Gap analysis on support of NTN related capabilities specified in EPS/5GS

-	Whether and how to support the features already in EPS/5GS, including dynamic satellite backhaul, discontinuous network coverage, UE-Satellite-UE communication, and S&F.

-	How to support multi-orbit satellite access and coordination

-	How to select the RAN node onboard to serve the UE, and how to support service continuity while switching connections between RAN nodes onboard different orbit satellites.

NOTE:	The solutions concluded for 6G to satisfy multi-orbit requirements could be considered for 5G (Rel-21), but do not prevent the study of alternative solutions for 5GS.

-	How to ensure service continuity across the different satellite access networks, or between terrestrial-satellite access networks in 6GS.

-	Whether and how to support multi-connectivity via TN and NTN simultaneously

-	Whether and how AI can benefit to 6G satellite access.

-	Whether and how to support on board computing service for 6G satellite access.

S2-2507179(NEC)：

5.X. Key Issue #X (7.X): Support of Service Continuity for NTN

This key issue (KI) will study to cover the following aspects:

Study on how 6G networks shall support service continuity when a UE moves among satellites in a “constellation” including

- the handover between satellites in a “constellation” for a UE including the signalling interaction between UE and the satellites, source and target satellite, satellite and ground network;

- the handover between ground network for a satellite in a “constellation” including the signalling interaction between UE and satellite, satellite and ground network.

S2-2507144(ZTE)：

5.X. Key Issue #X: Architecture support for multi-orbit NTN service continuity

This key issue will address whether and how the 6G system architecture can address the multi-orbit NTN system and enable service continuity when a UE transitions between satellites in different orbits (e.g., LEO, MEO, GEO), especially considering the following aspects:

-	How to define the reference model of the multi-orbit NTN system.

-	How to determine the optimal orbit (or orbit combination) for camping and for sessions based on service-specific requirements such as QoS, latency, reliability, and coverage.

-	How to ensure session continuity without packet loss during orbit transitions while satisfying those service-specific requirements.

-	How to ensure interworking between the multi-orbit NTN system and other accesses, including 3GPP accesses in the terrestrial network and non-3GPP satellite accesses.

S2-2507308(Huawei, HiSilicon)：

5.X. Key Issue #X: Support of NTN in 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

For the support of NTN, the following aspects are studied:

-	Study how to mitigate the effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance.

3.	Text proposal

In the merger of all the above documents, the “User Name” (company + tdoc number) shows from which document each of the specific text in the WT objective description comes from.

It is proposed to agree the following changes to TR 23.801-01:

**** First Change ****

5.X. Key Issue #X: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects

5.X. Key Issue #Y(7.1): study how to support 6G RAT for NTN in 6G system

This key issue will study how to support 6G RAT for NTN in 6G system, especially on the following aspects:

- Support for 6G satellite access in the 6G system.

- Support both transparent and regenerative payload.

- Support both normal and Store and Forward payload.

- Support for policy/charging differentiation and (re)selection among TN and satellite accesses with different orbit types (LEO, MEO, GEO).

- Lawful Interception.

- Study how to mitigate the effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance.

NOTE 1: The detailed scope on supporting 6G RAT for NTN will be coordinated and aligned with RAN.

NOTE 2: The detailed scope on overall 6G architecture will be coordinated and aligned with key issues related to WT#1.

NOTE 3:  Whether and how to re-use or enhance the 4G/5G satellite specific functions(e.g. earth fixed tracking areas, satellite specific RAT types etc.) should be evaluated.

NOTE 4:  The detailed scope of Lawful Interception will be coordinated and aligned with SA3-LI.

NOTE 5:  The normative work of multi-orbit satellite access can be deprioritized.

NOTE 6:  The deployment of CN functionalities on-board satellite should support the optimization of the UP path (e.g. by the Inter-satellite link) and take the deployment cost into consideration.

NOTE 7:  The mechanism design for intermittent NTN should follow the general principle designed for intermittent 3GPP network.

NOTE 8:  The policy/charging including the (re)selection among TN and satellite accesses with different orbit should be coordinated and aligned with general policy/charging framework in WT#1.2 and CT1/RAN2’s conclusions.

5.X. Key Issue #Z(7.2): How to support mobility and service continuity for NTN

This key issue will study how to support mobility and service continuity for NTN due to user mobility or other network conditions, especially on the following aspects:

- Whether and how to enhance mobility and service continuity between TN and NTN.

- Whether and how to enhance mobility and service continuity between the satellite accesses with the same and different orbits types (LEO, MEO, GEO).

5.X. Key Issue #M(7.3):Whether and how to support interworking with legacy system/RAT

This key issue will study whether and how to support interworking for NTN with legacy system/RAT, especially on the following aspects:

- Whether and how to support interworking with legacy system/RAT.

NOTE 9:	How to handle interworking with legacy system/RAT will be coordinated and aligned with key issues related to WT#2.

**** End of Changes ****