3GPP TSG-SA2 Meeting #171	S2-2509091

Wuhan, China, October 13-17 2025

Source:	Apple

Title:	KI#3: Proposal for agreements and conclusion

Document for:	Agreement

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes modifications to the interim agreements and conclusions for KI#3.

# 1.	Introduction

This paper proposes modification to the baseline agreements for KI#3 that were discussed in SA2#170.

In S2-2507677, the following aspect was listed for further consideration

- Whether considering the energy consumption of a PDU sessions or of a NF over a Path should be a trigger for reselection of UPFs (including embedding this information in DNAIs )

This contribution focusses on triggering UP reselection based on Energy consumption.

# 2. Discussion

The solutions in the TR that proposed triggering UP reselection based on Energy Consumption are listed in the table below.

| Solution | Key Principles |
| --- | --- |
| 19 : AMF (re-)selection based on energy related information | AMF evaluates its own status (Schedule of energy saving state) before accepting initial UE message. During AMF re-allocation, AMF takes energy related information into account for NF discovery |
| 20: Energy aware NF and path selection | NF profile is augmented with Energy Priority information, schedule of EnergyPriority information, schedule of EnergSaving/ not EnergySaving state. SMF considers these attributes when performing UPF selection |
| 21: UP path adjustment based on energy related information | The AF, PCF and/or SMF takes into account of energy related information for DNAI (re)selection to adjust the UP path of the PDU Session, during the AF influence on traffic routing procedure They subscribe to Energy consumption information of UPFs and calculates energy consumption per DNAI AF/PCF updates DNAI list based on energy related information. SMF selects the target DNAI. SMF reconfigures user plane of the PDU Session towards selected DNAI |
| 22: Support of UP path adjustments to maximize the usage of renewable energy | The SMF can subscribed to EIF to obtain the energy resource usage information per slice or for a set of slices, i.e., whether the renewable energy is used or not per slice. The SMF can be notified by EIF whether renewable energy per slice is used and then the SMF is triggered to query NRF by considering whether to reselect a UPF or not for the slice related to the existing PDU Sessions. |
| 23: UP path adjustment considering energy-related information | The operator may have various energy objectives such as maximizing renewable energy ratio or minimizing energy consumption. SMF reselects appropriate UPFs for the PDU session (subject to SSC mode) to optimize the operator's energy objective. |
| 28: Energy policy based NF discovery | Energy Priority in an NF Profile is defined as an array. Each value corresponds to an operator policy |
| 29: N3IWF/TNGF Reselection Based on Energy Related Information | Energy Priority Information or the Schedule of Energy Priority Information is taken into account for N3IWF or TNGF reselection. The AMF can then use the Rel-19 mechanism of sending a Registration Reject message to the UE |
| 30: Using the NF Status to control NF traffic before activating energy saving state | Augments solution 20. Proposes enhancement of NRF logic to enable the indication of EnergySaving State when the "UNDISCOVERABLE" NF Status cause an NF to be shut down because of energy related reasons. |

Solution #22 and Solution #23 are proposing energy consumption based UP path change (NF Reselection).

The Solution#22 proposes UP path re-election based on energy consumption/energy source information from EIF but such path reselection does not guarantee that system performance is optimised. Availability of UPFs with renewable energy source need not trigger a mass relocation of PDU Sessions. Energy consumption caused by UP path reselection may offset any potential gains (e.g., additional signalling).

Solution#23 proposes triggering path changes due to energy consumption. But a scenario of such a path change action ending up in a similar state (crossing the Energy Consumption threshold) in the newly selected NF cannot be ruled out.

In principle, UP path change triggered from node level energy consumption threshold may be possible at a macro level, say for a network slice and based on a long term trend. The aim here is to keep the overall energy consumption of the slice under check.

Performing UP path change/ NF reselection at a more granular level (say UE, PDU Session) may not be useful. If the NF reselection is just moving the biggest contributor of the energy consumption (UE or a PDU Session of the UE) of an NF to a new NF instance, it may only lead to increase in energy consumption of the newly selected NF.

The following two proposals are made regarding this open item.

Proposal 1: Energy Consumption at a slice level and based on a long term trend may be a trigger for reselecting UPFs

Proposal 2: Energy consumption at a granularity of a UE or a PDU Session does not trigger UP reselection.

In line with the above principles , the following changes are proposed to the agreed principles clause in TS 23.700-67

* * * Start of Change * * * *

### 7.1.3	Agreed Principles for KI#3

- Energy Consumption at a slice level and based on a long term trend may be a trigger for reselecting NFs.

- Energy Consumption at a granularity of a UE or a PDU Session is not used as a trigger for reselection of NFs.

* * * End of Change * * * *