SA WG2 Meeting #171	S2-2508245

13 - 17 October, 2025, Wuhan, China	(Revision of S2-2507206)

Source: 	vivo, China Mobile, MediaTek Inc., ZTE, Verizon, Ericsson, CATT, Nokia, Skylo, Thales, China Telecom, Spreadtrum, Xiaomi, OPPO, China Unicom, LG Electronics, Apple, Samsung, NTT DOCOMO, CSCN

Title: 	Interim conclusions on KI#1 Support of Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract: This contribution provides an interim conclusion of KI#1 Support of Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC.

1	Discussion

In SA2#169 meeting, a total of 11 solutions were proposed for Key Issue #1, forming a comprehensive set of CP/UP options.

The discussion paper S2-2506460 classified voice solutions into VoCP and VoUP, noting that major 3GPP specification and deployment upgrades would be needed for the current CP, with possibly higher costs than deploying UP. It therefore proposed adopting the UP approach as the way forward for voice transfer.

S2-2507021 further consolidated the proposals into four general voice transport options: CP non-IP, UP non-IP, UP IP, and CP IP, and analysed their respective advantages and limitations. Annexes of S2-2507021 provided detailed analysis regarding protocol overhead calculations for AS, IP (ROHC), non-IP (RTP reduction), CP (NAS reduction and corresponding architecture impacts) and UP non-IP (architecture impacts).

In last SA2#170 meeting, one LS-OUT (S2-2507636) was sent to RAN2, SA4, CT1, SA3, SA1, RAN1 with 11 questions related to KI#1. Except Question 7 (to SA3) and Question 8 (to CT1) on further NAS overhead reduction being key-factors for CP solution, all other questions are either orthogonal to or non-key-factors for CP vs. UP solution down-selection. The upcoming SA3/CT1 meetings are the same week as SA2 meeting, earlier reply LSes from these WGs are expected.

Given the expected further coordination workload with other working groups and the risk for solutions consolidation before the normative phase, it is proposed to firstly reach interim conclusions on UP voice transport options as one step, which could pave the way for further down-selection between IP and non-IP for voice over the user plane as next step.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-19 v1.0.0.

# * * * * First change* * * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.1	Interim conclusions for Key Issue #1

Editor's note 1:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

# * * * * End change* * * *