SA WG2 Meeting #171	S2-2508352

Wuhan, China, October 13 - 17, 2025	(is revision of S2-2506641)

Source:	vivo

Title:	[WT#1.2, Network slicing] Network Slicing

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper focus on promoting network slice in 6G that how to efficiently, simply and with low complexity implement the benefits of network slice.

1. Justifications

1.1 Architectural Motivation& Gap Analysis on simplifying the network slicing:

Network Slice is one of the core innovations in 5G communication technology. From the technical perspective of network slice, it can effectively satisfy the diverse business requirements, achieve efficient sharing and isolation of network resources, and accelerate the deployment and innovation of new services. The specific advantages can be reflected by the following aspects:

-	Precise guarantee of quality of service (QoS);

-	Optimized resource utilization rate and cost;

-	Flexibility and scalability;

-	Conducive to business model innovation;

-	Enhanced security and reliability;

However, after multiple release of study and evolution as the following table, network slicing has become extremely bloated, greatly increasing the processing logic and complexity of networks and UE.

| Release | Mainly study contents | Introduced NSSAI |
| --- | --- | --- |
| R15-NS (network slice) | Basic procedure and usage (e.g., request, allow, reject slice during registration procedure) | Requested NSSAI, Allowed NSSAI, rejected NSSAI, configured NSSAI… |
| R16-eNS | - network slicing interworking support from EPC to 5GC； - NSSAA | Pending NSSAI |
| R17-eNS_Ph2 , NRslice | - network slice access control - enhancement of RAN slicing for NR - network slice AS group information. (RAN alignments) - UE is only allowed to be registered with compatible Network Slices (GSMA alignments) | NSAG information, NSSRG information |
| R18-eNS_Ph3 | - Alternative NSSAI； - Slice-based SoR mechanism；(i.e., slice-based network selection, cancelled) - Partial NSSAI； - network slice validity； - Network controlled slice behavior | Replaced NSSAI, partially allowed NSSAI, partially rejected NSSAI, on-demand NSSAI |

On the other hand, in business, due to the insufficient support of UE for URSP, network slice has not been deployed and commercialized on a large scale.

Therefore, how to efficiently, simply and with low complexity implement the benefits of network slice requires 6G research.

Under the direction of simplifying 6G network slicing, the fundamental issue still needs to study how to efficiently map the application traffic to the network slice.

1.2. Technical Impact

To efficiently, simply and with low complexity implement the benefits of network slice in 6G for UE, there may be the following directions:

- Direction1: The UE is not aware of the slice, and the processing logic of the slice is executed by the network side.

- Direction2: The UE is aware of the slice, but some processing logics of the slice in UE is simplified, e.g., the RSD validation from slice perspective is implemented by PCF, and ensure the RSD is validate to UE and avoid the UE implementing it.

For the Direction1, since UE is not aware of the network slice, then the following issues needs to be considered:

-	For a given service, how the network side accurately determine the slices that to be applied;

-	For a given service, how the network decides to establish PDU session and how to establish PDU session.

For the Direction2, since the RSD validation from slice perspective is implemented by CN (i.e. PCF), then the RSD validation by UE based on the Allowed NSSAI is not needed, thus the following aspects may be impacted:

-	How the network ensures the RSD is valid from the network slice perspective to reduce the processing logic for UE;

-	If the network ensures the RSD is valid, whether Allowed NSSAI needs to be configured to UE, then whether and how to determine the Requested NSSAI.

Last meeting(SA2#170), the last version from the pen holder (S2-2508113) has pointed out the 6G network slicing studying direction including the following aspects:

- a.	Identify and address any areas of possible improvement and simplification;

- b.	Identify and address improvements of the application traffic mapping to network slice(s).

- c	Study impacts, if any, of the 6G network slicing on the interworking and migration.

**** First Change ****

Annex A.X. WT#1.2 Scope of Network slicing

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

The scope of the network slicing work task includes:

1.	Study the overall design and functionalities of the 6G network slicing, which may include existing 5G and new 6G functionality. The overall network slicing design will also consider:

a.	Identify and address any areas of possible improvement and simplification;

b.		Identify and address improvements of the application traffic mapping to network slice(s).

NOTE:	It will be studied which of the 5G functionality and potential new 6G functionality are mandatory to support in the UE and in the network.

2.	Study impacts, if any, of the 6G network slicing on the interworking and migration.

**** Second Change ****

5.X. Key Issue for 1.2 Network Slicing

Study the overall design and functionalities of the 6G network slicing, which may include existing 5G and new 6G functionality. The overall network slicing design will study the following aspects:

-	Study how to simplify the network slicing, including:

-	Identify and address any areas of possible improvement and simplification

-	Study how to map the application traffic to the network slice, including:

-		Identify and address improvements of the application traffic mapping to network slice(s)

-	Study impacts, if any, of the 6G network slicing on the interworking and migration.

**** End of Changes ****