3GPP TSG-SA2 Meeting #171	S2-2508363

Wuhan, CN, Oct 13 – 17, 2025                                          (was)

Source:	Nokia

Title:	Proposed agreement of principles for Key Issues 4

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This document proposes to agree principles for KI#4

# 1.	Introduction

This document discusses aspects related to several KIs and proposes principles for conclusions by considering the scope, architectural assumptions and the solutions captured in the TR.

## 1.1 Discussion for KI#4:

This KI mainly focuses on how the sensing data is collected and transported to the SF for sensing result generation. There are different options about the type of sensing data that a SE (i.e., gNB) can report to the SF, such as objects map, point clouds and periodograms . This issue needs to be coordinated with RAN WGs discussions and decisions, but the usage of point cloud seems to be a reasonable option that provides enough information to the SF for the generation of sensing results and allows their reusability for multiple sensing service requests without reporting a large volume of data from the SE to the SF. Such an information is transported to the SF using the control plane mechanism (i.e., NGAP, considering the que from positioning feature) for the SF to generate the sensing results.

Proposal for KI#4:

Following content for KI#4 is proposed to be captured as principles for agreement.

- SE provides the sensing data to the AMF using the existing communication method (i.e., NGAP between gNB and AMF).

- SE provides the sensing data in the form of point cloud information.

Note: The details of the point cloud information or the type of sensing data that are reported from the SE to the SF shall be co-ordinated with RAN WGs.

- AMF using the services for sensing, shall provide the consumer (SF), the received sensing data.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change (all new text) * * * *

### 7.4.1	Agreed Principles for KI#4

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following principles are agreed for KI#4:

- SE provides the sensing data to the AMF using the existing communication method (i.e., NGAP between gNB and AMF).

- SE provides the sensing data in the form of point cloud information.

Note: The details of the point cloud information or the type of sensing data that are reported from the SE to the SF shall be co-ordinated with RAN WGs.

- AMF using the services for sensing, shall provide the consumer (SF), the received sensing data.

# * * * * End of change * * * *