SA WG2 Meeting #171	S2-2508403

13 – 17 October, 2025, Wuhan, China	(revision of S2-25xxxxx)

Source:	China Telecom

Title:	KI#4: Interim agreements for Sensing Data and the Associated Information Collection and Transport

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#4 Sensing Data and the Associated Information Collection and Transport.

1. Discussion

According to TR23.700-14 V1.0.0, 12 solutions have been proposed to address KI#4 Sensing Data and the Associated Information Collection and Transport (Solution#1, Solution#3, Solution#5, Solution#18, Solution#21-26, Solution#34 and Solution#39. The open issues that need to be concluded are as following:

- Whether one Sensing Function is introduced or two logical functions Sensing Control Function (SCF) and Sensing Processing Function (SPF) are introduced to support sensing service? This is related to the basic sensing service architecture in KI#1.

- Whether the AMF is involved in sensing signalling delivery or sensing data transmission or both? This is also related to the basic sensing service architecture in KI#1.

- How to transfer the sensing signalling to the gNB sensing entity?

- How to transfer the collected 3GPP Sensing Data to the Core Network?

- Which entity is responsible for the Sensing Result generation?

Based on the assumption that two logical functions SCF and SPF are introduced to support sensing service, and the AMF can be optionally involved for sensing signalling delivery, we propose the following interim agreements for KI#4:

- The SCF sends the sensing service request and the configuration information to the gNB via a new interface defined between the SCF and the gNB, and optinally via the AMF, to trigger sensing data and the associated information collection and transport.

- The SCF needs to send the Sensing Initiated Request and the corresponding parameters to the SPF.

- When the gNB finishes the sensing data measurement, the gNB reports the sensing data to the SPF directly via the established data tunnel.

- The Sensing Entities can be multiple gNBs, and the SPF performs the aggregation of the sensing data collected from multiple sources.

- The SPF generates the Sensing Result based on the sensing data received from the gNB/gNBs

2. Proposal

It is proposed to agree the following changes into TR 23.700-14 V1.0.0.

# * * * * First change (All Text New) * * * *

7	Interim agreements

7.1	Agreed Principles

7.1.X	Agreed Principles for KI#4

Based on the assumption that two logical functions SCF and SPF are introduced to support sensing service, and the AMF can be optionally involved for sensing signalling delivery, the following principles are agreed for KI#4 Sensing Data and the Associated Information Collection and Transport:

- The SCF sends the sensing service request and the configuration information (e.g. the IP address of the SPF, the Sensing Task ID) to the gNB via a new interface defined between the SCF and the gNB, and optionally via the AMF, to trigger sensing data and the associated information collection and transport.

- The SCF needs to send the Sensing Initiated Request and the corresponding parameters (e.g. the gNB ID, the Sensing Task ID) to the SPF.

- When the gNB finishes the sensing data measurement, the gNB reports the sensing data to the SPF directly via the established data tunnel.

- The Sensing Entities can be multiple gNBs, and the SPF performs the aggregation of the sensing data collected from multiple sources.

- The SPF generates the Sensing Result based on the sensing data received from the gNB(s).

- The Sensing Result can be stored at the SPF/SCF for reuse.

NOTE 1:	The format and the detailed transport protocol of the 3GPP Sensing Data provided from the gNB to the Core Network will be determined by RAN WGs.

NOTE 2:	The detailed information and parameters required by the SPF for Sensing Result generation will be determined in the normative phase.

# * * * * End of changes * * * *