SA WG2 Meeting #171	S2-2508410

13 - 17 October, 2025, Wuhan, China

Source:	CATT

Title:	[WT#5 Data framework] Data Framework for 6G

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution is intended to describe the Work Task scope and the potential Key Issues for WT#5 Data Framework.

- Discussion

1.1	New services driven

The core objective of the mobile communication system defined by 3GPP (e.g., 5G System, 5GS) is to enable User Equipment (UE) to access the Data Network (DN) and to be able to use the data services provided by the network. To this end, 5GS defines Control Plane (CP) to support user access and control the signalling interactions within the system, and User Plane (UP) to support data transmission and QoS guarantee for user traffic data.

For 6G, SA1 defines the 6G System Data as follows.

6G System Data: the data that is controlled by 6G system and can be generated or collected by 6G system.

6G System Data, such as, Sensing data, AI data, NF context, is different from user traffic data. This type of data not only requires the 3GPP system to deliver application level data transparently between the UE and the DN, but presents some new requirements.

Examples of use cases for different data are given below:

- AI data：

  - AI data: data related to AI models/algorithms, which can be used to improve network performance and/or provide AI services (e.g. AI inference) for applications.

  - AI data including: e.g. raw data or processed datasets for training/testing/validating AI models, AI models and parameter data, AI inference input data and output data (e.g. AI analytic results).

  - Use cases: support AI model training data collection from UE, RAN, CN NFs (e.g. UPF, ADRF, other NFs) to Core Network for further processing to generate AI data set, AI model training, AI inference to generate analytic results, and/or storing for future use [refer to TR22.870 clause 5.9.2].

- Sensing data：

- Sensing data: data related to object or environment of interest for sensing purposes, which can be derived from 3GPP radio signals and also can be provided by non-3GPP devices.

- Sensing data including: e.g. sensing measurement data generated by sensing entities with sensing capabilities (e.g. UE, BS, and Non-3GPP devices), and sensing results generated after processing sensing data within the 6G system.

- Use cases: support sensing measurement data collection from UE, RAN to Core Network for further processing to generate sensing results and/or storing for future use [refer to TR22.870 clause 5.9.2, clause 5.4.6].

- NF context：

  - NF context: the “context” storage resource of the NF Service, which is fully or partially decoupled from the “compute” resource of the NF Service, e.g. UE context in 5G AMF, SM context in 5G SMF.

  - Use cases: support data sharing between different manufacturers or different places of the same manufacturer to avoid data silos; support trusted exposure of context data, such as expose network state data to trusted third parties or UEs.

Proposal#1: Regarding the data scope of Data Framework, considering the 6G System Data as defined by SA1, including e.g. AI, Sensing, and NF context. That is, the Data framework is designed for 6G System Data.

1.2	5G system gap analysis

Considering the following two aspects:

- Case-by-case data handling via CP/UP in 5GS

Beyond connectivity services (e.g. Sensing services, AI services) bring a large amount data (called 6G System Data) and some new data handling requirements. 6G System Data requires that the 6G system supports flexible and efficient data collection and processing for the data from different data sources (e.g., UEs, RANs, CN NFs) based on data consumer requirements.

However, traditional CP, designed for signalling delivery, which is not suitable for large amount of data transmission. UP as “pipeline” supports the data transmission between UE-UPFs for user plane traffic, and cannot well support the new transmission topology requirements (e.g., direct data transmission between RAN-CN) and on-demand data processing requirements.

At the same time, the current 5G data handling methods are varied and across different use cases (e.g., NWDAF-based for AI data handling, LPP-based UE positioning data handling, BS Sensing data handling is under discussion in R20), which introduces complexity to the architecture design, and the case-by-case solutions have poor scalability support for future new services.

- Data storage architecture in 5G

For supporting "stateless" NFs, 5G support the "compute" resource is decoupled from the "storage" resource of the NF Service. However, the compute/storage separation of 5G is not thorough enough, for example, the context data in 5G AMF/SMF is still unstructured, and a large amount of data is still dispersed in different 5GC NFs, which leads to insufficient data sharing among different manufacturers and inefficient data collection methods.

5G utilizes UDRs and UDSFs to support the storage of network data, such as in UDM/PCFs and AMF/SMFs, and ADRFs to support the storage of the collected and analytic data in the core network. For 6G, more data such as AI data and Sensing data will be generated or collected and controlled by the system, the 5G data storage architecture needs to be redesigned to facilitate data monetization via unified data management and control.

Therefore, a common data framework is needed for 6G to serve multiple use cases and easy extend to support future new services, including, e.g. to improve data collection efficiency and avoid duplication data collection, to support efficient and flexible data transmission between UE, RAN, and CN NFs, to support on-demand data processing of different service logic, to support efficient data storage for future use, to support data exposure for data monetization.

Proposal#2: The end-to-end 6G Data Framework provides generic data functionalities and service interfaces at the system level for different services. The data functionalities including, e.g. data management and control, data collection, data transmission, data processing, data storage and data exposure.

1.3 Coordination between SA2 and SA3/SA5/RAN WGs

Since the data framework involves all aspects related to data handling, the study is large and has some relationship with SA3, SA5 and RAN WGs.

In the process of data collection, processing, storage and exposure, the user consent, authorization, authentication, privacy protection, the confidentiality and integrity during data transmission, data access control are necessary. Also, the operator's unified management and control of the data, including e.g. where the data comes from, where it goes to, and what is processed, are all needed to be considered. Therefore, access control/user consent and privacy aspects will be coordinated with SA3.

Regarding the work split on the data framework between SA2 and SA5. The LS from SA plenary (SP-251261) includes:

TSG SA asks SA2 and SA5 to progress and collaborate (including with RAN2 and RAN3 where applicable) on 6G data related work tasks and taking the 5G working scope as the starting point.

SA2 and SA5 should coordinate with SA3 for the data security and privacy requirements.

Regular coordination and collaboration among the WGs are expected, including checks on the progress of each group for 6G data related work tasks in March 2026 (TSG SA#111) and/or June 2026 (TSG#112), to avoid the duplicate design for the same functionality and to develop system wide consistent solutions.

The Management data for SA5 in 5G is classified into performance measurements, end-to-end key performance indicators and trace metrics, which are irrelevant to UEs and for the operation of the network. But for this data framework in 6G system across UE, RAN and CN domains, considering the real-time and the dynamic of data type (e.g. sensing data, AI data), data provider and data consumer (e.g. UE, RAN, CN NF), it should be studied by SA2.

Proposal#3: SA2 should start to study 6G data framework and coordinate with SA3, SA5, RAN WGs, including e.g. access control/user consent and privacy aspects will be coordinated and aligned with SA3, charging and OAM aspects will be coordinated with SA5, Radio interface, RAN-CN interaction aspects will be coordinated and aligned with RAN WGs.

1.4 Coordination between WT#5 and other WTs

The WT#5 aims to study the data framework for 6G System with the considerations of different use cases (e.g. AI, Sensing services) and the data-related service requirements from SA1. The Data framework is designed to provide generic functionalities and service interfaces for different services. Therefore, this WT will to coordinate with other service-related WTs, such as WT#3, WT#4. More specifically, service-agnostic functionalities should be designed in this WT and service-specific functionalities will be coordinated and aligned with other WTs.

Considering the data transmission during the data collection, data processing, data storage and data consumption, the QoS and Policy of the data framework should be considered, and it will be coordinated and aligned with WT#1.2.

Proposal#4: The data framework focus on service-agnostic functionalities, interfaces and procedures design, the service-specific functionalities will be coordinated and aligned with other WTs (e.g. WT#3, WT#4), the QoS & Policy of data framework will be coordinated and aligned with WT#1.2.

**** First Change ****

Annex A.X. WT#5 Scope

WT#5: Study the unified data framework for all aspects related to efficient and scalable data handling including, for example, data collection/distribution, data transmission, data processing, data storage and retrieval, and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing.

The scope of this WT includes the following aspects:

- Study the end-to-end 6G system architecture design for data framework, including defining new NF(s), new data plane/enhancing CP, UP and interfaces, and high-level procedures, to support efficient and scalable data handling functionalities, e.g. data collection/distribution, data transmission, data processing, data storage, data retrieval, and data exposure.

- Study from SA2 architectural and system level on user consent, privacy and security to support the 6G data framework.

NOTE 1：The WT#5 study should follow the SA plenary guidance as LS in SP-251261. Access control/user consent and privacy aspects will be coordinated and aligned with SA3. Charging and OAM aspects will be coordinated with SA5. Radio interface, RAN-CN interaction aspects will be coordinated and aligned with RAN WGs.

NOTE 2：How to support data framework for roaming case should be considered during the study.

NOTE 3：The interworking between 6G data framework and 5G should be studied in WT#2.

NOTE 4：The Policy and QoS aspects for data framework should be studied in WT#1.2.

**** Second Change ****

5.X. Key Issue #X: End-to-end Data Framework for 6G

This key issue aims to study the data framework for 6G with the considerations of different use cases (e.g. AI services, Sensing services, and future other services) and the data-related service requirements from SA1.

The following aspects will be studied:

- Study the end-to-end 6G system architecture design to support the unified data framework, including e.g. defining new NF(s), new Data Plane (DP) and/or enhancing CP, UP and interfaces.

- Study the efficient and flexible data transmission mechanism and protocol stack design for data framework to transfer data between multiple data service entities (UE, RAN, CN NFs), e.g. a dedicated data session via CP, UP, or via new DP.

- Study how and which data processing functionalities are supported by data framework, e.g. data processing function discovery and selection, collaboration and/or fusion of multiple data types.

- Study how to support data stored and retrieved efficiently, e.g. unified data storage function, interface design between data provider/data consumer and data storage function.

- Study the procedures for data framework, including:

  - Data service entities (UE, RAN, CN NFs) and its capabilities registration and discovery, e.g. data generation/collection capability, data processing capability, data storage capability.

  - Data service entities (re)selection, e.g. data source (re)selection, data processing function (re)selection, and data storage function (re)selection.

  - Data transmission paths management (e.g. create/modify/delete of data session) between multiple data service entities, with the support of the UE’s mobility and its status change.

- Study how and which data to be exposed to the authorized third-parties and the UEs flexibly, with consideration of access control/user consent and privacy.

NOTE 1：This key issue study should follow the SA plenary guidance as LS in SP-251261. Access control/user consent and privacy aspects will be coordinated and aligned with SA3. Charging and OAM aspects will be coordinated with SA5. Radio interface, RAN-CN interaction aspects will be coordinated and aligned with RAN WGs.

NOTE 2：How to support data framework for roaming case should be considered during the study.

NOTE 3：The interworking between 6G data framework and 5G should be studied in WT#2.

NOTE 4：The Policy and QoS aspects for data framework should be studied in WT#1.2.

**** End of Changes ****