SA WG2 Meeting #171	S2-2508434

13 - 17 October, 2025, Wuhan, China                  	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0075 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Session Release Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper is proposed to clarify the session release procedure to align with RAN specification. In TS 38.413, the parameters in AIoT session release procedure include both the Correlation ID and AIOTF Identifier, which are missing in the session release procedure in TS 23.369. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the parameter of AIOTF Identifier in AIoT session release procedure; and remove cause parameter which is not used in TS 38.413. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The paremeters used in AIoT session release procedure do not align with RAN specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2.5	AIoT Session Release Procedure

This procedure is used to release the AIoT Session between the NG-RAN and the AIOTF. The AIoT Session release procedure can be triggered by the AIOTF or the NG-RAN node and is specified in TS 38.413 [10].

The initiation of AIoT Session release may be:

-	NG-RAN-initiated e.g. if the NG-RAN detects no AIoT Devices responds to the inventory procedure or the command procedure; or

-	AIOTF-initiated, e.g. if the AIOTF fails validating the results of AIOT NAS Inventory Response as specified in TS 33.369 [9]in the command procedure.

Both NG-RAN-initiated and AIOTF-initiated AIoT Session Release procedures are shown in Figure 6.2.5-1.

Figure 6.2.5-1: AIoT Session Release Procedure

1.	NG-RAN may decide to initiate the AIoT Session release procedure. NG-RAN sends AIoT Session Release request message (Correlation ID, AIOTF ID) to the AIOTF directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4.

2.	If the AIOTF receives the AIoT Session Release request message or the AIOTF decides to terminate all activities related to the AIoT Session, the AIOTF sends an AIoT Session Release Command message (Correlation ID, AIOTF ID) to the NG-RAN directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4.

3.	The NG-RAN node releases the AIoT Session and radio resources related to the AIoT session identified by the Correlation ID provided by the AIOTF, as specified in TS 38.300 [5]. And the NG-RAN confirms the AIoT Session Release by returning an AIoT Session Release Complete message (Correlation ID, AIOTF ID) to the AIOTF directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The AIOTF releases AIoT Session.

END OF CHANGES